| Product Model: MIX-Q370A1 |  | Date: 2023/8/11 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM16 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: Vbt_MIX_Q370A1_9.0.1098.bin |  |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |  |
| Released By: Andre_Liang (梁家偉) |  |  | Approved By: Danny Chiang(姜自立) |
| Item. | Root cause and change description |  |  |
| 01. | Adjust BIOS setup item for UEFI mode. |  |  |
| 02. | Fix UEFI RAID card didn't present HII setup item in Advanced page. |  |  |
| 03. | Secure Boot -> Disable. |  |  |
| 04. | Enable Graphics Output Protocol hot-plug support. |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_000000B4, M22906EB_000000B4, M22906EC_000000AE, M22906ED_000000BE |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM16.bin: EE90h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2022/11/23 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM15 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: Vbt_MIX_Q370A1_9.0.1098.bin |  |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |  |
| Released By: Bruce1_Liang (梁維剛) |  |  | Approved By: Danny Chiang(姜自立) |
| Item. | Root cause and change description |  |  |
| 01. | Support RAID mode. |  |  |
| 02. |  |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_000000B4, M22906EB_000000B4, M22906EC_000000AE, M22906ED_000000BE |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM15.bin: 5312h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2022/07/20 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM14 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: Vbt_MIX_Q370A1_9.0.1098.bin |  |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |  |
| Released By: Bruce1_Liang (梁維剛) |  |  | Approved By: Danny Chiang(姜自立) |
| Item. | Root cause and change description |  |  |
| 01. | Set default TPM to fTPM. |  |  |
| 02. | Enable secure boot. |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_000000B4, M22906EB_000000B4, M22906EC_000000AE, M22906ED_000000BE |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM14.bin: 92B3h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2021/04/23 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM13 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |  |
| Released By: Titanluke1_Lin (林治均) |  |  | Approved By: Roy1 Tseng(曾一峰) |
| Item. | Root cause and change description |  |  |
| 01. | Support audio codec ALC897. |  |  |
| 02. |  |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_000000B4, M22906EB_000000B4, M22906EC_000000AE, M22906ED_000000BE |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM13.bin: 74CB h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2020/01/10 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM12 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | Update the latest CPU microcode. |  |  |
| 02. | Update the latest ME. |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_000000B4, M22906EB_000000B4, M22906EC_000000AE, M22906ED_000000BE |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM12.bin: A79D h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2019/04/18 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM11 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.20.1301 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | Fix clear RTC or flash BIOS causing SMBIOS data lose. |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_00000096, M22906EB_0000008E, M22906EC_00000096 |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM11.bin: D03F h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2019/04/08 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM10 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.20.1301 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | ​Update the version to 1.0 for SAP process. |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_00000096, M22906EB_0000008E, M22906EC_00000096 |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM10.bin: AB99 h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2019/01/11 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM03 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.20.1301 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | Support I211/I219 UEFI LAN PXE. |  |  |
| 02. | Fix the AT behavior abnormal. |  |  |
| 03 | Fix the display fail from the external VGA. |  |  |
| 04 | Fix the M.2 device can’t be recognize. |  |  |
| 05 | Display the remove icon in USB 3,4 |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_00000096, M22906EB_0000008E, M22906EC_00000096 |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM03.bin:122B h |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2018/12/06 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM02 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.20.1301 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | Update the setup menu layout. |  |  |
| 02. | Correct the PCIE port setting |  |  |
| 03 | Poring the DIO function. |  |  |
| 04 | Fix the I211 PXE function support |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_00000096, M22906EB_0000008E, M22906EC_00000096 |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM02.bin: 02ABh |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |

| Product Model: MIX-Q370A1 |  | Date: 2018/12/04 |  |
| --- | --- | --- | --- |
| System BIOS: MQ37AM01 |  | VBIOS: MIX_Q370A1_cfl_1014.dat GOP: MIX_Q370A1_Vbt.bin |  |
| ME Type: Corporate ME Version: 12.0.20.1301 |  |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  | Approved By: Eric1 Lin(林志龍) |
| Item. | Root cause and change description |  |  |
| 01. | First release BIOS. |  |  |
| 02. |  |  |  |
| ## CPU Microcode |  |  |  |
| ## M22906EA_00000096, M22906EB_0000008E, M22906EC_00000096 |  |  |  |
| ## Remark |  |  |  |
| Checksum (BY Checksum.efi): MQ37AM01.bin: 106Eh |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS automatically. |  |  |  |