PCM-3335

## QE Compatibility Test Report

Release Date: Sep. 01, 1999

__________        __________     __________     __________

# Issue Stamp                    QA Manager               QE Manager               Test Engineer

Contents

1. Margin ----------------------------------------------------------------------------------------------------------------- 3

Includes temperature, DC voltage and IDE cable length tests.

2. Stability ---------------------------------------------------------------------------------------------------------------- 4

Includes burn-in and booting tests.

3. Performance ---------------------------------------------------------------------------------------------------------- 5

Includes performance indexes using various performance test software programs.

4. Hardware  compatibility ------------------------------------------------------------------------------------------- 6

Includes CPU, DRAM, adapter card and HDD compatibility tests.

5. Software compatibility --------------------------------------------------------------------------------------------- 8

Includes various OS and application software tests.

6. Power Consumption Data ----------------------------------------------------------------------------------------- 9

Includes +5V current rate.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

1. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 75℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -35℃ |  |

B. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.25V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4V |  |

C. IDE Drive Capability

Test Equipment:  Master Device: HDD Quantum FB1280A

Slave Device: CD-ROM  Creative CD620E

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |

2. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

B. Booting Stability Testing

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 996 | 0.4％ |  |
| KB Reset 20 Times | 20 | 20 | 0％ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0％ |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 994 | 0.6％ |  |
| H/W Reset 1000Times | 1000 | 1000 | 0％ |  |

3. Performance Testing

System Information:

| DRAM Type & Capacity | 4 MB(on board) |
| --- | --- |
| VGA Card | OnBoard (Chips F65545 B2) |
| HDD TYPE | Seagate ST51270A |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (Dos Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | ALI 6117B |  |  |  |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 54.56 |  |  |  |  | Mhz |
| FPU | N/A |  |  |  |  | Mhz |
| VIDEO | 3276 |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 32.9 |  |  |  |  | ID |
| Power Meter 2.02 | 5.5 |  |  |  |  | Mips |
| PcBench 9.0 CPUmark16 | 20.26 |  |  |  |  |  |
| H.E. WinStone 96 1.0 | N/A |  |  |  |  |  |
| WinBench 96 Ver 1.0 |  |  |  |  |  |  |
| Graphic WinMark 96 | 1.13 |  |  |  |  | MP/s |
| CPUmark 16 | 20.6 |  |  |  |  |  |
| CPUmark 32 | 24.2 |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 2688 |  |  |  |  | KB/sec |
| Performance ID | 21.23 |  |  |  |  |  |
| WINTECH 1.2 | 1.99 |  |  |  |  |  |
| NOTE |  |  |  |  |  |  |

4. Hardware Compatibility

A. Component Compatibility- CPU

386 CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Onboard | ALI M6117 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Onboard | MT4C16257DJ-7 4M | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| ###### Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PC-104 |  |  |  |
| on-board | 65545 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ###### ISA BUS |  |  |  |
| 3COM | 3C509 | Pass |  |
| Novell | NE2000 | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| AHA | 1542CF | Pass |  |

F. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| ###### Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| AAEON | PCM-3810 (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3200(SOUND) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) | Pass |  |

G. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Conner | CFS425A 425M | Pass |  |
| Quantum | 850AT | Pass |  |
| Quantum | TB540S SCSI | Pass |  |
| Maxtor | 7540AV 540M | Pass |  |
| Segate | ST51270A | Pass |  |
| Segote | ST32132A/2.1GB | Pass |  |
| WD | WDAC22100 | Pass |  |
| Fujitsu | M1638TAU | Pass |  |

H. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| PHILIPS | PCA123CD | Pass |  |
| VUEGO | 612A-004 12X | Pass |  |

I. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| FOCUS | 101KB | Pass |  |
| Mouse |  |  |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

5. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | ###### K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 3.12 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |

B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | N/A |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | N/A |  |  |

E. Diagnostics Software Testing

| Test Item / Software | QAFE5.29 |  |  | AMIDIAG4.5/5.1 |  |  | CHECKIT3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note |  |  |  |
| Main Component | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Hard Drive | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Keyboard | Pass |  | Fail |  |  | Pass |  |  |  |  |  |
| COM Port | Fail |  | Fail |  |  | Fail |  |  |  |  |  |
| LPT Port | Fail |  | Pass |  |  | Fail |  |  |  |  |  |
| Plug & play | N/A |  | N/A |  |  | N/A |  |  |  |  |  |

6. Power Consumption Data

System Information:

| ###### DRAM Type & Capacity | Onboard 4MB |
| --- | --- |
| VGA Card | OnBoard C&T F65545 |
| HDD TYPE | Quantum 3240AT |
| O.S. | DOS V 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| ##### Mode Type ## Processor Type | Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # ALI M6117 | 0.9 | 0.6 | Amp/Max |

Note :

                 1. Power supply voltage : ＋5V.

                 2. Normal model is measured while open system.

3. Suspend model all function off.