## PCM-3336

## QE Compatibility Test Report

Release Date : Dec. 18, 1999

__________        __________     __________     __________

# Issue Stamp                     QA Manager               QE Manager               Test Engineer

Summary of contents :

1. Cover Sheet																		1

2. Summary of contents																	2

3. Margin																					3

Includes temperature, DC voltage and IDE cable length tests.

4. Stability 																				4

Includes burn-in and booting tests .

A. Burn-In Testing(25℃/24hrs)															4

B. Burn-In Testing(Climatic Sequence/48hrs)												4

C. Booting Testing(25℃)																4

D. Booting Testing(Damp Heat,Cyclic/24hrs)													4

5. Performance																			5

Includes performance indexes using various performance test software programs .

6. Hardware compatibility																6

Includes CPU , DRAM , adapter card and HDD compatibility tests.

A. Component Compatibility- CPU														6

B. Adapter Compatibility- LAN															6

C. Adapter Compatibility- VGA															6

D. Adapter Compatibility- Other Card														6

E. Adapter Compatibility- Industry Control Card												7

F. Peripheral Compatibility- HDD Device													7

G. Peripheral Compatibility- Storage Device													7

H. Peripheral Compatibility- Others	                												7

I. Peripheral Compatibility- Input Device													7

I. Peripheral Compatibility- Input Device													8

7. Software compatibility																9

Includes various OS and application software tests.

A. O.S. Compatibility- QNX R4.22 &Windows												9

B. O.S. Compatibility- Windows 3.1														10

C. O.S. Compatibility- DOS and DOS Utility Compatibility Testing								11

6. Power Consumption Data																12

Includes +5V current rate.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

Quality Engineering Department of AAEON TECHNOLOGY  INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Hsin-Tien City, Taipei, Taiwan , ROC

Tel: 886-2-29121234

Fax:886-2-29130792

886-2-29130774

1. Margin Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard □ CHIPS F65545 B2 |
| DOC Type | M-System DOC2000 □4MB ■8MB □16MB □24MB □___MB |

Test Result:

| High Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: Ali M6117C |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FE V5.29 |  |  |  |  | □ AMIDIAG V4.50 |  |  |  |  |
| ■55℃ | ■60℃ |  | ■65℃ | ■ 70℃ |  | ■ 75℃ |  | ■ 80℃ | ■ 85℃ |  |  |

| Low Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: Ali M6117C |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FEV5.29 |  |  | □ AMIDIAG V4.50 |  | ■5℃ |  |  | ■0℃ |  |
| ■-5℃ | ■-10℃ |  | ■-15℃ | ■-20℃ |  | ■-25℃ |  | □-30℃ | □-35℃ |  | □-40℃ |

| High Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Ali M6117C |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 5.20V | ■Bgrade: 5.25V | ■CGrade: 5.30V | □DGrade: 5.35V | □EGrade: 5.4V |

| Low Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Ali M6117C |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 4.80V | ■Bgrade: 4.75V | ■CGrade: 4.70V | ■DGrade: 4.65V | ■EGrade: 4.6V |

2﹒Stability Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard □ CHIPS F65545 B2 |
| DOC Type | M-System DOC2000 □4MB ■8MB □16MB □24MB □___MB |

A. Burn-In Testing (24Hours)

Test Condition : Under Room Temp. 25℃

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| #### QAPlus/FE V5.29 |  |  |  |

B. Burn-In Testing (48 Hours)

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-61)

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| #### QAPlus/FE V5.29 |  |  |  |

##### C. Booting Testing

Test Condition : Under Room Temp. 25℃

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0 % | 21 Sec | 6 Sec |  |
| KB Reset 20 Times | 20 | 20 | 0 % | 13 Sec | 3 Sec |  |
| H/W Reset 1000Times | 1000 | 1000 | 0 % | 21 Sec | 6 Sec |  |

D. Burn-In Booting Testing (24 Hours)

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-30)

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off | 3020 | 3020 | 0 % | 27 Sec | 5 Sec |  |

3﹒Performance Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |
| O.S. Environment | ■MSDOS6.22 □Windows 95 □Windows NT □ |
| VGA Card | ■OnBoard □ CHIPS F65545 B2 |
| VGA Memory | □256K ■512K □1M □2M □4M □ |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : TM17A012 |
| Testing Feature | □32 bits Transfer ■Block Mode □DMA Mode |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD □Others : |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

| #### Testing Utility | CPU Type |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | ###### Ali M6117C |  |  |  |  |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 32.66 |  |  |  |  | MHz |
| ### FPU | N/A |  |  |  |  | MHz |
| VIDEO | 3947 |  |  |  |  | Chs/sec |
| ### Norton 8.0 |  |  |  |  |  |  |
| SI | 31.5 |  |  |  |  | ID |
| ### Power Meter 2.02 |  |  |  |  |  |  |
| MIPS Test | 4.5 |  |  |  |  | Mips |
| WinStone _____ |  |  |  |  |  |  |
| HighEnd Graphic WinStone_____ | #### N/A |  |  |  |  |  |
| Business Graphic Winstone_____ | N/A |  |  |  |  |  |
| WinBench 96 |  |  |  |  |  |  |
| Graphic WinMark 96 | 0.852 |  |  |  |  | MP/s |
| CPUmark 16 | 17.9 |  |  |  |  |  |
| CPUmark 32 | fail |  |  |  |  |  |
| ### Coretest V3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 1840 |  |  |  |  | KB/Sec |
| Performance ID | 17.34 |  |  |  |  |  |
| ### Wintech V1.2 |  |  |  |  |  |  |
| Overall WinTach RPM | 3.03 |  |  |  |  |  |

4﹒Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Ali | #### AliM6117C |  |  |  |

B. Adapter Compatibility- LAN Card

Action:

| 1. Load ODI Driver (Lsl,Ipxodi,Vlm Or Netx) | 2. Login To Server System |
| --- | --- |
| 3. Transfer Data Between SRV And WK(Mulity Task) | 4. logout |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Intel | Ether Express PRO100B |  |  |  |
| Realtek | RTL 8019A ISA |  |  |  |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| ### TopView | ### ET-4000/W32I |  |  |  |
| ### TopView | ### ET-4000/AX |  |  |  |

D. Adapter Compatibility- Other Card

Adapter Card List:

| Sound Card | Device Model | Pass | Fail |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Creative | SB16 PNP/CT2980 ISA |  |  |  |  |  |

##### E. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| 104 Module |  |  |  |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) |  |  |  |
| AAEON | PCM-3820 rev. C (SSD) |  |  |  |
| AAEON | PCM-3110 (PCMCIA MASTER) |  |  |  |
| AAEON | PCM-3660 (ETHERNET) |  |  |  |
| AAEON | PCM-3420 (SCSI) |  |  |  |
| AAEON | PCM-3200(SOUND) |  |  |  |

F. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Quantum | TM17A012(1.7G) |  |  |  |
| WD | AC26400-00RT(6.4G) |  |  |  |
| Seagate | 32155W(2G) |  |  |  |

##### G.Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| CD/ROM |  |  |  |  |
| NEC | CDR-3001A |  |  |  |
| SAMPO | CDM-T631 |  |  |  |
| ZIP |  |  |  |  |
| IOMEGA | ZIP 100MB Parallel Port |  |  |  |
| LS-120 |  |  |  |  |
| LS-120 | LKM-F934-1 |  |  |  |

H.Peripheral Compatibility- Others

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| RS-485 Port |  |  |  |  |
| ADAM | ADAM-4011 |  |  |  |
| Parallel Port |  |  |  |  |
| IOMEGA | ZIP 100MB Parallel Port |  |  |  |
| EPSON | COLOR 740 |  |  |  |

##### I. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |
| ACER | 6511-TW |  |  |  |
| LEMEL | 5121 |  |  |  |
| Mouse |  |  |  |  |
| IBM | PS/2 Mouse |  |  |  |
| Logitech | First Mouse |  |  |  |

5﹒Software Compatibility

A. O.S. Compatibility-QNX R4.22 & Windows

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K □ |  |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |  |
| VGA Card & BIOS | ■OnBoard CHIPS F65545 B2 ■BIOS: 65540/545 VGA |  |
| VGA Memory Capacity & Brand | Brand: NEC 424260-60 □60ns ■70ns □ ns □DRAM ■VRAM □ □256K ■512K □1M □2M □4M □ |  |
| CPU Type | Ali M6117C |  |
| HDD Type | □Max □Sea □Con ■Qua □WD: TM17A012 |  |
| CD-ROM Type | ■NEC □Panasonic □Pioneer : CDR-3001A |  |
| Driver or Utility | □VGA □HDD □Others : |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: Ali M6117C |
| SCSI HDD with AHA-1542 □CP □CF |  |  | Testing CPU: |
| SCSI HDD with AHA-□2940UW □2940W |  |  | Testing CPU: |
| Boot |  |  |  |
| Cold Boot |  |  |  |
| Warm Boot (Ctrl+Alt+Shift+Del) |  |  |  |
| Shutdown-Normal Shutdown |  |  |  |
| Shutdown-Fast Shutdown |  |  |  |
| Function Test |  |  |  |
| VEDIT-Split Windows Ver and Hor |  |  |  |
| VEDIT – Zoom |  |  |  |
| Serial Mouse |  |  |  |
| PS/2 Mouse |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+Enter |  |  |  |
| Ctrl+Enter |  |  |  |
| Windows for QNX |  |  |  |
| File Manager |  |  |  |
| Shell |  |  |  |
| Calculator |  |  |  |
| Clock |  |  |  |
| Ruler |  |  |  |
| Eyes |  |  |  |
| Life |  |  |  |
| Load Monitor |  |  |  |
| Mandelbrot |  |  |  |
| Puzzle |  |  |  |
| Rebound |  |  |  |
| Solitaire |  |  |  |
| Plaid |  |  |  |
| Video Poker |  |  |  |
| Zoom |  |  |  |

B. O.S. Compatibility-Windows 3.1

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K □ |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |
| VGA Card & BIOS | ■OnBoard CHIPS F65545 B2 ■BIOS: 65540/545 VGA |
| VGA Memory Capacity & Brand | Brand: NEC 424260-60 □60ns ■70ns □ ns □DRAM ■VRAM □ □256K ■512K □1M □2M □4M □ |
| CPU Type | Ali M6117C |
| HDD Type | □Max □Sea □Con ■Qua □WD: AC26400-00RT |
| CD-ROM Type | ■NEC □Panasonic □Pioneer : CDR-3001A |
| Driver or Utility | ■VGA □HDD □Others : |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  | Monitor turn into black |
|  | 32K |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
|  | 16.7M |  |  |  |  |  |
| 800X600 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  | Monitor turn into black |
|  | 32K |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  |  |
| 1280X1024 | 16 |  |  |  |  |  |
| 1600X1200 | 16 |  |  |  |  |  |

Test Results:

| #### Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: Ali M6117C |
| SCSI HDD with AHA-1542 □CP □CF Card |  |  | Testing CPU: |
| SCSI HDD with AHA-□2940UW □2940W |  |  | Testing CPU: |
| Keyboard Function Test |  |  |  |
| Ctrl+Alt+Del |  |  |  |
| Alt+Tab |  |  |  |
| Alt+Esc |  |  |  |
| Alt-F4 |  |  |  |
| Function Test |  |  |  |
| Virtual Memory |  |  |  |
| System Clock |  |  |  |
| DOS Prompt |  |  |  |
| Real Mode & Protect Mode Switch |  |  |  |
| Task Switching |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| Serial Mouse |  |  |  |
| PS/2 Mouse |  |  |  |

C. O.S. Compatibility-DOS and DOS Utility Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: MOSEL V53C518160OAK50 ■4□8 □16 □32 □64MB □DRAM□SDRAM□SODIMM ■OnBoard |
| --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K □ |
| System BIOS Version | □Award ■AMI Ver: 1.0 (12/16/1999) |
| VGA Card & BIOS | ■OnBoard CHIPS F65545 B2 ■BIOS: 65540/545 VGA |
| VGA Memory Capacity & Brand | Brand: NEC 424260-60 □60ns ■70ns □ ns □DRAM ■VRAM □ □256K ■512K □1M □2M □4M □ |
| CPU Type | Ali M6117C |
| HDD Type | □Max □Sea □Con ■Qua □WD: AC26400-00RT |
| CD-ROM Type | ■NEC □Panasonic □Pioneer : CDR-3001A |
| Driver or Utility | ■VGA □HDD □Others : |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: Ali M6117C |
| SCSI HDD with AHA-1542 □CP □CF Card |  |  | Testing CPU: |
| SCSI HDD with AHA-□2940UW □2940W |  |  | Testing CPU: |
| Function Test |  |  |  |
| Sysinfo Check (Before DOS Initialed ) |  |  |  |
| System Clock(Compare to RTC CLK) |  |  |  |
| DOS Prompt |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| Serial Mouse |  |  |  |
| DOS Commander |  |  |  |
| MSD.COM |  |  |  |
| EDIT.COM |  |  |  |
| EMM386.EXE |  |  |  |
| HIMEN.SYS |  |  |  |
| Norton Commander |  |  |  |
| Key F1~F10 Function |  |  |  |
| Key Alt+F1~F10 Function |  |  |  |
| Key Ctrl+F1~F10 Function |  |  |  |
| ET Ver3.53 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |

6. Power Consumption Data

System Information:

| ###### DRAM Type & Capacity | Onboard 4MB |
| --- | --- |
| VGA Card | OnBoard C&T F65545 |
| DOC Type | M-System DOC2000 16MB |
| O.S. | DOS V 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| ##### Mode Type ## Processor Type | Normal Mode | Unit |
| --- | --- | --- |
| # ALI M6117 | 0.9 | Amp/Max |

Note :

                 1. Power supply voltage : ＋5V.

