AAEON

### ISO-9001/ISO-14001 Certified

### Industrial Automation PCs

PCM-4896

QE Compatibility Test Report

Release Date: Aug. 10, 1999

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility --------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------------- 8

Includes burn-in and booting tests.

4. Performance --------------------------------------------------------------------------------------------------------- 9

Includes performance indexes using various performance test software programs.

5. Power consumption data ------------------------------------------------------------------------------------------ 10

Includes +5V current rate.

6. Margin ---------------------------------------------------------------------------------------------------------------- 11

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 | ˇ |  | ˇ |  |  | ˇ |  |  | ˇ |  |  |  |  |  |
| MS-DOS 6.2/V (JAPAN) | ˇ |  | ˇ |  |  | ˇ |  |  | ˇ |  |  |  |  |  |
| QNX 4.22 & Wins | N/A |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |

B. Microsoft Windows & IBM OS/2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | KB Function |  | Display Driver |  | Ethernet Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| ### WINDOWS 3.1 | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| ###### WINDOWS 95 (ENGLISH) | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| WINDOWS 95 (JAPAN) | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| WINDOWS 98 (ENGLISH) | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| ###### WINDOWS NT4.0W/S ###### (ENGLISH) | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| ###### WINDOWS NT4.0W/S ###### (JAPAN) | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |
| ###### OS/2 Warp 4.0 | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  | ˇ |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Business/Access 97 | Pass |  | Pass |  |
| Business/Corel DRAW! 7 | Pass |  | Pass |  |
| Business/Excel 97 | Pass |  | Pass |  |
| Business/Lotus 1-2-3 97 | pass |  | pass |  |
| Business/Navigator | Pass |  | Pass |  |
| Business/Power Point 97 | Pass |  | Pass |  |
| Business/Quattro Pro 7 | Pass |  | Pass |  |
| Business/Task Switching 1 | pass |  | pass |  |
| Business/Task Switching 2 | Pass |  | Pass |  |
| Business/Word 97 | Pass |  | Pass |  |
| Business/Word Perfect 7 | Pass |  | Pass |  |
| High-End/AVS/Express 3.1 | pass |  | pass |  |
| High-End/Front Page 97 | Pass |  | Pass |  |
| High-End/Microstation 97 | Pass |  | Pass |  |
| High-End/Photoshop 4.0 | Pass |  | Pass |  |
| High-End/Premiere 4.2 | Pass |  | Pass |  |
| High-End/PV-WAVE 6.1 | Pass |  | Pass |  |
| High-End/Visual C++ 5.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Amidiag4.5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | #### Pass |  | #### Pass |  |  |  |  |  |  |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Com Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Cyrix | Media GXM 200 | Pass |  |
| Cyrix | Media GXM 233 | Pass |  |
| Cyrix | Media GXM 266 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Siemens | HYB39S16800AT SDRAM 32M | Pass |  |
| MT | MT48LCM8A1 SDRAM 32M | Pass |  |
| LGX | GM72V661641CT7J SDRAM 64M | Pass |  |
| HYNDEAI | HY57V658TC-10 9828A 128MB | Pass |  |
| NEC | D4564841G5-A10-9JF 128MB | Pass |  |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On Board | C&T 69000 | pass |  |

D. Adapter Compatibility- LAN Card

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### On Board | Realtek RTL8139 | Pass |  |
| Surecoh | Digital 21140-AF | Pass |  |
| 3COM | 3C905B-TX | Pass |  |
| Realtek | ### RTL8139 | Pass |  |
| ### 3COM | 3C905-TX | Pass |  |

E. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| AHA | 2940UW | Pass |  |

F. Adapter Compatibility- SOUND Card

SOUND Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| Creative | ENSONIQ Audio PCI | Pass |  |
| Onboard | MediaGX | Pass |  |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| AAEON | ### PCM-3820/C-4 | Pass |  |
| AAEON | PCM-3820-2 | Pass |  |
| AAEON | PCM-3110 | Pass |  |
| AAEON | PCM-3111 | Pass |  |
| AAEON | PCM-3200 | Pass |  |
| AAEON | PCM-3420 | Pass |  |
| AAEON | PCM-3600 | Pass |  |
| AAEON | PCM-3660 | Pass |  |
| PS. PC-104 with add_on card conflict with CRT,Sound, and LPT connector. |  |  |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST34520A | Pass |  |
| WD | WDAC33100 | Pass |  |
| Quantum | Big-Foot TX4.0AT | Pass |  |
| Fujitsu | MPC3032AT | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand |  | Device Model | Result | Note |
| --- | --- | --- | --- | --- |
| CD/ROM |  |  |  |  |
| TEAC |  | CD-532E | Pass |  |
| FAX/Modem |  |  |  |  |
| LEMEL |  | 33.6 HIGH SPEED MODEM | Pass | Com1,com2 ring up pass |
| ZIP (ECP/EPP/SPP) |  |  |  |  |
| IOMEGA |  | ZIP 100M | Pass |  |
| MO |  |  |  |  |
| FUJITSU |  | MCB3064SS | Pass |  |
| DiskOnChip |  |  |  |  |
| Flushdisk |  | M-Systems DiskOnChip2000 24M | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 7932 (USB) | #### Pass |  |
| LEMEL | KB-5121 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |
| DEXIN | A2U8OOA(USB) | Pass |  |
| Adam (RS-485) |  |  |  |
| RS-485 | COM2 | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG 4.5 | Pass |  |

B. Booting Stability Testing

Test Condition: Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| H/W Reset 1000 Times | 1000 | 1000 | 0 |  |
| KB Reset 20 Times | 20 | 20 | 0 |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: Temp.: -10℃~ 70℃ Humidity: 0% ~ 95%

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| H/W Reset 42 Hours | 2444 | 2437 | 0.28% |  |

D. Burn-In Stability Testing

Test Condition: Burn-In Between -10℃~70℃	Humidity: 0% ~ 90%							 Burn-In 32 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAD 4.5 32 Hours | Pass |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | MT 48LC2M8A1TG-10S 32M |
| --- | --- |
| VGA Card | On Board (C&T 69000) |
| HDD TYPE | Fujitsu MPC3032AT |
| VGA Testing Mode | 1024X768X256 colors (For WinStone98 & WinBench98) |
| O.S. | DOS 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high Files=35 Buffers=40 | Prompt $p$g ### C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Cyrix GXM 200 | Cyrix GXM 233 | Cyrix GXM 266 |  |  |  |  |  |  |  |  | Unit |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU | 853.22 | 995.43 | 1288.29 |  |  |  |  |  |  |  |  |  | Mhz |
| FPU | 2344.10 | 2734.8 | 3516.19 |  |  |  |  |  |  |  |  |  | Mhz |
| VIDEO | 15653 | 13653 | 13653 |  |  |  |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 529.3 | 617.5 | 793.9 |  |  |  |  |  |  |  |  | ID |  |
| Power Meter 2.02 | 81.3 | 94.8 | 123.6 |  |  |  |  |  |  |  |  | Mips |  |
| PcBench 96 CPUmark16 | 199.78 | 224.98 | 289.57 |  |  |  |  |  |  |  |  |  |  |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 9.84 | 10.7 | 12.8 |  |  |  |  |  |  |  |  |  |  |
| High-End Winstone | 9.46 | 10.2 | 12.3 |  |  |  |  |  |  |  |  |  |  |
| Winbench 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### High-End/Business Graphics WinMark98 | 50.5/39.6 | 53.8/40.3 | 63.4/49.2 |  |  |  |  |  |  |  |  | MP/s |  |
| ###### CPUmark 32 | 201 | 225 | 258 |  |  |  |  |  |  |  |  |  |  |
| Coretest |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 8352 | 8432 | 8432 |  |  |  |  |  |  |  |  | KB/sec |  |
| Performance ID | 56.76 | 57.24 | 57.24 |  |  |  |  |  |  |  |  |  |  |
| WINTECH 1.2 | 184.97 | 243.40 | 243.40 |  |  |  |  |  |  |  |  |  |  |
| note |  |  |  |  |  |  |  |  |  |  |  |  |  |

5. Power Consumption Data

PCM-4896 System Information:

| DRAM Type & Capacity | NEC D4516821AG5-A10-9NF 32MB*1 |
| --- | --- |
| VGA Card | Onboard C&T B69000 |
| DOC Type | M-Systems MD-2800-D08 |
| O.S. | DOS V 6.22 |

PCM-4896L System Information:

| DRAM Type & Capacity | NEC D4516821AG5-A10-9NF 32MB*1 |
| --- | --- |
| VGA Card | Onboard Cyrix GXM |
| DOC Type | M-Systems MD-2800-D08 |
| O.S. | DOS V 6.22 |

Operating System Environment (DOS Only)

| Config.Sys | Autoexec.Bat |
| --- | --- |
| Device=C:\Dos\Himem.Sys Dos=High Files=35 Buffers=40 | Prompt $P$G C:\Dos\Smartdrv.Exe /X |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result for PCM-4896

| Mode Type ## Processor Type | # Normal Mode | Standby Mode | Unit |
| --- | --- | --- | --- |
| # Cyrix GXM200 | 2.7 | 1.2 | Amp/Max |
| Cyrix GXM233 | 2.8 | 1.2 | Amp/Max |
| Cyrix GXM266 | 2.6 | 1.2 | Amp/Max |

Test Result for PCM-4896L

| Mode Type ## Processor Type | Normal Mode | Standby Mode | Unit |
| --- | --- | --- | --- |
| # Cyrix GXM200 | 2.6 | 1.0 | Amp/Max |
| Cyrix GXM233 | 2.9 | 1.0 | Amp/Max |
| Cyrix GXM266 | 2.5 | 1.0 | Amp/Max |

Note :

       1.Power supply voltage : ＋5V.

       2.Normal Model is measured while open system.

       3.Standby Model all function off.

6. Margin Testing

A. Temperature Test (Power On/Off)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -10℃ |  |

B. IDE Drive Capability

Test Equipment

Primary IDE Port :

Master Device: HDD Fujitsu MPC3032AT

Slave  Device: CD-ROM TEAC CD-532E

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |