PCM-5894

QE Compatibility Test Report

Release Date: May. 12, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility ---------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ---------------------------------------------------------------------------------------------- 6

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability ----------------------------------------------------------------------------------------------------------------- 11

Includes burn-in and booting tests.

4. Performance ----------------------------------------------------------------------------------------------------------- 12

Includes performance indexes using various performance test software programs.

5. Power consumption -------------------------------------------------------------------------------------------------- 13

Includes +5V current rate.

6. Margin ------------------------------------------------------------------------------------------------------------------ 14

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

- Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | KB Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LINUX 2.0.30 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |

B Microsoft Windows & IBM OS/2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | KB Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows S 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows NT 4.0/ W |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0/S |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95(J) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 W(J) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 S(J) |  |  |  |  |  |  |  |  |  |  |  |  |

(J): Japanese

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| Norton Commander 5.0 |  |  | Pass |  |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |

E. Diagnostics Software Testing

| Test Item / Software | QAFE5.29 |  |  | QAPLUS4.8 |  |  | AMIDIAG4.5 |  |  | AMIDIAG5.1 |  |  | CHECKIT3.0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| ##### Main Component | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| ##### Video Adapter | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Hard Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Keyboard | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Com Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| LPT Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Plug & Play | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

F. N.O.S. & Application Software Compatibility (On_Board Network)

System Information:

| DRAM Capacity & Brand(SIMM Qty) | Brand:NEC □1□2□4□8■16□32M ■60ns □70ns □_____ ns Qty:□1 ■2 □3 □4 |
| --- | --- |
| System BIOS Ver. | ■AWARD □AMI Ver:2.0 |
| KB BIOS Version | □AMI- □VIA- ■Internal |
| Backplane | □05P2 □08P3 □14P3 □ 03 □08 □14 ■N/A |
| O.S. Environment | □DOS5.0 □DOS6.0 ■DOS6.22 |
| VGA Card & BIOS | ■OnBoard □ □BIOS: |
| Power Supply | □MPU-250W■SSH-250G □SSH-350G □ST-250AF |
| IDE HDD Type | □Max □Sea □Con ■Qua □Fuj □IBM : 1700AT |
| SCSI HDD Type | □Max □Sea □Con □Qua □Fuj □IBM: |
| Lan Card | ■OnBoard: RTL-8139 BIOS: |
| ### CPU Type | Intel Pentium MMX 233 |
| CD-ROM Type | □NEC ■PANASONIC □PIONEER □CREATIVE : |
| Driver or Utility | ■VGA □HDD ■Others : RTL-8139 |

Test Result:

| SUBJECT | Network Operation System Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| NetWare 4.11 |  | ˇ |  |  |
| NetWare 5.0 |  | N/A |  |  |
| Windows NT 4.0 |  | ˇ |  |  |
| Windows NT 5.0 |  | N/A |  |  |
| SCO Unix |  | N/A |  |  |
| Linux Unix |  | ˇ |  |  |
| ### NetBench 5.01 |  | ˇ |  | Figure 1 |
| SUBJECT | Workstation Application Function Testing |  |  |  |
| Dos 6.22 for IPXODI |  | ˇ |  |  |
| Dos 6.22 for package driver |  | ˇ |  |  |
| Windows 3.1 for IPXODI |  | ˇ |  |  |
| Windows 3.1 for package driver |  | ˇ |  |  |
| WFW 3.11 for NDIS 2.0/3.0 |  | N/A |  |  |
| WFW 3.11 for NDIS 3.0/ODI |  | N/A |  |  |
| Windows 95 for IPX/SPX |  | ˇ |  |  |
| Windows 95 for NetBEUI |  | ˇ |  |  |
| Windows 95 for TCP/IP |  | ˇ |  |  |
| Windows 98 for IPX/SPX |  | ˇ |  |  |
| Windows 98 for NetBEUI |  | ˇ |  |  |
| Windows 98 for TCP/IP |  | ˇ |  |  |
| Windows NT 4.0 for IPX/SPX |  | ˇ |  |  |
| Windows NT 4.0 for NetBEUI |  | ˇ |  |  |
| Windows NT 4.0 for TCP/IP |  | ˇ |  |  |
| Windows NT 5.0 for IPX/SPX |  | N/A |  |  |
| Windows NT 5.0 for NetBEUI |  | N/A |  |  |
| Windows NT 5.0 for TCP/IP |  | N/A |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | ### PENTIUM-75 | #### Pass |  |
| Intel | PENTIUM-100 | Pass |  |
| Intel | PENTIUM-120 | Pass |  |
| Intel | PENTIUM-133 | Pass |  |
| Intel | PENTIUM-150 | Pass |  |
| Intel | ### PENTIUM-166 | Pass |  |
| Intel | PENTIUM-200 | Pass |  |
| Intel | PENTIUM-200 MMX | Pass |  |
| Intel | PENTIUM-233 MMX | Pass |  |
| Cyrix | 6X86-110M(M1) | Pass |  |
| Cyrix | 6X86-120M(M1) | Pass |  |
| Cyrix | 6X86-133M(M1) | Pass |  |
| Cyrix | 6X86-150M(M1) | Pass |  |
| Cyrix | 6X86-200M(M2) | Pass |  |
| Cyrix | 6X86-233M(M2) | Pass |  |
| Cyrix | 6X86-300M(M2) | Pass |  |
| AMD | 5K86-75 | Pass |  |
| AMD | 5K86-100 | Pass |  |
| AMD | K6-200 | Pass |  |
| AMD | K6-233 | Pass |  |
| AMD | K6-266 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| NEC | 4217405 16M EDO RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| PANASONIC | MN4117400BSJ-06 32M EDO RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164BJC 8M EDO RAM | Pass |  |
| HYNDAI | ### HY5117400AJ-70 16M FPM RAM | Pass |  |
| MITSUBISH | M5M417400CJ-7 16M FPM RAM | Pass |  |
| SAMSUNG | KM44C4000AK-6 16M EDO RAM | Pass |  |
| SAMSUNG | KM41C16000BK-6 64M | Pass |  |
| LGS | GM71C17403CJ 32M EDO RAM | Pass |  |
| LGS | GM71C18163CJ 8M EDO RAM | Pass |  |
| TI | TMS417409ADJ 16M EDO RAM | Pass |  |
| PANASONIC | MN414400BSJ 8M FPM RAM | Pass |  |
| MICRON | MT4C4M4E8DJ 16M EDO RAM | Pass |  |
| SIEMENS | HYB5118165BSJ 8M EDO RAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On_board | CHIPS B65554 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| SVEC | ### AM79C970AKC PCI 10MBPS | Pass |  |
| 3COM | 3C905-TX PCI 100/10BPS | Pass |  |
| Intel | Pro-100 PILA 8465 PCI 100/10BPS | Pass |  |
| Surecom | EP-320X Rev.3B PCI 100/10BPS | Pass |  |
| On-board | RTL8139 | Pass |  |

E. On_Board Network Hardware (physical) Compatibility

System Information:

| DRAM Capacity & Brand(SIMM Qty) | Brand:NEC □1□2□4□8■16□32M ■60ns □70ns □_____ns Qty:□1 ■2 □3 □4 |
| --- | --- |
| System BIOS Ver. | ■AWARD □AMI Ver: |
| KB BIOS Version | □AMI- □VIA- ■Internal |
| Backplane PCA-61xx | □05P2 □08P3 □14P3 □ 03 □08 □14 ■N/A |
| O.S. Environment | □DOS5.0 □DOS6.0 ■DOS6.22 |
| VGA Card & BIOS | ■OnBoard □ □BIOS: |
| ### Power Supply | ■MPU-250W□SSH-250G □SSH-350G □ST-250AF |
| IDE HDD Type | □Max □Sea □Con ■Qua □Fuj □IBM : 1700AT |
| SCSI HDD Type | □Max □Sea □Con □Qua □Fuj □IBM: |
| Lan Card | ■OnBoard: BIOS: |
| CD-ROM Type | □NEC ■PANASONIC □PIONEER □CREATIVE : |
| Driver or Utility | □VGA □HDD ■Others : RTL-8139 |

Test Result:

| SUBJECT | Network Hardware Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Transceiver |  | ˇ |  |  |
| Receiver |  | ˇ |  |  |
| CRC error |  | N/A |  |  |
| Collision back-off generation |  | N/A |  |  |
| Auto-negotiation |  | ˇ |  |  |
| ### 100M Base-TX full duplex |  | ˇ |  |  |
| 100M Base-TX half duplex |  | ˇ |  |  |
| 10M Base-T full duplex |  | ˇ |  |  |
| 10M Base-T half duplex |  | ˇ |  |  |

F. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| AHA | 2940UW PCI | #### Pass |  |

G. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Sound Card(PCI Bus) |  |  |  |
| Creative | Audio PCI 4001043201 Rev.A | Pass |  |

H. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### 104 Module |  |  |  |
| AAEON | PCM-3820 rev. C (SSD) | Pass |  |
| AAEON | PCM-3820C (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3660 (ETHERNET) | Pass |  |
| AAEON | PCM-3420 (SCSI) | Pass |  |
| AAEON | PCM-3200(SOUND) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |

I. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Hitachi | DK226A-21U | Pass |  |
| Seagate | ST312768 | Pass |  |
| Seagate | ST33232A | Pass |  |
| Seagate | ST51270A | Pass |  |
| Seagate | ST32132A | Pass |  |
| Maxtor | 72700AP(2.7GB) | Pass |  |
| Maxtor | 63500D4(3.5GB) | Pass |  |
| Maxtor | 63240D4(3.2GB) | Pass |  |
| Maxtor | 85120A8(5.1GB) | Pass |  |
| Fujitsu | M1638TAU | Pass |  |
| Quantum | 1700AT TM17A012 Rev.02-D | Pass |  |
| Quantum | 850AT TR85A351 REV.02-B A450E | Pass |  |
| WD | WDAC22100 | Pass |  |
| WD | WDAC33100 | Pass |  |
| ### IBM | DHEA-34330 | Pass |  |

J. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| Philips | PCA123CD | Pass |  |
| VUGEO | 624A-004 | Pass |  |
| Panasonic | CR-585-B | Pass |  |
| Pioneer | DR-A24X | Pass |  |
| FAX/Modem |  |  |  |
| ### LEMEL | 33.6 FAX/MODEM | Pass |  |

K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | Pass |  |
| ACER | 6511 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Burn-In Under Temp. -40℃〜85℃(IEC-68-2-30)

| Test Software | Result | Note |
| --- | --- | --- |
| ### AMIDIAG4.5 | Pass | Pentium 233 MMX |

B. Booting Stability Testing

Test Condition: Burn-In Under Temp. 0℃〜60℃(IEC-68-2-30)

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0 | Pentium 233 MMX |
| KB Reset 20 Times | 20 | 20 | 0 | ### Pentium 233 MMX |
| H/W Reset 1000 Times | 1000 | 1000 | 0 | Pentium 233 MMX |

4. Performance Testing

System Information:

| DRAM Type & Capacity | Mitsubish M5M417400CJ 16M*2 |
| --- | --- |
| VGA Card | OnBoard (C&T 65554) |
| HDD TYPE | Quantum ST3240 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Pentium MMX-200 | Pentium MMX-233 | Cyrix 6X86 M1 PR200 | Cyrix 6X86 M2 PR200 | Cyrix 6X86 M2 PR233 | Cyrix 6X86 M2 PR300 | AMD K6-200 | AMD K6-233 | AMD K6-266 | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |
| CPU | 1311.80 | 1530.42 | 1813.87 | 2072.87 | 2331.10 | 2900.97 | 2025.13 | 2056.46 | 2741.97 | Mhz |
| FPU | 4010.90 | 4677.80 | 1966.51 | 2396.41 | 2646.92 | 3357.29 | 3221.68 | 3282.16 | 4376.26 | Mhz |
| VIDEO | 6922 | 6922 | 8474 | 8474 | 8623 | 8623.00 | 6971 | 6971 | 6922.00 | Chs/sec |
| Norton 8.0 SI | 680.6 | 794 | 1020.4 | 1133.9 | 1275.4 | 1235.1 | 1190.3 | 1190.3 | 1587.1 | ID |
| Power Meter 2.02 | 94.8 | 109.4 | 101.6 | 113.8 | 129.3 | 167.3 | 149.7 | 149.7 | 203.1 | Mips |
| PcBench 9.0 CPUmark16 | 454.73 | 514.33 | 404.74 | 503.74 | 540.65 | 672.89 | 590.88 | 594.46 | 743.74 |  |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |
| Business Winstone |  |  |  |  |  |  |  |  |  |  |
| High-End Winstone | 14.9 | 16.1 | 13.2 | 15.7 | 19.1 | 20.7 | 14.5 | 17.9 | 19.7 |  |
| WinBench 98 |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 75.4 | 79.4 | 65.3 | 109 | 124 | 104 | 79.1 | 82.8 | 77.5 | MP/s |
| ###### CPUmark 32 | 256 | 267 | 245 | 297 | 436 | 504 | 296 | 467 | 518 |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 12656 | 12752 | 11056 | 13184 | 12400 | 14512 | 13392 | 13904 | 13728 | KB/sec |
| Performance ID | 80.83 | 81.45 | 71.30 | 85.43 | 80.43 | 92.42 | 85.26 | 89.30 | 87.82 |  |
| WINTECH 1.2 (1024*768*256) | 162.08 | 172.37 | 150.45 | 211.84 | 207.43 | 189.21 | 164.49 | 193.84 | 173.08 |  |
| ### PS.CPU type of Cyrix PR-233/PR-300 & AMD K6-266 HDD is Segate ST34520A |  |  |  |  |  |  |  |  |  |  |

System Information:

| DRAM Type & Capacity | HITACHI 5165165FLTT6 32M*2 |
| --- | --- |
| VGA Card | OnBoard (C&T 65554) |
| HDD TYPE | IBM DPTA-372050 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 / Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AMD K6-2 300 |  |  |  |  |  |  |  |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |
| CPU | 3037.72 |  |  |  |  |  |  |  |  | Mhz |
| FPU | 4786 |  |  |  |  |  |  |  |  | Mhz |
| VIDEO | 6971 |  |  |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 5062.4 |  |  |  |  |  |  |  |  | ID |
| Power Meter 2.02 | 237.0 |  |  |  |  |  |  |  |  | Mips |
| PcBench 9.0 CPUmark16 | 853.99 |  |  |  |  |  |  |  |  |  |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 22.8 |  |  |  |  |  |  |  |  |  |
| High-End Winstone | 25.5 |  |  |  |  |  |  |  |  |  |
| WinBench 98 |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 139 |  |  |  |  |  |  |  |  | MP/s |
| ###### CPUmark 32 | 590 |  |  |  |  |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 15408 |  |  |  |  |  |  |  |  | KB/sec |
| Performance ID | 99.91 |  |  |  |  |  |  |  |  |  |
| WINTECH 1.2 (1024*768*256) | 262.14 |  |  |  |  |  |  |  |  |  |
| ### Note : |  |  |  |  |  |  |  |  |  |  |

5.Power Consumption

System Information:

| DRAM Type & Capacity | Panassonic MN4117405CSJ-06 32M*2 |
| --- | --- |
| VGA Card | OnBoard C&T65554 |
| ### DiskOnChip | MD2200-D08 |
| O.S. | DOS Mode |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result:

| Mode Type ## Processor Type | Normal Mode(+5V) | Suspend Mode(+5V) | Unit |
| --- | --- | --- | --- |
| # Intel Pentium MMX-200 | 4.4 | 2 | Amp/Max |
| Intel Pentium MMX-233 | 4.8 | 2 | Amp/Max |
| Cyrix M1 PR200 | 8 | 3.8 | Amp/Max |
| Cyrix M2 PR200 | 7.2 | 1.6 | Amp/Max |
| Cyrix M2 PR233 | 7.2 | 1.6 | Amp/Max |
| Cyrix M2 PR300 | 9 | 1.6 | Amp/Max |
| AMD K6 200 | 6 | 1.7 | Amp/Max |
| AMD K6 233 | 8.8 | 1.8 | Amp/Max |
| AMD K6 266 | 3.9 | 1.6 | Amp/Max |
| AMD K6-2 300 | 3.3 | 3.2 | Amp/Max |

Note :

1.Suspend Model all function off.

5. Margin Testing

A. Temperature Test (Software: AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 85℃ | Pentium 233 MMX |
| Low Temperature Margin (Burn-In 2 Hours) | -40℃ | Pentium 233 MMX |

B. DC Voltage Test (Software: AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.25V | Pentium 233 MMX |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.7V | ### Pentium 233 MMX |

C. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device : HD Seagate ST33232A

Slave Device : CD-ROM VUGEO 624A-204

Secondary IDE Port:

Master Device : N/A

Slave Device : N/A

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |