SBC-410A

QE Compatibility Test Report

## Release Date: Aug. 23, 1999

____________             ____________             ____________            ____________

# Issue Stamp                     QA Manager                  QE Manager                  Test Engineer

Contents

1. Margin -------------------------------------------------------------------------------------------------------------- 2

Includes temperature , DC voltage and IDE cable length tests.

2. Stability ------------------------------------------------------------------------------------------------------------- 3

Includes burn-in and booting tests .

3. Performance ------------------------------------------------------------------------------------------------------- 4

Includes performance indexes using various performance test software programs .

4. Hardware  compatibility ---------------------------------------------------------------------------------------- 5

Includes CPU , DRAM , adapter card and HDD compatibility tests.

5. Software compatibility ------------------------------------------------------------------------------------------ 12

Includes various OS and application software tests .

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

1. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 75℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -35℃ |  |

B. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.8V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4V |  |

C. IDE Drive Capability 1

### Test Equipment :

### Master Device: HDD Fujitsu M1638TAU

Slave Device: CD-ROM  Creative CD820E

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |

2. Stability Testing

### A. Burn-In Stability Testing

Test Condition: Room Temp. 48 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

### B. Booting Stability Testing

| Test Method | #### Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0％ |  |
| KB Reset 20 Times | 20 | 20 | 0％ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0％ |  |

### C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | ##### Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0％ |  |
| H/W Reset 1000Times | 1000 | 1000 | 0％ |  |

3. Performance Testing

System Information:

| DRAM Type & Capacity | 16 MB |
| --- | --- |
| VGA Card | OnBoard (C&T 65545) |
| HDD Type | Seagate 51080A |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | #### CPU Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | ###### IBM 5X86 100 | AMD 5X86 133 | AMD DX4 100 |  |  | #### Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 420.91 | 443.97 | 332.90 |  |  | #### Mhz |
| FPU | 1113.38 | 1171.07 | 878.07 |  |  | Mhz |
| VIDEO | 1989.00 | 1989.00 | 1989.00 |  |  | Chs/sec |
| Norton 8.0 SI | 262.7 | 286.7 | 215.0 |  |  | ID |
| Power Meter 2.02 | 26.8 | 40.6 | 30.9 |  |  | Mips |
| PcBench 9.0 CPUmark16 | 80.19 | 68.49 | 57.84 |  |  |  |
| H.E. WinStone 96 1.0 | 24 | 23.3 | 15.1 |  |  |  |
| WinBench 96 Ver 1.0 |  |  |  |  |  |  |
| Graphic WinMark 96 | 4.63 | 4.61 | 4.18 |  |  | MP/s |
| CPUmark 16 | 71.3 | 75.5 | 63.3 |  |  |  |
| CPUmark 32 | 68.1 | 70.6 | 60.5 |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 1008 | 1008 | 1008 |  |  | #### KB/sec |
| Performance ID | 11.50 | 9.64 | 11.50 |  |  |  |
| WINTECH 1.2 | 2.36 | 2.96 | 2.27 |  |  |  |
| Power consumption (+5V in normal mode) | 2.09 | 2.26 | 2.19 |  |  | A |
| Note |  |  |  |  |  |  |

4. Hardware Compatibility

### A. Component Compatibility- CPU

486 CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | 486DX4-100M | Pass |  |
| Cyrix | 486DX2-66M | Pass |  |
| Cyrix | 486DX2-80M | Pass |  |
| AMD | 5X86-P75 | Pass |  |
| ST | 486DX4-100(33M) | Pass |  |

### B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| HYNDAI | HY5117400AJ-70 16M | Fail |  |
| SAMSUNG | KM44C4104AK-6 16M EDO RAM | Fail |  |
| NEC | 4217405-60 16M EDO RAM | Pass |  |
| NEC | 424100-70 16M | Fail |  |
| NEC | 424400-70 4M | Pass |  |

### C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| TopView | ET 4000 ISA | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| 3COM | 3C509 | Pass |  |
| Novell | NE2000 | Pass |  |
| Novell | NE2000 + | Pass |  |
| IBM | Token 16 | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| AHA | 1542CP | Pass |  |

### F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Sound Card |  |  |  |
| Creative | AWE 32 | Pass |  |
| Creative | SB 16 Value PNP | Pass |  |
| Creative | SB 16 Basic Edition | Pass |  |

### G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| AAEON | PCM-3520 (VGA) | Pass |  |
| AAEON | PCM-3820 rev. C (SSD) | Pass |  |
| AAEON | PCM-3820C (SSD) | Pass |  |
| AAEON | PCM-3810 (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) | Pass |  |

### H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Quantum | FB1280AT | Pass |  |
| MAXTOR | 7540 AV | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| Philip | PCA123CD | Pass |  |
| NEC | CDR1400A | Pass |  |
| Pioneer | DR-U124X SCSI | Pass |  |

### J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| Chicony | KB-5311 | Pass |  |
| FOCUS | 101KB | Pass |  |
| Mouse |  |  |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse SUPER | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

5. Software Compatibility

### A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 3.12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS NT 2.51 W/S |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS NT 4.0 SRV |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |

### C. Windows Application

| Application / Test Item | #### Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

### D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | #### Result | Note |
| --- | --- | --- | --- | --- |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | N/A |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | N/A |  |  |
|  | 24 * 24 Font Mode | N/A |  |  |

### E. Diagnostics Software Testing

| Test Item / | QAFE5.29 |  |  | AMIDIAG4.5/5.1 |  |  | CHECKIT3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Software | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Hard Drive | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| COM Port | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  | Pass |  |  |  |  |  |
| Plug & play | N/A |  | Fail |  |  | N/A |  |  |  |  |  |