AAEON

ISO-9001/ISO-14001 Certified

Industrial Automation PCs

SBC-455B

### QE Compatibility Test Report

Release Date : Jun. 08, 1999

__________     __________    __________     __________

# Issue Stamp                     QA Manager                 QE Manager                  Test Engineer

Contents

1. Margin ---------------------------------------------------------------------------------------------------------- 3

Includes temperature , DC voltage and IDE cable length tests.

2. Stability --------------------------------------------------------------------------------------------------------- 4

Includes burn-in and booting tests .

3. Performance --------------------------------------------------------------------------------------------------- 5

Includes performance indexes using various performance test software programs .

4. Hardware  compatibility ------------------------------------------------------------------------------------- 6

Includes CPU , DRAM , adapter card and HDD compatibility tests.

5. Software compatibility ---------------------------------------------------------------------------------------- 8

Includes various OS and application software tests .

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

1. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 75℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -35℃ |  |

B. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 6.0V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4V |  |

## C. IDE Drive Capability

Test Equipment

Master Device:

HDD Quantum FB1280A

Slave Device: CD-ROM  Creative CD620E

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |

2. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

## B. Booting Stability Testing

| Test Method | #### Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0％ |  |
| KB Reset 20 Times | 20 | 20 | 0％ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0％ |  |

## C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0％ |  |
| H/W Reset 1000Times | 1000 | 1000 | 0％ |  |

3. Performance Testing

System Information:

| DRAM Type & Capacity | 16 MB |
| --- | --- |
| VGA Card | OnBoard (C&T 65545) |
| HDD TYPE | Seagate 51080A |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AMD 5X86-133 | CYRIX 5X86-100 | AMD DX4-100 | INTEL DX4-100 |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 443.98 | 420.88 | 332.97 | 358.06 |  | Mhz |
| FPU | 1171.15 | 1113.26 | 878.21 | 879.31 |  | Mhz |
| VIDEO | 18204.00 | 18204.00 | 18204.00 | 16384.00 |  | Chs/sec |
| Norton 8.0 SI | 286.7 | 262.7 | 215 | 211.4 |  | ID |
| Power Meter 2.02 | 40.6 | 17.2 | 31.6 | 23.9 |  | Mips |
| PcBench 9.0 CPUmark16 | 101.11 | 43.65 | 57.85 | 61.75 |  |  |
| H.E. WinStone 96 1.0 | 28.7 | 13.2 | 18.4 | 18.9 |  |  |
| WinBench 96 Ver 1.0 |  |  |  |  |  |  |
| Graphic WinMark 96 | 4.14 | 3.23 | 2.33 | 6.7 |  | MP/s |
| CPUmark 16 | 77.2 | 65.9 | 61.0 | 56 |  |  |
| CPUmark 32 | 71.7 | 62.6 | 58.6 | 54.4 |  |  |
| Coretest 3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 1008 | 1008 | 1168 | 3200 |  | KB/sec |
| Performance ID | 11.50 | 12.11 | 12.45 | 24.65 |  |  |
| WINTECH 1.2 | 4.57 | 3.06 | 4.52 | 6.76 |  |  |
| Power consumption (+5V in normal mode) | 1.71 | 1.65 | 1.66 | 1.60 |  | A |
| ##### Note |  |  |  |  |  |  |

4. Hardware Compatibility

A. Component Compatibility- CPU

486 CPU Device List:

| ###### Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | 486DX2-66M | Pass |  |
| Intel | 486DX4-100M | Pass |  |
| Cyrix | 486DX2-66M | Pass |  |
| Cyrix | 486DX2-80M | Pass |  |
| ###### Cyrix | 5X86-100M | Pass |  |
| AMD | 486DX2-66M | Pass |  |
| AMD | 486DX4-100M | Pass |  |
| AMD | 5X86-P75 | Pass |  |
| ST | 486DX2-66M | Pass |  |
| ST | 486DX4-100(33M) | Pass |  |

## B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| TOSHIBA | TC514256AJ-70 1M | Pass |  |
| PANASONIC | MN4117400BSJ-06 32M | Pass |  |
| PANASONIC | MN4117400CSJ-06 16M | Pass |  |
| MITSUBISH | M5M417400CJ-7 16M | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| HYNDAI | HY5117400AJ-70 16M | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| SAMSUNG | KM44C4104AK-6 16M EDO RAM | Pass |  |
| SAMSUNG | KM41C16000BK-6 64M | Pass |  |
| VANGUARD | VG264400BJ-6 8M | Pass |  |
| NEC | 4217405-60 16M EDO RAM | Pass |  |
| NEC | 424100-70 16M | Pass |  |
| NEC | 424400-70 4M | Pass |  |
| NEC | 4218165-60 8M(×4) EDO RAM | Pass |  |
| NEC | 4217400-60 16M | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ###### ISA BUS |  |  |  |
| on-board | 65548 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| 3COM | 3C509 | Pass |  |
| Novell | NE2000 | Pass |  |
| Novell | NE2000 + | Pass |  |
| SVAC | ETHER 16 | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| AHA | 1542CF | Pass |  |

F. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| ###### AAEON | PCM-3820 rev. C (SSD) | Pass |  |
| AAEON | PCM-3810 (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) | Pass |  |

## G. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Conner | CFS425A 425M | Pass |  |
| WD | 22100 | Pass |  |
| Fujitsu | M1638TAU | Pass |  |
| Quantum | 850AT | Pass |  |
| Quantum | TB540S SCSI | Pass |  |
| Maxtor | 7540AV 540M | Pass |  |
| Segate | ST 51270A | Pass |  |
| Segate | ST 32132A 2.1GB | Pass |  |

## H. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 620E | Pass |  |
| Creative | CD 820E | Pass |  |
| Panasonic | CR-585B | Pass |  |
| Philips | PCA123CD | Pass |  |
| VUGEO | 624A-204 | Pass |  |
| Printer |  |  |  |
| HP | Laser jet iii | Pass |  |

## I. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | Pass |  |
| ACER | 6511 | Pass |  |
| FOCUS | 101KB | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

5. Software Compatibility

## A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 3.12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS NT 4.0 W/S |  |  |  |  |  |  |  |  |  |  | N/A |  |
| WINDOWS NT 4.0 SRV |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows NT3.51 |  |  |  |  |  |  |  |  |  |  | N/A |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |

## C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

## D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |

E. Diagnostics Software Testing

| Test Item / Software | QAFE5.29 |  |  | AMIDIAG5.11 |  |  | AMIDIAG5.1 |  |  | CHECKIT3.0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Video Adapter | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Hard Drive | N/A |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Keyboard | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| COM Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| LPT Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Plug & play | Pass |  | Pass |  |  | Pass |  |  | N/A |  |  |