SBC-556

QE Compatibility Test Report

Release Date: Aug. 26, 1999

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Notice

1. Software compatibility -------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------- 11

Includes burn-in and booting tests.

4. Performance --------------------------------------------------------------------------------------------------- 12

Includes performance indexes using various performance test software programs.

5. Margin ---------------------------------------------------------------------------------------------------------- 13

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2/V (J) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 (E/J) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 (E) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Windows NT4.0W/S(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Windows NT4.0W/S(J) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Business/Access 97 | Pass |  | Pass |  |
| Business/Corel DRAW! 7 | Pass |  | Pass |  |
| Business/Excel 97 | Pass |  | Pass |  |
| Business/Lotus 1-2-3 97 | Pass |  | Pass |  |
| Business/Navigator | Pass |  | Pass |  |
| Business/Power Point 97 | Pass |  | Pass |  |
| Business/Quattro Pro 7 | Pass |  | Pass |  |
| Business/Task Switching 1 | Pass |  | Pass |  |
| Business/Task Switching 2 | Pass |  | Pass |  |
| Business/Word 97 | Pass |  | Pass |  |
| Business/Word Perfect 7 | Pass |  | Pass |  |
| High-End/AVS/Express 3.1 | Pass |  | Pass |  |
| High-End/Front Page 97 | Pass |  | Pass |  |
| High-End/Microstation 97 | Pass |  | Pass |  |
| High-End/Photoshop 4.0 | Pass |  | Pass |  |
| High-End/Premiere 4.2 | Pass |  | Pass |  |
| High-End/PV-WAVE 6.1 | Pass |  | Pass |  |
| High-End/Visual C++ 5.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Amidiag4.5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Com Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | Pentium MMX-200 | Pass |  |
| Intel | Pentium MMX-233 | Pass |  |
| AMD | K6-233 | Pass |  |
| AMD | K6-266 | Pass |  |
| AMD | K6-300 | Pass |  |
| Cyrix | 6X86 MX-PR200 | Pass |  |
| Cyrix | 6X86 MX-PR233 | Pass |  |
| Cyrix | M2 6X86 MX-300GP | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PANASONIC | MN4117400BSJ-06 32M FPM RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| PANASONIC | W/P PP716M66 16M FPM RAM | Pass |  |
| NEC | 4217405-60 16M EDO RAM | Pass |  |
| PANASONIC | MN414400BSJ-07 8M FPM RAM | Pass |  |
| LGS | GM71C17403CJ6 32M EDO RAM | Pass |  |
| NEC | 4218165-60 8M EDO RAM | Pass |  |
| MICON | MT4C4M4E8DJ-6 16M EDO RAM | Pass |  |
| TI | TMS417409ADJ-6 16M EDO RAM | Pass |  |
| ### MITSUBISHI | M5M418165CJ-6 8M EDO RAM | Pass |  |
| LGS | GM71C17403CJ6 16M EDO RAM | Pass |  |
| LGS | GM71C18163CJ6 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164BJC-60 8M EDO RAM | Pass |  |
| PANASONIC | MN4117405CSJ-6 32M EDO RAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On Board | C&T 69000 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### On Board | Realtek RTL8139 | Pass |  |
| ISA Bus |  |  |  |
| 3COM | 3C509 | Pass |  |
| INTEL | Ether Express PRO/100B | Pass |  |

E. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MPEG Card |  |  |  |
| LEMEL Magic | ### MPEG Card | Pass |  |
| ### Sound Card |  |  |  |
| ### Creative | SB 16 Value PNP | Pass |  |
| Creative | AWE 64 PNP | Pass |  |

F. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA Card |  |  |  |
| AAEON | PCD-894A | Pass |  |
| AAEON | PCD-895A | Pass |  |
| AAEON | RDM-1010A | Pass |  |
| AAEON | RDM-1020N | Pass |  |
| 104 Module |  |  |  |
| AAEON | PCM-3820/C | Pass |  |
| AAEON | ### PCM-3820-2 | Pass |  |
| AAEON | PCM-3110B | Pass |  |
| AAEON | PCM-3111 | Pass |  |
| AAEON | PCM-3420 | Pass |  |
| AAEON | PCM-3200 | Pass |  |
| AAEON | PCM-3600 | Pass |  |
| AAEON | PCM-3660 | Pass |  |

G. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Quantum | ST 32A013 | Pass |  |
| Seagate | ST34520A | Pass |  |
| Quantum | Big-Foot | Pass |  |
| Fujitsu | MPC3032AT | Pass |  |
| Seagate | ST31230N SCSI | Pass |  |
| Quantum | 540-1049/16/63 SCSI | Pass |  |

H. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| VUGEO | 624A-004 | Pass |  |
| PANSONIC | CR-585-B | Pass |  |
| SAMPO | CDM-T631 | Pass |  |
| TEAC | CD-532E | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 33.6 HIGH SPEED MODEM | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| ACER | 6311-TW | Pass |  |
| BTC | KB-5121 | Pass |  |
| LEMEL | KB-5122 | Pass |  |
| BTC | 7932 (USB) | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |
| Logitech | First Mouse | Pass |  |
| DEXIN | A2U800A (USB) | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG 4.5 | Pass |  |

B. Booting Stability Testing

Test Condition: Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 999 | 0.1﹪ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0﹪ |  |
| KB Reset 20 Times | 20 | 20 | 0﹪ |  |

C. Burn-In Stability Testing

Test Condition: Burn-In Under -40℃~85℃ Burn-In 30 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAD 4.5 | #### Pass |  |

D. Booting Stability Testing

Test Condition: Under -40℃~85℃ Burn-In 30 Hours

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 999 | 0.1﹪ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0﹪ |  |
| KB Reset 20 Times | 20 | 20 | 0﹪ |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | NEC 4217405-60 16MB X2 |
| --- | --- |
| VGA Card | On Board (C&T69000) |
| HDD TYPE | Quantum ST32A013 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 , Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=hing Files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Pentium MMX-200 | Pentium MMX-233 | AMD K6-233 | AMD K6-266 | AMD K6-300 | AMD K6-2 300 | Cyrix6x86MXPR 200 | Cyrix M2 6X86-233 | Cyrix M2 6x86-300 | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |
| ### CPU | 1311.80 | 1530.42 | 2362.62 | 2700.20 | 3037.72 | 3037.72 | 2072.15 | 2325.01 | 2900.49 | Mhz |
| FPU | 4010.94 | 4679.38 | 3765.85 | 4303.83 | 4841.84 | 4841.84 | 2379.26 | 2567.51 | 3353.85 | Mhz |
| VIDEO | 15123 | 15123 | 15123 | 15123 | 15123 | 15123 | 19660 | 19660 | 19660 | Chs/sec |
| Norton 8.0 SI | 680.6 | 794.0 | 1388.7 | 1587.1 | 5062.4 | 5062.4 | 1133.9 | 1272.6 | 1235 | ID |
| Power Meter 2.02 | 105.3 | 123.6 | 177.7 | 203.1 | 237.0 | 237.0 | 113.8 | 129.3 | 158 | Mips |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 17.3 | 18.3 | 21.1 | 20.1 | 20.6 | 20.6 | 17.6 | 18.4 | 20.5 |  |
| High-End Winstone | 17.4 | 18.3 | 22.4 | 19.4 | 20.1 | 19.6 | 17.3 | 18.1 | 20.1 |  |
| Winbench 98 |  |  |  |  |  |  |  |  |  |  |
| ###### High-End/Business Graphics Winmark98 | 107/82.8 | 116/90.1 | 120/96.6 | 134/106 | 142/108 | 149/112 | 124/96.4 | 129/100 | 153/120 | MP/s |
| ###### Cpumark 32 | 404 | 437 | 518 | 535 | 579 | 598 | 420 | 449 | 506 |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 14032 | 14048 | 14336 | 15568 | 15472 | 15504 | 13312 | 13232 | 14864 | KB/sec |
| Performance ID | 90.57 | 90.57 | 92.38 | 99.71 | 99.14 | 99.16 | 86.28 | 85.81 | 95.52 |  |
| WINTECH 1.2 | 306.51 | 332.67 | 328.93 | 345.16 | 370.91 | 378.31 | 312.04 | 313.75 | 369.42 |  |
| Note |  |  |  |  |  |  |  |  |  |  |

5. Margin Testing

A. Temperature Test (Software: AMIDIAG4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -40℃ |  |

B. Temperature Test (Boot Up)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -40℃ |  |

C. DC Voltage Test (Software:Qaplus/fe 5.29)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.25 |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.75 |  |

D. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device: HDD Quantum ST32A013

Slave Device: CD-ROM Vugeo 624A-004

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | Panasonic MN4117405CSJ-06 32MBx2 |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| DiskOnChip | M-System 2000 24MB |
| ### VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Process Type | Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # PENTIUM MMX-200 | 4.4 | 1.8 | Amp/Max |
| PENTIUM MMX-233 | 4.6 | 1.8 | Amp/Max |
| AMD K6-233 | 9.2 | 1.6 | Amp/Max |
| AMD K6-266 | 3.4 | 1.4 | Amp/Max |
| AMD K6-300 | 4 | 1.6 | Amp/Max |
| AMD K6/2 300 | 4.6 | 1.4 | Amp/Max |
| Cyrix M1 PR-200 | 7.2 | 1.6 | Amp/Max |
| Cyrix M2 PR-233 | 7.2 | 1.4 | Amp/Max |
| Cyrix M2 PR-300 | 9.2 | 1.4 | Amp/Max |

Note :

 1.Power supply voltage : ＋5V

   2.Suspend Model all function off