###### SBC-558

## QE Compatibility Test Report

Release Date : Oct. 14, 1999

__________        __________     __________     __________

# Issue Stamp                     QA Manager               QE Manager               Test Engineer

Summary of contents :

1. Cover Sheet																		1

2. Summary of contents																	2

3. Margin																					3

Includes temperature, DC voltage and IDE cable length tests.

4. Stability 																				4

Includes burn-in and booting tests .

A. Burn-In Testing(25℃/24hrs)															4

B. Burn-In Testing(Climatic Sequence/48hrs)												4

C. Booting Testing(25℃)																4

D. Booting Testing(Damp Heat,Cyclic/24hrs)													4

5. Performance																			5

Includes performance indexes using various performance test software programs .

6. Power consumption data																6

Includes +5V current rate .

7. Hardware compatibility																7

Includes CPU , DRAM , adapter card and HDD compatibility tests.

A. Component Compatibility- CPU														7

B. Component Compatibility- DRAM SIMM Module										7

C. Adapter Compatibility- VGA															7

D. Adapter Compatibility- LAN															7

E. Adapter Compatibility- SCSI Card														7

F. Adapter Compatibility- Other Card														8

G. Adapter Compatibility- Industry Control Card												8

H. Peripheral Compatibility- HDD Device													8

I. Peripheral Compatibility- Storage Device													9

J. Peripheral Compatibility- Storage Device													9

K. Peripheral Compatibility- Input Device													10

L. On_Board Network Hardware (physical) Compatibility										11

8. Software compatibility																13

Includes various OS and application software tests.

A. O.S. Compatibility  -																	13

B. Microsoft Windows & IBM OS/2 Compatibility												13

C. Microsoft Windows 98 Hardware Compatibility Test(HCT)										14

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

Quality Engineering Department of AAEON TECHNOLOGY  INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Hsin-Tien City, Taipei, Taiwan , ROC

Tel: 886-2-29121234

Fax:886-2-29130792

886-2-29130774

1. Margin Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4516821AG5-A10-9NF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 1.0 (08/18/1999) |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard □ |
| DOC Type | M-System DOC2000 □4MB □8MB □16MB ■24MB □___MB |

Test Result:

| High Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU:Tillamook MMX 266MHz |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | □ QAPlus/FE V5.29 |  |  |  |  | ■ AMIDIAG V5.11 |  |  |  |  |
| ■ 55℃ | ■ 60℃ |  | ■ 65℃ | ■ 70℃ |  | ■ 75℃ |  | ■ 80℃ | ■ 85℃ |  |  |

| Low Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU:Tillamook MMX 266MHz |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | □ QAPlus/FEV5.29 |  |  | ■ AMIDIAG V5.11 |  | ■5℃ |  |  | ■0℃ |  |
| ■-5℃ | ■-10℃ |  | ■-15℃ | ■-20℃ |  | ■-25℃ |  | ■-30℃ | ■-35℃ |  | ■-40℃ |

| High Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU:Tillamook MMX 266MHz |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 5.20V | ■Bgrade: 5.25V | ■CGrade: 5.30V | □DGrade: 5.35V | □EGrade: 5.4V |

| Low Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU:Tillamook MMX 266MHz |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 4.80V | ■Bgrade: 4.75V | ■CGrade: 4.70V | ■DGrade: 4.65V | ■EGrade: 4.6V |

2﹒Stability Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4516821AG5-A10-9NF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 1.0 (08/18/1999) |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard □ |
| DOC Type | M-System DOC2000 □4MB □8MB □16MB ■24MB □___MB |

- Burn-In Testing (24Hours)

Test Condition : Under Room Temp. 25℃

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| QAPlus/FE V5.29 |  |  |  |

B. Burn-In Testing (48 Hours)

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-61)

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| #### QAPlus/FE V5.29 |  |  |  |

##### C. Booting Testing

Test Condition : Under Room Temp. 25℃

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 999 | 0.1 % | __70_Sec | __5__Sec |  |
| KB Reset 20 Times | 20 | 20 | % | _____Sec | _____Sec |  |
| H/W Reset 1000Times | 1000 | 999 | 0.1 % | __63__Sec | __5__Sec |  |

D. Burn-In Booting Testing (24 Hours)

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-30)

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off | 2172 | 2172 | 0 % | __70_Sec | __8__Sec |  |
|  |  |  | % | _____Sec | _____Sec |  |

3﹒Performance Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:NEC D4564163G5-A10-9JF □8 □16 □32 ■64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver:1.0 (08/18/1999) |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 ■Windows NT □ |
| VGA Card | ■OnBoard □ |
| VGA Memory | □256K □512K □1M ■2M □4M □ |
| HDD Type | □Max □Sea □Con □Qua □Fuj ■WD :AC26400 –00RT |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD □Others : LAN |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Tillamook |  |  |  |  | Unit |
|  | MMX 266M |  |  |  |  |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 901.03 |  |  |  |  | MHz |
| ### FPU | 3441.22 |  |  |  |  | MHz |
| VIDEO | 15123 |  |  |  |  | Chs/sec |
| ### Norton 8.0 |  |  |  |  |  |  |
| SI | 264.7 |  |  |  |  | ID |
| ### Power Meter 2.02 |  |  |  |  |  |  |
| MIPS Test | 58.0 |  |  |  |  | Mips |
| WinStone 99 |  |  |  |  |  |  |
| HighEnd Graphic WinStone 99 | 10.1 |  |  |  |  |  |
| Business Graphic Winstone 99 | 11.0 |  |  |  |  |  |
| WinBench 99 |  |  |  |  |  |  |
| H.E. Graphic WinMark 99 | 137 |  |  |  |  | MP/s |
| Business Graphic WinMark 99 | 50.7 |  |  |  |  | TB/s |
| CPUmark 32 | 349 |  |  |  |  |  |
| ### Coretest V3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 12752 |  |  |  |  | KB/Sec |
| Performance ID | 82.37 |  |  |  |  |  |
| ### Wintech V1.2 |  |  |  |  |  |  |
| Overall WinTach RPM | 16.05 |  |  |  |  |  |

4﹒Power Consumption

System Information:

| DRAM Type & Capacity | NEC D4564163G5-A10-9JF |
| --- | --- |
| VGA Card | OnBoard : C&T 69000 |
| DOC or Floppy | M-Systems : MD2200-D24 |
| VGA Testing Mode | 640X480X16 |
| O.S. | MSDOS 6.22 |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

| Mode Type | CPU Type |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Tillamook MMX 266MHz |  |  |  |  |  |
| Normal Mode(+5V) | 2.2 |  |  |  |  | amp/max |
| Suspend Mode(+5V) | 1.2 |  |  |  |  | amp/max |

Note :

             1.Suspend Mode all function off.

5﹒Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Intel | Tillamook MMX 266MHz | ####  |  | OnBoard |

##### B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Model List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| NEC | D4564163G5-A10-9JF 64M |  |  | S-DRAM |
| NEC | D4516821AG5-A10-9NF 16M |  |  | S-DRAM |
| TOSHIBA | TC59SM716FT-80 128M |  |  | S-DRAM |
| SIEMENS | HYB39S64160AT-8 64M |  |  | S-DRAM |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | #### Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| OnBoard | C&T 69000 |  |  |  |

D. Adapter Compatibility- LAN Card

Action:

| 1. Load ODI Driver (Lsl,Ipxodi,Vlm Or Netx) | 2. Login To Server System |
| --- | --- |
| 3. Transfer Data Between SRV And WK(Mulity Task) | 4. logout |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Intel | PRO/100 + Management Adapter |  |  |  |
| 3Com | EtherLink III 3C509B |  |  |  |
| Intel | Ether Express PRO/100 |  |  |  |
| Realtek | RTL 8019 |  |  |  |
| Svec | Ethernet Card |  |  |  |

E. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| AHA | 1520B |  |  |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| MPEG Card |  |  |  |  |
| Lemel Magic | Mpeg Card |  |  |  |
| Sound Card |  |  |  |  |
| Creative | CT2502-SDQ |  |  |  |
| Creative | CT2511-SBT |  |  |  |
| Ess | ES1868F |  |  |  |

##### G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| 104 Module |  |  |  |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) |  |  |  |
| AAEON | PCM-3600(FAX/MODEM) |  |  |  |
| AAEON | PCM-3820 rev. C (SSD) |  |  |  |
| AAEON | PCM-3820C (SSD) |  |  |  |
| AAEON | PCM-3110 (PCMCIA MASTER) |  |  |  |
| AAEON | PCM-3660 (ETHERNET) |  |  |  |
| AAEON | PCM-3420 (SCSI) |  |  |  |
| AAEON | PCM-3111 (PCMCIA MASTER) |  |  |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Western Digital | AC26400 |  |  |  |
| Western Digital | AC28400 |  |  |  |
| Maxtor | 910801D5 |  |  |  |
| Fujitsu | MPC3032AT |  |  |  |
| Quantum | TM17A012 |  |  |  |
| Quantum | FIREBALL |  |  |  |
| Seagate | ST34520W SCSI |  |  |  |
| ### Seagate | ST51270A |  |  |  |

##### I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| CD/ROM |  |  |  |  |
| Acer | 650P-072 |  |  |  |
| Philips | CDD3600 |  |  |  |
| ZIP |  |  |  |  |
| Iomega | ZIP 100M |  |  |  |
| LS-120 |  |  |  |  |
| LS-120 | LKM-F934-1 |  |  |  |
| Fax/Modem |  |  |  |  |
| Rock Modem | Cyber Lander 2000 |  |  |  |
| Lemel | Rockwell 33.6 dpf |  |  |  |

J.Peripheral Compatibility- Others

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| RS-485 Port |  |  |  |  |
| Adam | ADAM-4011 |  |  |  |
| USB Port |  |  |  |  |
| Dexin | Mouse |  |  |  |
| Epson | Color 740 Print |  |  |  |
| Vcam | Capture |  |  |  |
| Tekcam | P35u |  |  |  |
| Wintek | AM-767-U Mouse |  |  |  |
| BTC | 7932 Keyboard |  |  |  |
| D-Link | DU-H4 HUB |  |  |  |
| HP | ScanJet 4200C |  |  |  |
| Parallel Port |  |  |  |  |
| IOMEGA | ZIP 100M |  |  | ECP,ECP+EPP |
| IR Port |  |  |  |  |
| ACT | IR 210L |  |  |  |

##### K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |
| Actis | SMK-8851 |  |  |  |
| Acer | 6311-TW |  |  |  |
| Mouse |  |  |  |  |
| Logitech | MouseMan |  |  |  |
| Microsoft | Serial Mouse |  |  |  |

L. On_Board Network Hardware (physical) Compatibility

System Information(Client Site):

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4516821AG5-A10-9JF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: 1.0 |
| O.S. Environment | ■DOS6.22 □WINNT____ □NOVELL_____ □__________ |
| VGA Card | ■OnBoard C&T 69000 □_____________________ |
| POWER SUPPLY | □MPU-250W□SSH-250G □SSH-350G ■BPS-1004 |
| IDE HDD TYPE | □Max □Sea □Con ■Qua □Fuj □IBM : TM17A012 |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| LAN CARD | ■OnBoard: GD82559 |
| CD-ROM TYPE | □NEC □PANASONIC □PIONEER □CREATIVE : |
| Cable/Cable Length |  |

System Information(Server Site):

| Board System | ■NOVELL 4.11 |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □8 □16 □32 ■64MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: 1.0 |
| VGA Card | ■OnBoard C&T 69000 □_____________________ |
| O.S. Environment | □DOS6.22 □WINNT____ ■NOVELL 4.11 □__________ |
| IDE HDD TYPE | □Max □Sea □Con □Qua □Fuj ■WD : AC26400 |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| LAN CARD | ■OnBoard: GD82559 |
| ### CPU Type | ■Intel Tillamook MMX 266 |
| CD-ROM TYPE | ■Acer □PANASONIC □PIONEER □CREATIVE : 650P-072 |
| Cable/Cable Length |  |

HUB Type: D-Link DU-H4

Test Result:

| SUBJECT | Network Hardware Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Transceiver |  |  |  |  |
| Receiver |  |  |  |  |
| Auto-negotiation |  |  |  |  |
| 100M Base-TX half duplex |  |  |  |  |
| 10M Base-T half duplex |  |  |  |  |
| SUBJECT | Network Operation System Function Testing |  |  |  |
| Test Item |  | Pass | Fail | Note |
| NetWare 4.11 |  |  |  |  |
| Windows NT 4.0 |  |  |  |  |
| SUBJECT | Network Performance Testing |  |  |  |
| Test Item |  | Result |  | Note |
| ### NWTEST 24HR |  |  |  |  |

| SUBJECT | Workstation Application Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Dos 6.22 for IPXODI |  |  |  |  |
| Dos 6.22 for package driver |  |  |  |  |
| Windows 3.1 for IPXODI |  |  |  |  |
| Windows 3.1 for package driver |  |  |  |  |
| Windows 95 for IPX/SPX |  |  |  |  |
| Windows 95 for NetBEUI |  |  |  |  |
| Windows 95 for TCP/IP |  |  |  |  |
| Windows 98 for IPX/SPX |  |  |  |  |
| Windows 98 for NetBEUI |  |  |  |  |
| Windows 98 for TCP/IP |  |  |  |  |
| Windows NT 4.0 for IPX/SPX |  |  |  |  |
| Windows NT 4.0 for NetBEUI |  |  |  |  |
| Windows NT 4.0 for TCP/IP |  |  |  |  |
| Transfer Data Between SRV And WK |  | N/A |  | Mulity Task 12 Hours(NOTE 1.) |

Note : 1.Mulity Task is that transfers files(copy/verify/delet) and VCD in the same time.

6﹒Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| ### MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2/V (JAPAN) |  |  |  |  |  |  |  |  |  |  |
| ### NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |
| ### Linux Slakware 4.0 |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM OS/ 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  | Ethernet Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| ### Windows 3.11 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| ###### Windows 95 (English) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 (Simplified-Chinese) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| Windows 95 (Japan) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| Windows 98(English) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 (Simplified-Chinese) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| Windows 98(Japan) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| ###### Windows Nt4.0w/S ###### (English) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Windows Nt4.0w/S (Simplified-Chinese) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| ###### Windows Nt4.0w/S (Japan) |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |
| ###### OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |

C. Microsoft Windows 98 Hardware Compatibility Test(HCT)

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □8 □16 □32 ■64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K ■512K □ |
| System BIOS Version | ■Award □AMI Ver: 1.0 |
| VGA Card & BIOS | □OnBoard □ □BIOS: |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns □DRAM ■VRAM □ □256K □512K □1M ■2M □4M □ |
| CPU Type | Tillamook-MMX 266MHz |
| HDD Type | □Max □Sea □Con □Qua □Fuj ■WD : AC26400-00RT |
| CD-ROM Type | □NEC □Panasonic □Pioneer □Creative ■Acer : 650P-072 |
| VGA Card & BIOS | On Board: C&T 69000 |
| Keyboard TYPE | Acer : 6311-TW |
| Mouse | Logitech: MouseMan |
| LAN CARD | On Board: GD82559 |
| Driver or Utility | ■VGA □HDD ■Others : LAN |

Test Result:

| HCT 7.6 Features Test | Pass | Fail | ### Remark |
| --- | --- | --- | --- |
| DMPS Test |  |  |  |
| Video Playback |  |  |  |
| BCT API Test |  |  |  |
| INT 13 Extension |  |  |  |
| Disk(Stress) Individual Drive |  |  |  |
| Disk –CD (Stress) |  |  |  |
| Disk (Stress) |  |  |  |
| Coprocessor(Stress) |  |  |  |
| Net Class Dynamic |  |  |  |
| PCI I/O Space |  |  |  |
| PCI 2.1 (BIOS) |  |  |  |
| PCI 2.1 (Card) |  |  |  |
| Unreported I/O |  |  |  |
| Unreported Memory |  |  |  |
| PnP BIOS |  |  |  |
| NOTE: |  |  |  |