SBC-770

QE Compatibility Test Report

Release Date: May. 12, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility --------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------------- 8

Includes burn-in and booting tests.

4. Performance --------------------------------------------------------------------------------------------------------- 9

Includes performance indexes using various performance test software programs.

5. Power consumption data-------------------------------------------------------------------------------------------11

Includes +5V current rate

6. Margin ----------------------------------------------------------------------------------------------------------------12

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2/V (JAPAN) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins | N/A |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  | Ethernet Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### WINDOWS 95 (ENGLISH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 95(JAPAN) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 98 (ENGLISH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### WINDOWS NT4.0W/S ###### (ENGLISH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### WINDOWS NT4.0W/S ###### (JAPAN) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Business/Access 97 | Pass |  | Pass |  |
| Business/Corel DRAW! 7 | Pass |  | Pass |  |
| Business/Excel 97 | Pass |  | Pass |  |
| Business/Lotus 1-2-3 97 | Pass |  | Pass |  |
| Business/Navigator | Pass |  | Pass |  |
| Business/Power Point 97 | Pass |  | Pass |  |
| Business/Quattro Pro 7 | Pass |  | Pass |  |
| Business/Task Switching 1 | Pass |  | Pass |  |
| Business/Task Switching 2 | Pass |  | Pass |  |
| Business/Word 97 | Pass |  | Pass |  |
| Business/Word Perfect 7 | Pass |  | Pass |  |
| High-End/AVS/Express 3.1 | Pass |  | Pass |  |
| High-End/Front Page 97 | Pass |  | Pass |  |
| High-End/Microstation 97 | Pass |  | Pass |  |
| High-End/Photoshop 4.0 | Pass |  | Pass |  |
| High-End/Premiere 4.2 | Pass |  | Pass |  |
| High-End/PV-WAVE 6.1 | Pass |  | Pass |  |
| High-End/Visual C++ 5.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Amidiag4.5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | #### Pass |  | #### Pass |  |  |  |  |  |  |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Com Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | Pentium II 300 | Pass | 100*3 |
| Intel | Pentium II 400 | Pass | 100*4 |
| Intel | Pentium II 450 | Pass | 100*4.5 |
| Intel | Pentium III 450 | Pass | 100*4.5 |
| Intel | Celeron 300A/66 | Pass | 66*4.5 |
| Intel | Celeron 366 | Pass | 66*5.5 |
| Intel | Celeron 400 | Pass | 66*5.5 |
| Intel | Celeron 433 | Pass | 66*5.5 |
| Intel | Celeron 466 | Pass | 66*5.5 |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| NEC | SDNX64M 64MB SDRAM | Pass | PC-100 |
| NEC | SDNX128M 128MB SDRAM | Pass | PC-100 |
| MICON | MT48LC2M8A1 32MB SDRAM | Pass | PC-100 |
| HYUNDA | HY57V658020 TC-10 128M SDRAM | Pass | PC-100 |
| ### LGS | GM72V661641CT7J 64MB | Pass | PC-100 |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On Board | C&T 69000 | Pass |  |

D. Adapter Compatibility- LAN Card

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### On Board | Realtek RTL8139 A | Pass |  |
| Surecom | Digital 21140-AF | Pass |  |
| 3COM | 3C905B | Pass |  |
| 3COM | 3C905 | Pass |  |
| Realtek | RTL8139 | Pass |  |
| ### Intel | SB 82558B | Pass |  |
| ISA Bus |  |  |  |
| Intel | PRO 100 ISA | Pass |  |

E. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| AHA | 2940UW | Pass |  |
| ISA Bus |  |  |  |
| ### AHA | 1520B | Pass |  |

F. Adapter Compatibility- Modem Card

Modem Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| SMART_LINK | ESS ES2898S | #### Pass |  |
| GVC | LUCENT 1646T00 | Pass |  |

G. Adapter Compatibility- SOUND Card

SOUND Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| Creative | ENSONIQ Audio PCI | Pass |  |
| ISA Bus |  |  |  |
| ### Creative | ### SB 16 Value PnP | Pass |  |
| Creative | AWE64 PnP | Pass |  |

G. Adapter Compatibility- MPEG Card

MPEG Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MPEG CARD |  |  |  |
| LEMEL Magic | MPEG Card | Pass |  |

H. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA Card |  |  |  |
| AAEON | PCD-894A | Pass |  |
| AAEON | PCD-895A | Pass |  |
| AAEON | RDM-1010A | Pass |  |
| AAEON | RDM-1020N | Pass |  |

I. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST34520A | #### Pass |  |
| WD | WDAC33100 | Pass |  |
| Quantum | ### Big-Foot TX4.0AT | Pass |  |
| Quantum | ST3240 AT | Pass |  |
| Quantum | TM17A012 | Pass |  |
| Fujitsu | MPC3032AT | Pass |  |
| IBM | DHEA-34330 | Pass |  |
| MAXTOR | 851204B | Pass |  |

J. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| VUGEO | 624A-004 24X | #### Pass |  |
| SAMPO | CDM-T631 36X | Pass |  |
| NEC | CDR-1901A 32X | Pass |  |
| PHILIPS | PCA123CD 8X | Pass |  |
| PANASONIC | CR-585-B 24X | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 33.6 HIGH SPEED MODEM | Pass |  |
| ZIP (ECP/EPP/SPP) |  |  |  |
| IOMEGA | ZIP 100M | Pass |  |
| MO |  |  |  |
| FUJTSU | MCB3064SS | Pass |  |
| DiskOnChip |  |  |  |
| Flushdisk | M-Systems DiskOnChip2000 24M | Pass |  |

K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| ACER | 6311-TW | Pass |  |
| LEMEL | KB-5122 | Pass |  |
| BTC | 7932 (USB) | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |
| DEXIN | A2U800A (USB) | Pass |  |
| Scanner |  |  |  |
| UMAX | Astra1200s | Pass |  |
| Adam (RS-485) |  |  |  |
| Advantech | ADAM-4011 | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG 4.5 | Pass |  |

B. Booting Stability Testing

Test Condition: Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0﹪ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0﹪ |  |
| KB Reset 20 Times | 20 | 20 | 0﹪ |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: -10℃~ 70℃ Humidity: 0% ~ 95% Burn-In 48 Hours

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 42 Hours | 4726 | 4726 | 0﹪ |  |
| H/W Reset 42 Hours | 1000 | 1000 | 0﹪ |  |

D. Burn-In Stability Testing

Test Condition: -10℃~70℃ Humidity: 0% ~ 90% Burn-In 24 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAD 4.5 32 Hours | Pass |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A80-9JF 64MBx2 SDRAM |
| --- | --- |
| VGA Card | On Board (C&T 69000) |
| HDD TYPE | Quantum ST32A013 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone98 & WinBench98) |
| O.S. | DOS 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high Files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Penitum II 300MMX | Pentium II 400MMX | Pentium II 450MMX | Celeron 300MMX |  | PentiumIII 450MMX | Celeron 366 | Celeron 400 |  | Celeron 433 |  | Unit |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU | 1873.85 | 2498.58 | 2810.94 |  | 1873.88 | 2810.91 | 2290.32 |  | 2498.56 |  | 2706.77 |  | Mhz |
| FPU | 5842.25 | 7790.05 | 8763.80 |  | 5815.30 | 8723.25 | 7107.73 |  | 7753.92 |  | 8400.13 |  | Mhz |
| VIDEO | 16384.0 | 17246.0 | 16948.0 |  | 14894.0 | 16948 | 16384 |  | 16384 |  | 16384 |  | Chs/sec |
| Norton 8.0 SI | 752.1 | 1002.9 | 1128.2 | 752.1 |  | 1128.2 | 919.3 | 1002.9 |  | 1086.4 |  | ID |  |
| Power Meter 2.02 | 142.2 | 189.6 | 218.8 | 142.2 |  | 218.8 | 177.7 | 189.6 |  | 218.8 |  | Mips |  |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 25.6 | 30.3 | 31.1 | 25.5 |  | 31.9 | 27.7 | 28.5 |  | 29.1 |  |  |  |
| High-End Winstone | 30.1 | 35.9 | 37.3 | 30 |  | 38.1 | 32.7 | 33.3 |  | 34.8 |  |  |  |
| WinBench 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphics WinMark98 | 143 | 202 | 217 | 160 |  | 225 | 190 | 202 |  | 215 |  |  |  |
| ###### Business Graphics WinMark98 | 113 | 153 | 164 | 126 |  | 168 | 145 | 153 |  | 161 |  | MP/s |  |
| ###### CPUmark 32 | 657 | 988 | 1090 | 599 |  | 1140 | 667 | 702 |  | 734 |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 6928 | 8608 | 8720 | 7040 |  | 8672 | 7408 | 7472 |  | 7472 |  | KB/sec |  |
| Performance ID | 48.28 | 58.28 | 58.95 | 48.86 |  | 58.4 | 51.23 | 51.26 |  | 51.35 |  |  |  |
| Note |  |  |  |  |  |  |  |  |  |  |  |  |  |

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A80-9JF 64MBx2 SDRAM |
| --- | --- |
| VGA Card | On Board (C&T 69000) |
| HDD TYPE | Quantum ST32A013 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone98 & WinBench98) |
| O.S. | DOS 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=hing Files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Celeron 466 | Celeron 500 | Celeron 533 | Pentium II 500MMX |  | Pentium II 550MMX |  |  |  |  |  | Unit |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU | 2915.02 | 3163.5 | 3373.92 |  | 3163.11 | 3479.42 |  |  |  |  |  |  | Mhz |
| FPU | 9046.36 | 7998.14 | 8531.29 |  | 7998.31 | 8798.18 |  |  |  |  |  |  | Mhz |
| VIDEO | 16384 | 20915 | 20915 |  | 21370 | 21370 |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 1170 | 1253.6 | 1337.1 | 1253.6 |  | 1379 |  |  |  |  |  | ID |  |
| Power Meter 2.02 | 237 | 237 | 258.5 | 237 |  | 284.4 |  |  |  |  |  | Mips |  |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 29.6 | 31.9 | 33.1 | 35.4 |  | 36.5 |  |  |  |  |  |  |  |
| High-End Winstone | 35.4 | 36.6 | 37.1 | 40.5 |  | 41.7 |  |  |  |  |  |  |  |
| WinBench 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphics WinMark98 | 226 | 227 | 234 | 236 |  | 253 |  |  |  |  |  |  |  |
| ###### Business Graphics WinMark98 | 169 | 173 | 178 | 180 |  | 191 |  |  |  |  |  | MP/s |  |
| ###### CPUmark 32 | 751 | 777 | 790 | 1250 |  | 1360 |  |  |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 7728 | 7632 | 7632 | 8800 |  | 8848 |  |  |  |  |  | KB/sec |  |
| Performance ID | 52.79 | 53.63 | 53.63 | 60.46 |  | 60.75 |  |  |  |  |  |  |  |
| ### Notes : CPU type of Intel Celeron 500MHz / 533MHz & PentiumIII 500MHz / 553MHz HDD is IBM ### DPTA-372050 |  |  |  |  |  |  |  |  |  |  |  |  |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A10-9JF 64M |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| Floppy | Teac FD-235HF |
| VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Processor Type | # Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # Pentium II MMX-300 | 8.4 | 2 | Amp/Max |
| Pentium II MMX-400 | 5.6 | 2.4 | Amp/Max |
| Pentium II MMX-450 | 6.2 | 2.4 | Amp/Max |
| Pentium III MMX-450 | 4.4 | 2 | Amp/Max |
| Celeron 300a | 4.4 | 2 | # Amp/Max |
| Celeron 366 | 5.2 | 2 | Amp/Max |
| Celeron 400 | 5.2 | 2 | Amp/Max |
| Celeron 433 | 5.6 | 2 | Amp/Max |
| Celeron 466 | 5.4 | 2 | Amp/Max |
| Celeron 500 | 5.7 | 2.3 | Amp/Max |
| Celeron 533 | 6 | 2.3 | Amp/Max |
| Pentium III MMX-500 | 6.4 | 2.6 | Amp/Max |
| Pentium III MMX-550 | 7 | 2.6 | Amp/Max |

Note:

 1.Power supply voltage : ＋5V

 2.SUSPEND Model all function off

5. Margin Testing

A. Temperature Test (Power On/Off)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -10℃ |  |

B. DC Voltage Test (Software: Qaplus/fe 5.29)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.25V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.71V |  |

C. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device: HDD Quantum Bigfoot TX4.0AT

Slave Device: CD-ROM NEC CDR-1901A

Secondary IDE Port:

Master Device: HDD WD WDAC33100

Slave Device: HDD Seagate ST34520A

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | 75CM |  |