STD-5094

QE Compatibility Test Report

Release Date: Apr. 19, 1999

__________   __________   __________   __________

# Issue Stamp         QA Manager        QE Manager         Test Engineer

Contents

1. Software compatibility ---------------------------------------------------------------------------------------- 3

Includes various OS and application software tests .

2. Hardware compatibility ---------------------------------------------------------------------------------------5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Performance ------------------------------------------------------------------------------------------------------7

Includes performance indexes using various performance test software programs.

4. Margin ------------------------------------------------------------------------------------------------------------ 8

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

- Software Compatibility

A.O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | KB Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | KB Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.11(E) |  |  |  |  |  |  |  |  |  |  |  (Note) |  |
| Windows 3.11 (Chinese-Simplified) |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows 95(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 (Chinese-Simplified) |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows 98 (Chinese-Simplified) |  |  |  |  |  |  |  |  |  |  | N/A |  |

Note. VGA driver for WIN311(English) was failure. Stiped Figure will be found when we moved the

windows or the mouse point.

Soulation:Add “redundancy=off” in SYSTEM.INI.

C. Windows Application

| ### Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D.DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | QAFE 5.29 |  | AMIDIAG 4.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Result | Fail | Result | Fail | Result | Fail |
| ##### Main Component |  |  |  |  |  |  |
| Memory |  |  |  |  |  |  |
| ##### Video Adapter |  |  |  |  |  |  |
| Floppy Drive |  |  |  |  |  |  |
| Hard Drive |  |  |  |  |  |  |
| Keyboard |  |  |  |  |  |  |
| COM Port |  |  |  |  |  |  |
| LPT Port |  |  |  |  |  |  |
| Plug & play |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### SGS | ### STPC CLIENT 66 | #### Pass | On board |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| NEC | D4265165G5-A60-7JF 32MB | Pass | On board |

C. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST312768 | Pass |  |
| Seagate | ST33232A | Pass |  |
| Maxtor | 90648D3 | Pass |  |
| Quantum | 3240AT | Pass |  |
| Quantum | Big foot 4.0AT | Pass |  |
| Quantum | Big foot 4320AT | Pass |  |

D. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| TEAC | CD-532E-B | Pass |  |
| VUGEO | 624A-004 | Pass |  |
| Panasonic | CR-585-B | Pass |  |
| Pioneer | DR-A24X | Pass |  |
| ZIP |  |  |  |
| ZIP | 100MB(ECP/EPP) | Pass |  |

E. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | Pass |  |
| ACER | 6511 | Pass |  |
| Mouse |  |  |  |
| Microsoft | ### Serial Mouse | Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |

## 3. Performance Testing

System Information:

| DRAM Type & Capacity | OnBoard 64MB NEC D4265165G5-A60-7JF 32MB |
| --- | --- |
| VGA Card | OnBoard (SGS STPC CLIENT 66 /Share VGA Menory) |
| HDD TYPE | Quantum ST3240 |
| VGA Testing Mode | 640X480X16 colors |
| O.S. | DOS V 6.22 & Windows 96 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |
| --- | --- | --- | --- | --- |
| Testing Utility | SGS STPC Client 66 |  |  |  |
| LandMark Speed 2.0 |  |  |  |  |
| CPU | 217.73 |  |  | Mhz |
| FPU | 617.30 |  |  | Mhz |
| VIDEO | 4292 |  |  | Chs/sec |
| Norton 8.0 SI | 112 |  |  | ID |
| Power Meter 2.02 | 19.1 |  |  | Mips |
| WinStone96 |  |  |  |  |
| Overall WinStone96 | 17.6 |  |  |  |
| WinBench96 |  |  |  |  |
| ###### Graphic WinMark96 | 2.17 |  |  | MP/s |
| CPU mark 16 | 50.7 |  |  |  |
| CPU mark 32 | 70.9 |  |  |  |
| Coretest 3.02 |  |  |  |  |
| DTR (Buffered Read) | 4240 |  |  | KB/sec |
| Performance ID | 32.76 |  |  |  |
| Power consumption |  |  |  |  |
| (+5V)DOS Mode Run SpeedCom Rev.2.0 | 1.5 |  |  | A |

4. Margin Testing

A. DC Voltage Test (Software: AMI_DIAG 4.51)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.25V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.7V |  |

B. IDE Drive Capability

Test Equipment

Primary IDE Port

Master Device: HD Seagate ST33232A

Slave Device: CD-ROM PIONEER DR-A24X

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |