| #### Backplane Test Report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Model | Rev | ## Hardware Description | Run Winstone99 Business v1.2 | ## Mulity Task | VCC ##### LED | -5V LED | +12V LED | -12V LED | VCC3X LED |  |
| ## BP-204SG – P1 | ## A0 | SBC-770 (PentiumⅡ 400) + YUAN MPG-200C DVD MPEG-2(PCI) +AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD + DVD Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP-206SG – P3 | A0 | SBC-770 (PentiumⅡ 400) + YUAN MPG-200C DVD MPEG-2 (PCI) + AHA 2940UW SCSI (PCI) + Intel 82557 LAN (PCI) + AHA 1520B SCSI(ISA) + Creative PNP 32bit Sound (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN + DVD Play | Pass | Pass | Pass | ## Pass | — |  |
| BP-208SG – P3 | A0 | SBC-770 (PentiumⅡ 400) + YUAN MPG-200C DVD MPEG-2(PCI) +AHA 2940UW SCSI (PCI) + Intel 82557 LAN (PCI) + AHA 1520B SCSI(ISA) +Creative PNP 32bit Sound (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN + DVD Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP– 206SI | A0 | SBC-656 (Celeron 366) + AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) + TAKON MR34PV 33.6K FAX/Modem (ISA) + 3 Com 3C509B LAN (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN +Sound Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP– 208SI | A0 | SBC-656 (Celeron 366) + AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) + TAKON MR34PV 33.6K FAX/Modem (ISA) + 3 Com 3C509B LAN (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to ### HDD by LAN +Sound Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP–202SG-P1 | A0 | SBC-770 (PentiumⅡ 400) +AHA 2940UW SCSI (PCI) | Pass | From the SCSI HDD copy a lot of data to IDE HDD | N/A | N/A | N/A | N/A | N/A |  |
| BP–205SG-P2 | ## A0 | SBC-770 (PentiumⅡ 400) + AHA 2940UW SCSI (PCI) + Intel 82557 LAN (PCI) + AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN +Sound Play | Pass | Pass | Pass | ## Pass | N/A |  |
| Note: 1. N/A : There is no power LED. 2. — : There is no 3.3V power input that 3.3V power LED can’t work. |  |  |  |  |  |  |  |  |  |  |
| #### Backplane Test Report |  |  |  |  |  |  |  |  |  |  |
| # Model | Rev | ## Hardware Description | Run Winstone99 Business v1.2 | ## Mulity Task | VCC ##### LED | -5V LED | +12V LED | -12V LED | VCC3X LED |  |
| ## BP-214SI | ## A0 | SBC-656 (Celeron 366) + AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) + TAKON MR34PV 33.6K FAX/Modem (ISA) + 3 Com 3C509B LAN (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to ### HDD by LAN +Sound Play | Pass | Pass | Pass | ## Pass | N/A |  |
| ## BP-214SG-P7 | A0 | SBC-770 (PentiumⅡ 400) + AHA 2940UW SCSI (PCI) + YUAN MPG-200C DVD MPEG-2 (PCI) + AHA 1520B SCSI (ISA) + Creative PNP 32bit Sound (ISA) + Intel EtherExpressTM PRO/100 LAN (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN + DVD Play | Pass | Pass | Pass | ## Pass | ## Pass |  |
| BP-214DI | A1.0 | System 1 SBC-656 (Celeron 500) + Creative PNP 32bit Sound (ISA) + Intel EtherExpressTM PRO/100 LAN (ISA) + TAKON MR34PV 33.6K FAX/Modem (ISA) System 2 SBC-558 (Onboard Tillamook 266 MMx) + AHA 1520B SCSI (ISA) + Creative VIBRA 16XV (ISA) | ## Pass | System 1 Data stream deliver toHDD by LAN +Sound Play System 2 From the SCSI HDD copy a lot of data to IDE HDD + VCD Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP-213DG-P6 | A1.0 | System 1 SBC-770 (PentiumⅢ 550) + D-Link DFE-530TX LAN (PCI) + AHA 2940U2W SCSI (PCI) + Creative ENSONIQ ES1370 AUDIO(PCI) System 2 SBC-770 (PentiumⅢ 500) + Creative PNP 32bit Sound (ISA) + D-Link DFE-540TX LAN (PCI) + GENUINEGM56-AMI2019 V.90 56.6K FAX/Modem (PCI) | Pass | System 1 Data stream deliver toHDD by LAN +Sound Play System 2 From the SCSI HDD copy a lot of data to IDE HDD + VCD Play +Data stream deliver to HDD by LAN | Pass | Pass | Pass | ## Pass | ## Pass |  |
| Note: 1. N/A : There is no power LED. 2. — : There is no 3.3V power input that 3.3V power LED can’t work. |  |  |  |  |  |  |  |  |  |  |
| #### Backplane Test Report |  |  |  |  |  |  |  |  |  |  |
| # Model | Rev | ## Hardware Description | Run Winstone99 Business v1.2 | ## Mulity Task | VCC ##### LED | -5V LED | +12V LED | -12V LED | VCC3X LED |  |
| ## BP-214SG-P4 | ## A1.0 | SBC-770 (PentiumⅢ 550) + Creative PNP 32bit Sound (ISA) + D-Link DFE-540TX LAN (PCI) + GENUINEGM56-AMI2019 V.90 56.6K FAX/Modem (PCI) + AHA 2940U2W SCSI (PCI) | Pass | From the SCSI HDD copy a lot of data to IDE HDD + VCD Play +Data stream deliver to HDD by LAN | Pass | Pass | Pass | ## Pass | ## Pass |  |
| ## BP-219SG-P4 | ## A1.0 | SBC-770 (Celeron 300) + Creative PNP 32bit Sound (ISA) + D-Link DFE-540TX LAN (PCI) + TAKON MR34PV 33.6K FAX/Modem (ISA) + AHA 1520B SCSI (ISA) | Pass | From the SCSI HDD copy a lot of data to IDE HDD + VCD Play +Data stream deliver to HDD by LAN | Pass | Pass | ## Pass | ## Pass | N/A |  |
| BP-220SI | A1.0 | SBC=558 (Tillamook-266) + Creative PNP 16bit Sound (ISA) + TAKON MR34PV 33.6K FAX/Modem (ISA) + AHA 1520B SCSI (ISA) + Intel EtherExpressTM PRO/100 LAN (ISA) | ## Pass | From the SCSI HDD copy a lot of data to IDE HDD +Data stream deliver to HDD by LAN +Sound Play | Pass | Pass | Pass | ## Pass | N/A |  |
| BP-220DI | A1.0 | System 1 SBC-656 (Celeron-366) + TAKON MR34PV 33.6K FAX/Modem (ISA) + Creative PNP 16bit Sound (ISA) + 3 Com 3C509B LAN (ISA) System 2 SBC-558 (Tillamook-266) + Intel EtherExpressTM PRO/100 LAN (ISA) + Sound Blaster 16bit Sound (ISA) + AHA 1520B SCSI (ISA) | Pass | System 1 Data stream deliver toHDD by LAN +Sound Play System 2 From the SCSI HDD copy a lot of data to IDE HDD +Sound Play | Pass | Pass | Pass | ## Pass | ## N/A |  |
| Note: 1. N/A : There is no power LED. 2. — : There is no 3.3V power input that 3.3V power LED can’t work. |  |  |  |  |  |  |  |  |  |  |

| Note: 1. N/A : There is no power LED. 2. — : There is no 3.3V power input that 3.3V power LED can’t work. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #### Backplane Test Report |  |  |  |  |  |  |  |  |  |  |
| # Model | Rev | ## Hardware Description | Run Winstone99 Business v1.2 | ## Mulity Task | VCC ##### LED | -5V LED | +12V LED | -12V LED | VCC3X LED |  |
| ## BP-220QI | ## A1.0 | System 1 SBC-558 (Tillamook-266) + Intel EtherExpressTM PRO/100 LAN (ISA) + Sound Blaster 16bit Sound (ISA) System 2 SBC-656 (Celeron-366) + Creative PNP 16bit Sound (ISA) + 3 Com 3C509B LAN (ISA) System 3 SBC-558 (Tillamook-266) +ES1868 Sound Card (ISA) + AHA 1520B SCSI (ISA) System 4 SBC-557 (AMD K6-2/300) + TAKON MR34PV 33.6K FAX/Modem (ISA) + AHA 1520B SCSI (ISA) | Pass | System 1 Data stream deliver toHDD by LAN +Sound Play System 2 Data stream deliver toHDD by LAN +Sound Play System 3 From the SCSI HDD copy a lot of data to IDE HDD +Sound Play System 4 From the SCSI HDD copy a lot of data to IDE HDD + VCD Play | Pass | Pass | Pass | ## Pass | ## N/A |  |
| Note: 1. N/A : There is no power LED. 2. — : There is no 3.3V power input that 3.3V power LED can’t work. |  |  |  |  |  |  |  |  |  |  |