EI7BM

## QE Compatibility Test Report

Release Date :Sep. 13, 2000

__________        __________     __________     __________

# Issue Stamp                     QA Manager               QE Manager               Test Engineer

Summary of contents :

Cover Sheet																		1

Summary of contents																2

1. Margin																					3

Includes temperature, DC voltage and IDE cable length tests.

2. Stability 																				4

Includes burn-in and booting tests .

A. Burn-In Testing(25℃/24hrs)															4

B. Burn-In Testing(Climatic Sequence/48hrs)												4

C. Booting Testing(25℃)																4

D. Booting Testing(Damp Heat,Cyclic/24hrs)												4

3. Performance																			5

Includes performance indexes using various performance test software programs .

4. Power consumption data																7

Includes +5V current rate .

5. Hardware compatibility																8

Includes CPU , DRAM , adapter card and HDD compatibility tests.

A. Component Compatibility- CPU														8

B. Component Compatibility- DRAM SIMM Module										8

C. Adapter Compatibility- VGA															8

D. Adapter Compatibility- LAN															8

E. Adapter Compatibility- SCSI Card														9

F. Adapter Compatibility- SCSI Card														9

G. Adapter Compatibility- Others															9

H. Peripheral Compatibility- HDD Device													9

I . Peripheral Compatibility- Storage Device												9

J. Peripheral Compatibility- Others  														9

K. Peripheral Compatibility- Input Device													10

L.On_Board Network Hardware (physical) Compatibility										10

6. Software compatibility																12

Includes various OS and application software tests.

A. O.S. Compatibility- Novell Network O.S./V5.0												12

B. O.S. Compatibility- Linux RedHat 6.0 (Kernal 2.2.5)											13

C. O.S. Compatibility- Windows 95														14

D. O.S. Compatibility- Windows 98														15

D-1. Microsoft Windows 98 Hardware Compatibility Test(HCT)									16

E. O.S. Compatibility- Windows NT R4.0													17

F. O.S. Compatibility- Windows 2000 Professional											19

G. O.S. Compatibility- DOS and DOS Utility Compatibility Testing								21

H. Word Processing/Spread Sheet Compatibility Testing										22

I. Business Graphics/DTP/DB Compatibility Testing											24

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

Quality Engineering Department of AAEON TECHNOLOGY  INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Hsin-Tien City, Taipei, Taiwan , ROC

Tel: 886-2-29121234

Fax:886-2-29130792

886-2-29130774

1. Margin Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■C&T 69000 |
| DOC Type | Seagete ST34311A 4.3GB |

Test Result:

| High Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: NS Geode GXLV-233 |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FE V5.29 |  |  |  |  |  | □ AMIDIAG V4.50 |  |  |  |  |
| ■50℃ | □60℃ |  | □65℃ | □70℃ |  | □ 75℃ |  |  | □ 80℃ | □ 85℃ |  |  |

| Low Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: NS Geode GXLV-233 |  |  |  |  | □ COM1 LB |  |  | □ COM2 LB |  |  |  | □ LPT LB |  |  |
| Test Program: |  | ■ QAPlus/FEV5.29 |  |  |  | □ AMIDIAG V4.50 |  |  | □5℃ |  |  |  | □0℃ |  |
| ■ 0℃ | □-10℃ |  | □-15℃ | □-20℃ |  |  | □-25℃ |  |  | □-30℃ | □-35℃ |  |  | □-40℃ |

| High Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: NS Geode GXLV-233 |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| □A Grade: 5.20V | ■B grade: 5.25V | □C Grade: 5.30V | □D Grade: 5.35V | □E Grade: 5.4V |

| Low Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: NS Geode GXLV-233 |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| □A Grade: 4.80V | ■B grade: 4.75V | □C Grade: 4.70V | □D Grade: 4.65V | □E Grade: 4.6V |

2﹒Stability Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver:4.51PG |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■C&T 69000 |
| DOC Type | Seagete ST34311A 4.3GB |

A. Burn-In Testing (24Hours)

Test Condition : Under Room Temp. 25℃

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| QAPlus/FE V5.29 |  |  |  |

B. Burn-In Testing (48 Hours)

Test Condition : Burn-In Under Temp.  0℃〜50℃(IEC-68-2-61)

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| ### Mulity Task |  |  |  |

Note : 1.Mulity Task is that transfers files(copy/verify/delet)

##### C. Booting Testing

Test Condition : Under Room Temp. 25℃

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0 % | 20 Sec | #### 5 Sec |  |
| KB Reset 20 Times | 20 | 20 | 0 % | 20 Sec | 5 Sec |  |

3﹒Performance Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D4564841G5-A10-9JF □8 □16 □32□64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 ■Windows NT |
| VGA Card | ■C&T 69000 |
| VGA Memory | □256K □512K □1M ■2M □4M |
| HDD Type | □Max■IBM □Con □Qua □Fuj : DTLA – 307045 40GB |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Type (Socket 370) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Intel | Intel | Intel | Intel | Intel | Intel |  |  | Intel | Unit |
|  | Celeron 366 | Celeron 400 | Celeron 433 | Celeron 466 | Celeron 500 | Celeron 533 |  |  | Celeron 600 |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |
| CPU | 2319.76 | 2530.64 | 2741.53 | 2952.45 | 3163.33 |  | 3374.22 |  | 3796.00 | MHz |
| ### FPU | 5865.79 | 6399.07 | 6932.36 | 7465.59 | 7998.81 |  | 8532.04 |  | 9525.72 | MHz |
| VIDEO | 11170 | 11170 | 11170 | 11170 | 11170 |  | 11170 |  | 11170 | Chs/sec |
| ### Power Meter 2.02 |  |  |  |  |  |  |  |  |  |  |
| MIPS Test | 177.7 | 189.6 | 218.8 | 237.0 | 237.0 | 258.5 |  |  | 316.0 | Mips |
| Content Creation WinStone 2000 V1.0 |  |  |  |  |  |  |  |  |  |  |
| Content Creation WinStone 2000 | 13.4 | 14 | 14.5 | 15 | 15.5 | 15.9 |  |  | 16.9 |  |
| WinStone 99 V1.2 |  |  |  |  |  |  |  |  |  |  |
| HighEnd Graphic WinStone99 | 18.9 | 19.5 | 20.1 | 20.8 | 21.5 | 22 |  |  | 23.2 |  |
| Business Graphic Winstone99 | 18.8 | 19.1 | 19.5 | 20.2 | 20.5 | 20.7 |  |  | 21.4 |  |
| WinBench 99 V1.1 |  |  |  |  |  |  |  |  |  |  |
| H.E. Graphic WinMark 99 | 269 | 288 | 303 | 314 | 327 | 343 |  |  | 379 | MP/s |
| Business Graphic WinMark99 | 90.3 | 94.8 | 97.6 | 101 | 105 | 107 |  |  | 115 | TB/s |
| CPUmark 99 | 23.1 | 24.4 | 25.4 | 26.5 | 27.1 | 27.8 |  |  | 30.3 |  |
| ### Coretest V3.02 |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 7472 | 7488 | 7488 | 8000 | 8000 | 8016 |  |  | 8192 | KB/Sec |
| Performance ID | 56.43 | 56.52 | 56.52 | 59.07 | 59.07 | 59.16 |  |  | 60.21 |  |
| ### Wintech V1.2 |  |  |  |  |  |  |  |  |  |  |
| Overall WinTach RPM | 378.67 | 392.84 | 403.23 | 413.54 | 434.65 |  |  | 441.33 | 455.88 |  |

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D4564841G5-A10-9JF □8 □16 □32□64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 ■Windows NT |
| VGA Card | ■C&T 69000 |
| VGA Memory | □256K □512K □1M ■2M □4M |
| HDD Type | □Max■Sea □Con □Qua □Fuj : ST34311A 4.3GB |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Type (Socket 370) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Intel | Intel | #### Intel | Intel | Intel |  | Unit |
|  | PentiumⅢ 550 | PentiumⅢ 600 | PentiumⅢ 700 | PentiumⅢ 750 | PentiumⅢ 850 |  |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |  |  |
| CPU | 3479.67 | 3796.04 | 4428.71 | 4745.05 | 5377.69 |  | MHz |
| ### FPU | 8732.10 | 9525.95 | 11113.69 | 11907.47 | 13495.17 |  | MHz |
| VIDEO | 16384 | 16384 | 16384 | 16384 | 16384 |  | Chs/sec |
| ### Power Meter 2.02 |  |  |  |  |  |  |  |
| MIPS Test | 284.4 | 284.4 | 355.5 | 355.3 | 406.3 |  | Mips |
| Content Creation WinStone 2000 V1.0 |  |  |  |  |  |  |  |
| Content Creation WinStone 2000 | 20.4 | 21.3 | 22.8 | 23.4 | 25 |  |  |
| WinStone 99 V1.2 |  |  |  |  |  |  |  |
| HighEnd Graphic WinStone99 | 29.3 | 30.5 | 32.5 | 33.2 | 35.7 |  |  |
| Business Graphic Winstone99 | 27.4 | 28.2 | 29.4 | 29.9 | 30.9 |  |  |
| WinBench 99 V1.1 |  |  |  |  |  |  |  |
| H.E. Graphic WinMark 99 | 411 | 432 | 478 | 502 | 535 |  | MP/s |
| Business Graphic WinMark99 | 133 | 138 | 148 | 152 | 158 |  | TB/s |
| CPUmark 99 | 45.1 | 48.2 | 53.5 | 56.2 | 61 |  |  |
| ### Coretest V3.02 |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 8848 | 9488 | 10048 | 10144 | 10144 |  | KB/Sec |
| Performance ID | 64.62 | 68.43 | 71.76 | 72.33 | 72.33 |  |  |

4﹒Power Consumption

System Information:

| DRAM Type & Capacity | ### NEC D4564163G5-A10-9JF 32MB |
| --- | --- |
| VGA Card | C&T 69000 |
| DOC or Floppy | AAEON PCM-3528 rev1.0 |
| O.S. | DOS6.22 |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

A.AT Power

| Mode Type | CPU Type (Socket 370 ) |  |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Celeron 366 | Celeron 400 | Celeron 433 | Celeron 466 | Celeron 500 | Celeron 533 | Celeron 600 |  |
| Normal Mode(+5V) | 4.7 | 4.9 | 5.1 | 5.1 | 5.5 | 5.8 | 4.2 | amp/max |
| Doze Mode(+5V) | 3.4 | 3.5 | 3.6 | 3.6 | 3.8 | 4.0 | 3.2 | amp/max |
| Standby Mode(+5V) | 3.4 | 3.5 | 3.6 | 3.6 | 3.8 | 3.9 | 3.2 | amp/max |
| Suspend Mode(+5V) | 2.1 | 2.2 | 2.2 | 2.3 | 2.3 | 2.3 | 2.3 | amp/max |

| Mode Type | CPU Type (Socket 370 ) |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PentiumⅢ 550 | PentiumⅢ 600 | PentiumⅢ 700 | PentiumⅢ 750 | PentiumⅢ 850 |  |  |
| Normal Mode(+5V) | 4.4 | 4.7 | 5.1 | 5.4 | 5.8 |  | amp/max |
| Doze Mode(+5V) | 3.3 | 3.5 | 3.7 | 3.8 | 4 |  | amp/max |
| Standby Mode(+5V) | 3.3 | 3.5 | 3.7 | 3.8 | 4 |  | amp/max |
| Suspend Mode(+5V) | 2.3 | 2.4 | 2.3 | 2.4 | 2.4 |  | amp/max |

Note :

Normal Model is meassured while open system

Doze Model VGA controller is still enabled

STANDBY Model VGA controller is still enabled

SUSPEND Model all function off

5﹒Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Intel | ### Celeron 366 MHz ( 66x5.5 ) |  |  |  |
| #### Intel | ### Celeron 400 MHz ( 66x6.0 ) |  |  |  |
| #### Intel | ### Celeron 433 MHz ( 66x6.5 ) |  |  |  |
| #### Intel | ### Celeron 466 MHz ( 66x7.0 ) |  |  |  |
| #### Intel | ### Celeron 500 MHz ( 66x7.5 ) |  |  |  |
| #### Intel | ### Celeron 533 MHz ( 66x8.0 ) |  |  |  |
| #### Intel | ### Celeron 600 MHz ( 66x9.0 ) |  |  |  |
| #### Intel | PentiumⅢ550 MHz ( 100x5.5 ) |  |  |  |
| #### Intel | ### PentiumⅢ600 MHz ( 100x6.0 ) |  |  |  |
| #### Intel | ### PentiumⅢ700 MHz ( 100x7.0 ) |  |  |  |
| #### Intel | ### PentiumⅢ750 MHz ( 100x7.5 ) |  |  |  |
| #### Intel | ### PentiumⅢ850 MHz ( 100x8.5 ) |  |  |  |

##### B. Component Compatibility- DRAM DIMM Module

DRAM DIMM Model List:

| Brand | Device Model | #### Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### NEC | D4564163G5-A10-9JF 32MB |  |  |  |
| TOSHIBA | TC59S6408CFT-80 64MB |  |  |  |
| NEC | D4564841G5-A10-9JF 128MB |  |  |  |
| #### NEC | D45128841G5-A10-9JF 256MB |  |  |  |
| #### NEC | D4564841G5-A80-9JF 64MB |  |  |  |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Onboard VGA | C&T 69000 |  |  |  |

D. Adapter Compatibility- LAN Card

Action:

| 1. Load ODI Driver (Lsl,Ipxodi,Vlm Or Netx) | 2. Login To Server System |
| --- | --- |
| 3. Transfer Data Between SRV And WK(Mulity Task) | 4. logout |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Onboard REL | RTL8139B |  |  |  |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Intel | 82557 |  |  |  |
| Intel | 82559 |  |  |  |
| Intel | EtherExpressTM PRO/100 (ISA) |  |  |  |
| D-Link | DFE-500TX |  |  |  |
| D-Link | DFE-540TX |  |  |  |
| 3 COM | 3C905B-TX |  |  |  |

E. Adapter Compatibility- Sound Card

Sound Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Onboard Sound | ESS Solo-1 |  |  |  |

F. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Adaptec | AHA-2940U/UW |  |  |  |
| Adaptec | AHA-1520 (ISA) |  |  |  |

G. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| MPEG Card |  |  |  |  |
| YUAN | MPG-200C DVD MPEG-2 |  |  |  |
| Modem Card (PCI) |  |  |  |  |
| SmartLink | 5634PEW 56000bps Win modem |  |  |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Seagate | ST34311A 4.3GB |  |  |  |
| #### IBM | DPTA-372050 20GB |  |  |  |
| #### IBM | DTLA-307045 40GB |  |  |  |
| Quantum | Fireball LB5100AT 5.1GB |  |  |  |
| #### Maxtor | 94098U8 40GB |  |  |  |

##### I .Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| CD/DVD ROM |  |  |  |  |
| TEAC | CD-224E 24X |  |  |  |
| #### Pioneer | DVD-A03S |  |  |  |
| ZIP |  |  |  |  |
| Iomega | Z100P2 |  |  |  |
| LS-120 |  |  |  |  |
| Panasonic | LKM-F934-1 |  |  |  |
| Fax/Modem |  |  |  |  |
| #### Cyber Lander | Rock Modem E210 V.90 56K |  |  |  |
| LEMEL | MD336RSVD 56K |  |  |  |

J .Peripheral Compatibility- Others

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| RS-485 Port |  |  |  |  |
| Adventech | Adam 401 |  |  |  |
| USB Port |  |  |  |  |
| WINTEK | AM-737-U USB Mouse |  |  |  |
| WINTEK | AM-767-U USB Mouse |  |  |  |
| DEXIN | A2U800A USB Mouse |  |  |  |
| Solidtek | ACK-298 USB Keyboard |  |  |  |
| BTC | 7932 USB Keyboard |  |  |  |
| Vcam | CU-98 USB Camara |  |  |  |
| Tekom | TekCam P35U USB Camara |  |  |  |
| HP | ScanJet 4200C USB Scanner |  |  |  |
| HP | Photo Smart P1000 USB Printer |  |  |  |
| D-Link | DU-E10 Ethernet Adapter |  |  |  |
| D-Link | DU-H4 USB HUB |  |  |  |
| D-Link | DU-560M USB modem |  |  |  |
| Iomega | Z100USB ZIP |  |  |  |
| Parallel Port |  |  |  |  |
| HP | Photo Smart P1000 USB Printer |  |  |  |
| iMars | MA01 MP3 Player |  |  |  |
| IR Port |  |  |  |  |
| ACTiSYS | ACT-IR210L |  |  |  |
| HOYA | Irwave 520F |  |  |  |

##### K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |
| Acer | 6511-TW1C/B |  |  |  |
| Mouse |  |  |  |  |
| DEXIN | K3P800A |  |  |  |
| LOGITECH | M-M35 |  |  |  |

L. On_Board Network Hardware (physical) Compatibility

System Information(Client Site):

| Board System | ■EI7BM |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D4564841G5-A10-9JF □8 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty:■1□2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: 4.51PG |
| O.S. Environment | ■DOS6.22 ■WINNT____ □NOVELL_____ |
| VGA Card | ■C&T 69000 |
| POWER SUPPLY | □MPU-250W□SSH-250G □SSH-350G ■MPE-8071 |
| IDE HDD TYPE | □Max■Sea □Con □Qua □Fuj □IBM : ST34311A 4.3GB |
| LAN CARD | ■OnBoard: RTL8139B |
| CD-ROM TYPE | □NEC ■TEAC □PIONEER □CREATIVE : CD-224E |
| Cable/Cable Length | CAT5 100M |

System Information(Server Site):

| Board System | ■MB-668 |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: LGS GM72V661641CT7J □8 □16 □32 ■64MB□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: 4.51PG |
| VGA Card | ■OnBoard C&T 69000 |
| O.S. Environment | ■DOS6.22 ■WINNT4.0 ■NOVELL 4.11 ■NOVELL 5.0 |
| IDE HDD TYPE | □Max □Sea □Con ■Qua □Fuj □IBM : Bigfoot 4.0AT |
| LAN CARD | ■OnBoard: RTL-8139B |
| ### CPU Type | ■Intel □Cyrix □AMD : Celeron 466 MHz |
| CD-ROM TYPE | □NEC ■TEAC □PIONEER □CREATIVE : CD-540E |
| Cable/Cable Length | CAT5 100M |

HUB Type: D-Link DES1008 10/100 Fast Ethernet Switch

Test Result:

| SUBJECT | Network Hardware Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Transceiver |  |  |  |  |
| Receiver |  |  |  |  |
| CRC error |  |  |  |  |
| Collision back-off generation |  |  |  |  |
| Auto-negotiation |  |  |  |  |
| ### 100M Base-TX full duplex |  |  |  |  |
| 100M Base-TX half duplex |  |  |  |  |
| 10M Base-T full duplex |  |  |  |  |
| 10M Base-T half duplex |  |  |  |  |
| SUBJECT | Network Operation System Function Testing |  |  |  |
| Test Item |  | Pass | Fail | Note |
| NetWare 4.11 |  |  |  |  |
| Windows NT 4.0 |  |  |  |  |
| Linux Unix |  |  |  |  |
| SUBJECT | Network Performance Testing |  |  |  |
| Test Item |  | Result |  | Note |
| ### NWTEST 24HR |  | #### Pass |  |  |

| SUBJECT | Workstation Application Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Dos 6.22 for IPXODI |  |  |  |  |
| Dos 6.22 for package driver |  |  |  |  |
| Windows 3.1 for IPXODI |  |  |  |  |
| Windows 3.1 for package driver |  |  |  |  |
| WFW 3.11 for NDIS 2.0/3.0 |  |  |  |  |
| WFW 3.11 for NDIS 3.0/ODI |  |  |  |  |
| Windows 95 for IPX/SPX |  |  |  |  |
| Windows 95 for NetBEUI |  |  |  |  |
| Windows 95 for TCP/IP |  |  |  |  |
| Windows 98 for IPX/SPX |  |  |  |  |
| Windows 98 for NetBEUI |  |  |  |  |
| Windows 98 for TCP/IP |  |  |  |  |
| Windows NT 4.0 for IPX/SPX |  |  |  |  |
| Windows NT 4.0 for NetBEUI |  |  |  |  |
| Windows NT 4.0 for TCP/IP |  |  |  |  |
| Transfer Data Between SRV And WK |  |  |  | Mulity Task 12 Hours(NOTE 1.) |

Note : 1.Mulity Task is that transfers files(copy/verify/delet) and VCD in the same time.

6﹒Software Compatibility

A. O.S. Compatibility-Novell Network O.S./V 5.0

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:NEC D4564841G5-A80-9JF □8 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard C&T69000 |
| VGA Memory Capacity & Brand | Brand: C&T69000 □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel□Cyrix □AMD:Celeron 600MHz |
| HDD Type | □Max ■Sea □Con □Qua □Fuj : ST34311A 4.3GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Server Function Testing |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU:Celeron 600 MHz |
| Boot File Server From HDD |  |  |  |
| Down & Exit |  |  |  |
| Data Transfer |  |  |  |
| Workstation Function Testing |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU:Celeron 600 MHz |
| Login |  |  |  |
| Logout |  |  |  |
| Data Transfer |  |  |  |

B. O.S. Compatibility -Linux RedHat 6.0 (kernel 2.2.5)

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D45128841G5-A10-9JF □16 □32 □64■128□256MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51GP |
| VGA Card & BIOS | ■OnBoard: C&T 69000 |
| VGA Memory Capacity & Brand | ### Brand: C&T69000 □DRAM ■VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850 MHz |
| HDD Type | □Max■Sea □Con □Qua □WD : ST34311A 4.3GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative :CD48341 |
| Driver or Utility | ■RTL 8139B LAN ■C&T69000 |

Test Result:

| Test Item |  | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Subject | Installation to HDD |  |  |  |
| Install from CD-ROM |  |  |  | Testing CPU: PentiumⅢ 850 MHz |
| Subject | # Base Function |  |  |  |
| Default Booting |  |  |  |  |
| Failsafe Booting |  |  |  |  |
| Gnome work plane |  |  |  |  |
| KDE work plane |  |  |  |  |
| Reboot |  |  |  |  |
| Logout |  |  |  |  |
| Halt |  |  |  |  |
| Subject | ## Xf86config Setup(3.3.3.1) |  |  |  |
| Xf86config |  |  |  | BIOS adjust to 1024x768 VGA Mode |
| XF86Setup |  |  |  |  |
| X –probeonly > |  |  |  |  |
| Subject | ## X Windows Function |  |  |  |
| Netscape 4.51 |  |  |  |  |
| Work space switching |  |  |  |  |
| Ctrl+alt+back space |  |  |  |  |
| Ctrl+alt+F1 |  |  |  |  |
| Ctrl+alt+F7 |  |  |  |  |
| Ctrl+alt +/- |  |  |  |  |
| Subject | ## Xterm Function |  |  |  |
| Dmesg |  |  |  |  |
| adduser 、passwd |  |  |  |  |
| Ping |  |  |  |  |
| ls、cp、rm、mv、tar、gzip |  |  |  |  |
| mkdir、rmdir |  |  |  |  |
| Subject | ## Mount Function |  |  |  |
| Floppy、CD-ROM、HD |  |  |  |  |
| MSDOS FAT |  |  |  |  |
| Windows 98 FAT |  |  |  |  |

C. O.S. Compatibility-Windows 95

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:TOSHIBA TC59S6408CFT-80 □8 □16 □32 ■64□128MB □DRAM■SDRAM□SODIMM □________ Qty:■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM ■VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □NS □AMD: PentiumⅢ 850 MHz |
| HDD Type | □Max □WD □Con ■Qua □Fuj : Fireball LB5100AT |
| CD-ROM Type | □NEC ■TEAC □Pioneer □Creative : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 | — | — |  |  |  |
|  | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: PentiumⅢ 850 MHz |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

##### D. O.S. Compatibility-Windows 98

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:NEC D4564841G5-A10-9JF □8 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850 MHz |
| HDD Type | □Max■Sea □Con □Qua □Fuj : ST34311A 4.3GB |
| CD-ROM Type | □NEC ■TEAC □Pioneer □Creative : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 | — | — |  |  |  |
|  | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1280X1024 | 256 |  |  |  |  |  |
| 1600X1200 | 256 |  |  |  |  | Display device:Both |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: PentiumⅢ 850 MHz |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

D-1.Microsoft Windows 98 Hardware Compatibility Test(HCT)

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564841G5-A10-9JF □32 □64■128□256MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850 MHz |
| HDD Type | □Max □WD □Con □Qua■IBM : DPTA-372050 20GB |
| CD-ROM Type | □NEC ■TEAC □Pioneer □Creative : CD-224E |
| VGA Card & BIOS | C&T 69000 |
| Keyboard TYPE | Acer 6511-TW1C/B |
| FDD TYPE | TEAC FD-05HG |
| Mouse | LOGITECH M-CQ38 |
| LAN CARD | RTL8139B |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Test Result:

| HCT 7.6 Features Test | Pass | Fail | ### Remark |
| --- | --- | --- | --- |
| Virtual Dos Mach |  |  |  |
| DMPS Test |  |  |  |
| Video Playback |  |  |  |
| Extended BIOS Parameter Block |  |  |  |
| MCI CD Test |  |  |  |
| BCT API Test |  |  |  |
| INT 13 Extension |  |  |  |
| Disk(Stress) Individual Drive |  |  |  |
| Disk –CD (Stress) |  |  |  |
| Disk (Stress) |  |  |  |
| Coprocessor(Stress) |  |  |  |
| Net Class Dynamic |  |  |  |
| PCI I/O Space |  |  |  |
| PCI 2.1 (BIOS) |  |  |  |
| PCI 2.1 (Card) |  |  |  |
| Unreported I/O |  |  |  |
| Unreported Memory |  |  |  |
| PnP BIOS |  |  |  |
| APM 1.2 |  |  |  |
| Media Class Dynamic Config |  |  |  |
| Wave Drift | ─ |  |  |
| Note : PCI 2.1 (Card) / Media Class Dynamic Config fail |  |  |  |

E. O.S. Compatibility-Windows NT R4.0

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□ 2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ### Onboard C&T 69000 |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □NS □AMD : PentiumⅢ 850 MHz |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : Fireball LB5100AT |
| CD-ROM Type | □NEC ■TEAC □PIONEER □CREATIVE : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 | — | — |  |  |  |
|  | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
|  | 64K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1280X1024 | 256 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: PentiumⅢ 850 MHz |
| Boot |  |  |  |
| Boot to Windows NT |  |  |  |
| Boot to MS-DOS |  |  |  |
| Last known good configuration |  |  |  |
| Original Configuration |  |  |  |
| Shutdown |  |  |  |
| Shutdown & Restart |  |  |  |
| CPU/Memory/File System |  |  |  |
| Memory size less than 64MB |  |  |  |
| Memory size Larger than 64MB |  |  |  |
| Virtual memory |  |  |  |
| System tasking settings |  |  |  |
| FAT |  |  |  |
| NTFS |  |  |  |
| Nwtwork |  |  |  |
| System clock |  |  |  |
| Task switching |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| CapLock,NumLock,ScrLock |  |  |  |
| Ctrl+Alt+Del |  |  |  |
| Ctrl+Esc,Alt+Tab/Esc/Space |  |  |  |
| Alt+Tab,Alt+Esc,Alt+Space |  |  |  |
| Ctrl+Alt+Pause/Print Screen |  |  |  |
| Ctrl+Alt+F1~F12 |  |  |  |
| COM Port |  |  |  |
| Serial Mouse |  |  |  |
| Security/Audit/Control |  |  |  |
| User manager |  |  |  |
| Server manager |  |  |  |
| Disk administrator |  |  |  |
| Performance monitor |  |  |  |
| User profile editor |  |  |  |
| Event Viewer |  |  |  |
| Windows NT Diagnostics |  |  |  |

F. O.S. Compatibility-Windows 2000 Professional

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D45128841G5-A10-9JF □8 □16 □32 □64■256MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: Rev1.0 |
| VGA Card & BIOS | ■OnBoard C&T 69000 |
| VGA Memory Capacity & Brand | Brand: C&T69000 □60ns □70ns □ ns □DRAM ■VRAM □256K □512K □1M ■2M □4M |
| CPU Type | Intel PentiumⅢ 750MHz |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : Fireball LB5100AT 5.1GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer □Creative ■TEAC : CD-532E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 | — | — |  |  |  |
|  | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
|  | 64K | — | — |  |  |  |
| 800X600 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1280X1024 | 256 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install English to IDE HDD |  |  | Testing CPU:PentiumⅢ 750MHz |
| Install Traditional Chinese to IDE HDD |  |  | Testing CPU:Celeron 500 MHz |
| Windows 2000 Advanced Options Menu |  |  |  |
| Safe Mode |  |  |  |
| Safe Mode with Networking |  |  |  |
| Safe Mode with Command Prompt |  |  |  |
| Enable Boot Logging |  |  |  |
| Enable VGA Mode |  |  |  |
| Last known Good Configuration |  |  |  |
| Directory Services Restore Mode (windows 2000 domain controllers only) |  |  |  |
| Debugging Mode |  |  |  |
| Boot Normally |  |  |  |
| Return to OS Choices Menu |  |  |  |

| Function Test |  |  |  |
| --- | --- | --- | --- |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Alt+Esc Switch Windows |  |  |  |
| Ctrl+Alt+Del Windows Security Window |  |  |  |
| F8 Start up Windows 2000 Advanced Options Menu Menu |  |  |  |
| Shutdown |  |  |  |
| Log off User |  |  |  |
| Shut down |  |  |  |
| Restart |  |  |  |

G. O.S. Compatibility-DOS and DOS Utility Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564841G5-A10-9JF □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard C&T69000 |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM ■VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850 MHz |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : Fireball LB5100AT 5.1GB |
| CD-ROM Type | □NEC ■TEAC □PIONEER □CREATIVE : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: PentiumⅢ 850 MHz |
| Function Test |  |  |  |
| Sysinfo Check (Before DOS Initialed ) |  |  |  |
| System Clock(Compare to RTC CLK) |  |  |  |
| DOS Prompt |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| Serial Mouse |  |  |  |
| DOS Commander |  |  |  |
| MSD.COM |  |  |  |
| EDIT.COM |  |  |  |
| EMM386.EXE |  |  |  |
| HIMEN.SYS |  |  |  |
| Norton Commander |  |  |  |
| Key F1~F10 Function |  |  |  |
| Key Alt+F1~F10 Function |  |  |  |
| Key Ctrl+F1~F10 Function |  |  |  |
| Telix |  |  |  |
| Setup Function |  |  |  |
| Connect with E288MX Fax |  |  |  |
| Dialing Function |  |  |  |
| File Upload Function |  |  |  |
| File Download Function |  |  |  |
| ET Ver3.53 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |
| ET 2000 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |
| ETFree Function |  |  |  |

H. Word Processing/Spread Sheet Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564841G5-A10-9JF □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850MHz |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : Fireball LB5100AT 5.1GB |
| CD-ROM Type | □NEC ■TEAC □PIONEER □CREATIVE : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Ami Pro 3.1 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Style Function |  |  |  |
| Page Function |  |  |  |
| Frame Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Word 6.0c |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Table Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| WordPerfect 6.1 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Table Function |  |  |  |
| Graphics Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Works 3.0b |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Lotus 1-2-3 R5 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Style Function |  |  |  |
| Tools Function |  |  |  |
| Range Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Excel 5.0c |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Data Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Quattro Pro 6.01 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Block Function |  |  |  |
| Notebook Function |  |  |  |
| Graphics Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |

I. Business Graphics/DTP/DB Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564841G5-A10-9JF □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: C&T 69000 □60ns □70ns □ ns □DRAM ■VRAM □256K □512K □1M ■2M □4M |
| CPU Type | ■Intel □Cyrix □AMD: PentiumⅢ 850MHz |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : Fireball LB5100AT 5.1GB |
| CD-ROM Type | □NEC ■TEAC □PIONEER □CREATIVE : CD-224E |
| Driver or Utility | ■VGA □HDD ■Others : RTL 8139B LAN , ESS Solo-1 Sound |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| PageMaker 5.0a |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| Utilities Function |  |  |  |
| Layout Function |  |  |  |
| Type Function |  |  |  |
| Element Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| CorelDraw 5.0E2 |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Layout Function |  |  |  |
| Arrange Function |  |  |  |
| Effects Function |  |  |  |
| Text Function |  |  |  |
| Special Function |  |  |  |
| Help Function |  |  |  |
| PowerPoint 4.0c |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Draw Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| DBase 5.0 |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Report Function |  |  |  |
| Program Function |  |  |  |
| Table Function |  |  |  |
| Properties Function |  |  |  |
| Help Function |  |  |  |
| Paradox 5.0 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Table Function |  |  |  |
| Query Function |  |  |  |
| Properties Function |  |  |  |
| Tools Function |  |  |  |
| Record Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Access 2.0 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Security Function |  |  |  |
| Format Function |  |  |  |
| Record Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |

- Problem List :

- Windows 98 + HCT 7.6 , 經測試在1)PCI 2.1 card  ; 2) Media Class Dynamic Config fail ,

- Sound 的部份, MIC測試時音量很小 , 該線路可能未經過放大