## GENE-4310T

## QE Compatibility Test Report

Release Date : Spe. 16, 2000

__________        __________     __________     __________

# Issue Stamp                     QA Manager               QE Manager               Test Engineer

Summary of contents :

Cover Sheet																		1

Summary of contents																2

1. Margin																					3

Includes temperature, DC voltage and IDE cable length tests.

2. Stability 																				4

Includes burn-in and booting tests .

A. Burn-In Testing(25℃/24hrs)															4

B. Burn-In Testing(Climatic Sequence/48hrs)												4

C. Booting Testing(25℃)																4

D. Booting Testing(Damp Heat,Cyclic/24hrs)												4

3. Performance																			5

Includes performance indexes using various performance test software programs .

4. Power consumption data																6

Includes +5V current rate .

5. Hardware compatibility																7

Includes CPU , DRAM , adapter card and HDD compatibility tests.

A. Component Compatibility- CPU														7

B. Component Compatibility- DRAM SIMM Module										7

C. Adapter Compatibility- VGA															7

D. Adapter Compatibility- LAN															7

E. Adapter Compatibility- SCSI Card														7

F. Adapter Compatibility- Other Card														7

G. Adapter Compatibility- Industry Control Card												8

H. Peripheral Compatibility- HDD Device													8

I. Peripheral Compatibility- Storage Device													8

J. Peripheral Compatibility- Others		              											8

K. Peripheral Compatibility- Input Device													9

L. On_Board Network Hardware (physical) Compatibility										9

6. Software compatibility																11

Includes various OS and application software tests.

A. O.S. Compatibility- Windows 95														11

B. O.S. Compatibility- Windows 98														12

B-1. Microsoft Windows 98 Hardware Compatibility Test(HCT)									13

C. O.S. Compatibility- Windows NT R4.0													14

D. O.S. Compatibility- DOS and DOS Utility Compatibility Testing								16

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

Quality Engineering Department of AAEON TECHNOLOGY  INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Hsin-Tien City, Taipei, Taiwan , ROC

Tel: 886-2-29121234

Fax:886-2-29130792

886-2-29130774

1. Margin Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D4516821AG5-A10-9NF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard Cyrix CX5530 |
| FDD Type | TEAC FD-235HF |

Test Result:

| High Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU:Cyrix MediaGX 200 |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FE V5.4 |  |  |  |  | □ AMIDIAG V4.50 |  |  |  |  |
| ■55℃ | ■60℃ |  | □65℃ | □ 70℃ |  | □ 75℃ |  | □ 80℃ | □ 85℃ |  |  |

| Low Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: Cyrix MediaGX 200 |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FEV5.4 |  |  | □ AMIDIAG V4.50 |  | ■5℃ |  |  | ■0℃ |  |
| □-5℃ | □-10℃ |  | □-15℃ | □-20℃ |  | □-25℃ |  | □-30℃ | □-35℃ |  | □-40℃ |

| High Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Cyrix MediaGX 200 |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.4 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 5.20V | ■BGrade: 5.25V | ■CGrade: 5.30V | □DGrade: 5.35V | □EGrade: 5.4V |

| Low Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Cyrix MediaGX 200 |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.4 |  | □ AMIDIAG V4.50 |  |
| ■AGrade: 4.80V | ■BGrade: 4.75V | ■CGrade: 4.70V | □DGrade: 4.65V | □EGrade: 4.6V |

Note:QAPluse 5.4 is running in harddisk and skip NPU testing item

SODIMM type : NEC  D4516821AG5-A10-9NF

HD type : WD AC26400 6.4G

CPU: NS Geode GXLV 233

2﹒Stability Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: D4516821AG5-A10-9NF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard Cyrix CX5530 |
| FDD Type | TEAC FD-235HF |

A. Burn-In Testing (24Hours)

Test Condition : Under Room Temp. 25℃

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| ###### QAPlus/FEV5.4 |  |  |  |

B. Burn-In Testing (48 Hours)

Test Condition : Burn-In Under Temp.  0℃〜60℃(IEC-68-2-61)

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| ###### QAPlus/FEV5.4 |  |  |  |

##### C. Booting Testing

Test Condition : Under Room Temp. 25℃

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0 % | 5 Sec | 3 Sec |  |
| KB Reset 20 Times | 20 | 20 | 0 % | 5 Sec | 3 Sec |  |

D. Burn-In Booting Testing (24 Hours)

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-30)

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off | 812 | 812 | 0 % | 60 Sec | 40 Sec |  |
| H/W Reset |  |  | % | Sec | Sec |  |

Note : QAPluse 5.4 is running in harddisk and skip NPU testing item

SODIMM type : NEC  D4516821AG5-A10-9NF

HD type : WD AC26400 6.4G

CPU: NS Geode GXLV 233

3﹒Performance Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D45128163G5-A80-9JF □8 □16 □32 □64■128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 □Windows NT |
| VGA Card | ■OnBoard Cyrix CX5530 |
| VGA Memory | □256K □512K □1M ■2.5M □4M □ |
| HDD Type | □Max ■Sea □Con □Qua □Fuj : ST34311A 4.3GB |
| Testing Feature | □32 bits Transfer ■Block Mode □DMA Mode |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | □VGA □HDD ■Others : MediaGX 4.0 |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

| Testing Utility | NS CPU Type |  |  |  | #### Unit |
| --- | --- | --- | --- | --- | --- |
|  | Geode GXLV 233B | Geode GX1 300B | Geode GX1 233B |  |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |
| CPU | 982.56 | 1167.32 | 982.56 |  | MHz |
| ### FPU | 2638.20 | 3135.62 | 2638.20 |  | MHz |
| VIDEO | 17873 | 20915 | 17873 |  | Chs/sec |
| ### Norton 8.0 |  |  |  |  |  |
| SI | 606.6 | 736.7 | 606.6 |  | ID |
| ### Power Meter 2.02 |  |  |  |  |  |
| MIPS Test | 91.7 | 109.4 | 91.8 |  | Mips |
| WinStone _____ |  |  |  |  |  |
| HighEnd Graphic WinStone_99_ |  |  |  |  |  |
| Business Graphic Winstone_99_ | 7.64 | 9.22 | 7.67 |  |  |
| WinBench __99__ |  |  |  |  |  |
| H.E. Graphic WinMark | 84.9 | 110 | 84.9 |  | MP/s |
| Business Graphic WinMark | 29.2 | 37.6 | 29.2 |  | TB/s |
| CPUmark 99 | 7.48 | 9.78 | 7.48 |  |  |
| FPUmark 99 | 379 | 493 | 379 |  |  |
| ### Coretest V3.02 |  |  |  |  |  |
| DTR (Buffered Read) | 7888 | 7961 | 7888 |  | KB/Sec |
| Performance ID | 52.13 | 53.49 | 52.13 |  |  |
| ### Wintech V1.2 |  |  |  |  |  |
| Overall WinTach RPM | 145.15 | 175.07 | 145.15 |  |  |

4﹒Power Consumption

System Information:

| DRAM Type & Capacity | NEC D45128163G5-A80-9JF 128MB |
| --- | --- |
| VGA Card | Cyrix CX5530 |
| DOC or Floppy | Floppy |
| VGA Testing Mode | 800X600 |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

| #### Mode Type | NS CPU Type |  |  |  |  | #### Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Geode GXLV 233B | Geode GX1 300B | Geode GX1 233B |  |  |  |
| Normal Mode(+5V) | 1.85 | 2.1 | 1.85 |  |  | amp/max |
| Doze Mode(+5V) | 1.1 | 1.1 | 0.9 |  |  | amp/max |
| Sleep Mode(+5V) | 1.0 | 1.0 | 0.9 |  |  | amp/max |

Note :

             1.STANDBY Model all function off.

2.The unit of current is A .

5﹒Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| ### NS | ### Geode GXLV 233 MHz |  |  |  |

##### B. Component Compatibility- DRAM SO-DIMM Module

DRAM SO-DIMM Model List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| NEC | D4516821AG5-A10-9NF 16MB PC-100 |  |  |  |
| NEC | D4564163G5-A10-9JF 32MB PC-100 |  |  |  |
| NEC | D4564163G5-A10-9JF 64MB PC-100 |  |  |  |
| NEC | D45128163G5-A80-9JF 128MB PC-100 |  |  |  |
| Siemens | HYB39S64160AT-8 64MB PC-100 |  |  |  |
| Toshiba | TC59SM716FT-80 128MB PC-100 |  |  |  |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| ### Cyrix | CX5530 |  |  |  |

D. Adapter Compatibility- LAN Card

Action:

| 1. Load ODI Driver (Lsl,Ipxodi,Vlm Or Netx) | 2. Login To Server System |
| --- | --- |
| 3. Transfer Data Between SRV And WK(Mulity Task) | 4. logout |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Intel | 82559ER |  |  |  |

E. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| AAEON | PCM-3420 |  |  |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Sound Card |  |  |  |  |
| On board | Cyrix 5530 |  |  |  |

##### G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| 104 Module |  |  |  |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) |  |  |  |
| AAEON | PCM-3600(FAX/MODEM) |  |  |  |
| AAEON | PCM-3420 (SCSI) |  |  |  |
| AAEON | PCM-3200(SOUND) |  |  | no -12V power can work |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Segate | ST51270A 1.2G |  |  |  |
| IBM | ### DHEA-34330 3.4G |  |  |  |
| Quantum | TX4000AT 4.3G |  |  |  |
| WD | AC26400 6.4G |  |  |  |
| Maxtor | 91021U2 10GB |  |  |  |
| Quantum | KX13600AT 13.6G |  |  |  |
| Segate | ST34311A 4.3G |  |  |  |

##### I.Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| CD/ROM |  |  |  |  |
| TEAC | ### CD-532E 32X |  |  |  |
| SAMPO | CDM-T631 36X |  |  |  |
| NEC | CDR-3001A |  |  |  |
| ZIP |  |  |  |  |
| ZIP | IOMEGA ZIP 100MB Parallel Port |  |  |  |
| ZIP | IOMEGA ZIP 100MB USB Port |  |  |  |
| LS-120 |  |  |  |  |
| LS-120 | LMK-F934-1 |  |  |  |

J.Peripheral Compatibility- Others

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| RS-485 Port |  |  |  |  |
| Adam | ADAM-4011 |  |  |  |
| USB Port |  |  |  |  |
| Dexin | Mouse |  |  |  |
| Vcam | Capture |  |  |  |
| Tekcam | P35u |  |  |  |
| BTC | 7932 Keyboard |  |  |  |
| D-Link | DU-H4 HUB |  |  |  |
| HP | ScanJet 4200C |  |  |  |
| HP | PhotoSmart P1000 Printer |  |  |  |

| Parallel Port |  |  |  |  |
| --- | --- | --- | --- | --- |
| HP | PhotoSmart P1000 Printer |  |  |  |
| ZIP | IOMEGA ZIP 100MB Parallel Port |  |  |  |
| IR Port |  |  |  |  |
| ACT | IR 210L |  |  |  |

##### K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |
| BTC | 7932 Keyboard |  |  |  |
| Acer | 6311-TW |  |  |  |
| ### Mouse |  |  |  |  |
| Dexin | Mouse |  |  |  |
| Logitech | MouseMan |  |  |  |
| Logitech | Serial Mouse |  |  |  |

L. On_Board Network Hardware (physical) Compatibility

System Information(Client Site):

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □8 □16 ■32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: V 4.51 PG |
| O.S. Environment | ■DOS6.22 □WINNT____ □NOVELL_____ □__________ |
| VGA Card | ■OnBoard Cyrix CS5530 □_____________________ |
| POWER SUPPLY | □MPU-250W■SSH-250G □SSH-350G □ST-250AF |
| IDE HDD TYPE | □Max □Sea □Con ■Qua □Fuj □IBM : TX4000AT 4.3G |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| LAN CARD | ■OnBoard: Intel 82559ER |
| CD-ROM TYPE | ■NEC □PANASONIC □PIONEER □CREATIVE : CDR-3001A |
| Cable/Cable Length |  |

System Information(Server Site):

| Board System | ■M/B:SBC-656 |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564163G5-A10-9JF □8 □16 ■32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: V 4.51 PG |
| VGA Card | ■OnBoard C&T69000 □_____________________ |
| O.S. Environment | □DOS6.22 □WINNT____ ■NOVELL 4.11 □__________ |
| IDE HDD TYPE | □Max □Sea □Con □Qua □Fuj ■WD: AC26400 |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| LAN CARD | ■OnBoard: RTL 8139B |
| ### CPU Type | ### ■Intel Celeron 366MHz |
| CD-ROM TYPE | □NEC □PANASONIC □PIONEER ■TEAC : CD-540E 40X |
| Cable/Cable Length | CAT 5 / 100M |

HUB Type:

Test Result:

| SUBJECT | Network Hardware Function Testing |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Item |  |  |  |  | Pass |  |  | Fail |  |  | Note |
| Transceiver |  |  |  |  |  |  |  |  |  |  |  |
| Receiver |  |  |  |  |  |  |  |  |  |  |  |
| CRC error |  |  |  |  |  |  |  |  |  |  |  |
| Collision back-off generation |  |  |  |  |  |  |  |  |  |  |  |
| Auto-negotiation |  |  |  |  |  |  |  |  |  |  |  |
| 100M Base-TX half duplex |  |  |  |  |  |  |  |  |  |  |  |
| 10M Base-T half duplex |  |  |  |  |  |  |  |  |  |  |  |
| SUBJECT | Network Operation System Function Testing |  |  |  |  |  |  |  |  |  |  |
| Test Item |  |  | Pass |  |  | Fail |  |  |  | Note |  |
| NetWare 4.11 |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 |  |  |  |  |  |  |  |  |  |  |  |
| SUBJECT | Network Performance Testing |  |  |  |  |  |  |  |  |  |  |
| Test Item |  |  | Result |  |  |  |  |  |  | Note |  |
| ### NWTEST 24HR |  |  | #### Pass |  |  |  |  |  |  |  |  |
| SUBJECT |  | Workstation Application Function Testing |  |  |  |  |  |  |  |  |  |
| Test Item |  |  |  | Pass |  |  | Fail |  | Note |  |  |
| Dos 6.22 for IPXODI |  |  |  |  |  |  |  |  |  |  |  |
| Dos 6.22 for package driver |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 for IPX/SPX |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 for NetBEUI |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 for TCP/IP |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 for IPX/SPX |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 for NetBEUI |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 for TCP/IP |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 for IPX/SPX |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 for NetBEUI |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 for TCP/IP |  |  |  |  |  |  |  |  |  |  |  |

Note : 1.Mulity Task is that transfers files(copy/verify/delet) and VCD in the same time.

6﹒Software Compatibility

A. O.S. Compatibility-Windows 95

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand:NEC D4564163G5-A10-9JF □8 □16 ■32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K □ |  |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |  |
| VGA Card & BIOS | ■OnBoard CyrixCS5530 |  |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns ■DRAM □VRAM □256K □512K □1M □2.5M ■4M |  |
| CPU Type | ■OnBoard:NS Geode GXLV 233 MHz |  |
| HDD Type | □Max □Sea □Con ■Qua □WD: KX13600AT 13.6G |  |
| CD-ROM Type | ■SAMPO □Panasonic □Pioneer □Creative : CDM-T631 36X |  |
| Driver or Utility | □VGA □HDD ■Others : MediaGX 4.0 |  |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1280X1024 | 16 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: NS Geode GXLV 233 |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

B. O.S. Compatibility-Windows 98

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand:HYB39S64160AT-8 □8 □16 □32 ■64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K |  |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |  |
| VGA Card & BIOS | ■OnBoard CS5530 |  |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns ■DRAM □VRAM □256K □512K □1M □2.5M ■4M |  |
| CPU Type | ■OnBoard:NS Geode GXLV 233 MHz |  |
| HDD Type | ■Max □Sea □Con □Qua □WD: 91021U2 10GB |  |
| CD-ROM Type | ■SAMPO □Panasonic □Pioneer □Creative : CDM-T631 36X |  |
| Driver or Utility | □VGA □HDD ■Others : MediaGX 4.0 |  |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1280X1024 | 16 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: NS Geode GXLV 233 |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

B-1.Microsoft Windows 98 Hardware Compatibility Test(HCT)

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand:NEC D4564163G5-A10-9JF □8 □16 □32 ■64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K |  |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |  |
| VGA Card & BIOS | ■OnBoard Cyrix CS5530 |  |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns ■DRAM □VRAM □256K □512K □1M □2.5M ■4M |  |
| CPU Type | ■OnBoard:NS Geode GXLV 233 MHz |  |
| HDD Type | ■IBM □Sea □Con □Qua □WD: DHEA-34330 3.4G |  |
| CD-ROM Type | ■TEAC □Panasonic □Pioneer □Creative : CD-532E 32X |  |
| Driver or Utility | □VGA □HDD ■Others : MediaGX 4.0 |  |

Test Result:

| HCT 7.6 Features Test | Pass | Fail | ### Remark |
| --- | --- | --- | --- |
| Virtual Dos Mach |  |  |  |
| DMPS Test |  |  |  |
| Video Playback |  |  |  |
| Extended BIOS Parameter Block |  |  |  |
| MCI CD Test |  |  |  |
| BCT API Test |  |  |  |
| INT 13 Extension |  |  |  |
| Disk(Stress) Individual Drive |  |  |  |
| Disk –CD (Stress) |  |  |  |
| Disk (Stress) |  |  |  |
| Coprocessor(Stress) |  |  |  |
| Net Class Dynamic |  |  |  |
| PCI I/O Space |  |  |  |
| PCI 2.1 (BIOS) |  |  |  |
| PCI 2.1 (Card) |  |  |  |
| Unreported I/O |  |  |  |
| Unreported Memory |  |  |  |
| PnP BIOS |  |  |  |
| APM 1.2 |  |  |  |
| Media Class Dynamic Config |  |  |  |
| Wave Drift |  |  |  |

C. O.S. Compatibility-Windows NT R4.0

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand:HYB39S64160AT-8 □8 □16 □32 ■64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K □ |  |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |  |
| VGA Card & BIOS | ■OnBoard CS5530 |  |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns ■DRAM □VRAM □256K □512K □1M □2.5M ■4M |  |
| CPU Type | ■OnBoard:NS Geode GXLV 233 MHz |  |
| HDD Type | ■Max □Sea □Con □Qua □WD: 91021U2 10GB |  |
| CD-ROM Type | ■SAMPO □Panasonic □Pioneer □Creative : CDM-T631 36X |  |
| Driver or Utility | □VGA □HDD ■Others : MediaGX 4.0 |  |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 |  |  |  |  |  |
|  | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1280X1024 | 16 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: NS Geode GXLV 233 |
| Boot |  |  |  |
| Boot to Windows NT |  |  |  |
| Boot to MS-DOS |  |  |  |
| Last known good configuration |  |  |  |
| Original Configuration |  |  |  |
| Shutdown |  |  |  |
| Shutdown & Restart |  |  |  |
| CPU/Memory/File System |  |  |  |
| Memory size less than 64MB |  |  |  |
| Memory size Larger than 64MB |  |  |  |
| System tasking settings |  |  |  |
| FAT |  |  |  |
| NTFS |  |  |  |
| Nwtwork |  |  |  |
| System clock |  |  |  |
| Task switching |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| CapLock,NumLock,ScrLock |  |  |  |
| Ctrl+Alt+Del |  |  |  |
| Ctrl+Esc,Alt+Tab/Esc/Space |  |  |  |
| Alt+Tab,Alt+Esc,Alt+Space |  |  |  |
| COM Port |  |  |  |
| Serial Mouse |  |  |  |
| Security/Audit/Control |  |  |  |
| User manager |  |  |  |
| Server manager |  |  |  |
| Disk administrator |  |  |  |
| Performance monitor |  |  |  |
| User profile editor |  |  |  |
| Event Viewer |  |  |  |
| Windows NT Diagnostics |  |  |  |

D. O.S. Compatibility-DOS and DOS Utility Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) |  | Brand: D4516821AG5-A10-9NF □8 ■16 □32 □64□128MB □DRAM□SDRAM■SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- | --- |
| Cache Memory Capacity & Brand | Brand: □60ns □70ns □ ns □PB □128K □256K □512K |  |
| System BIOS Version | ■Award □AMI Ver: V 4.51 PG |  |
| VGA Card & BIOS | ■OnBoard CS5530 |  |
| VGA Memory Capacity & Brand | Brand: □60ns □70ns □ ns ■DRAM □VRAM □256K □512K □1M □2.5M ■4M |  |
| CPU Type | ■OnBoard:NS Geode GXLV 233 MHz |  |
| HDD Type | ■IBM □Sea □Con □Qua □WD: DHEA-34330 3.4G |  |
| CD-ROM Type | ■NEC □Panasonic □Pioneer □Creative : CDR-3001A |  |
| Driver or Utility | □VGA □HDD □Others : |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | Testing CPU: NS Geode GXLV 233 |
| Function Test |  |  |  |
| Sysinfo Check (Before DOS Initialed ) |  |  |  |
| System Clock(Compare to RTC CLK) |  |  |  |
| DOS Prompt |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| Serial Mouse |  |  |  |
| DOS Commander |  |  |  |
| MSD.COM |  |  |  |
| EDIT.COM |  |  |  |
| EMM386.EXE |  |  |  |
| HIMEN.SYS |  |  |  |
| Norton Commander |  |  |  |
| Key F1~F10 Function |  |  |  |
| Key Alt+F1~F10 Function |  |  |  |
| Key Ctrl+F1~F10 Function |  |  |  |
| ET Ver3.53 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |