PCM-4894

QE Compatibility Test Report

Release Date: AUG. 20, 1999

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility -------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------- 9

Includes burn-in and booting tests.

4. Performance ---------------------------------------------------------------------------------------------------10

Includes performance indexes using various performance test software programs.

5. Power consumption data-------------------------------------------------------------------------------------11

Includes +5V current rate

6. Margin ----------------------------------------------------------------------------------------------------------12

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

- Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | KB Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2/V (J) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  | N/A |  |
| WINDOWS 95 (E/J) |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 98 (E) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### WINDOWS NT4.0W/S(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### WINDOWS NT4.0W/S(J) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Amidiag4.5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Com Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Lpt Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| AMD | AM5X86-P75-S at 133M | Pass | OnBoard |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PANASONIC | MN4117400BSJ-06 32M FPM RAM | Pass |  |
| MITSUBISH | M5M417400CJ-7 16M FPM RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| PANASONIC | W/P PP716M66 16M FPM RAM | Pass |  |
| NEC | 4217405-60 16M EDO RAM | Pass |  |
| SIEMENS | HYB5118165BSJ-60 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 16M EDO RAM | Pass |  |
| PANASONIC | MN414400BSJ-07 8M FPM RAM | Pass |  |
| LGS | GM71C17403CJ6 32M EDO RAM | Pass |  |
| NEC | 4218165-60 8M EDO RAM | Pass |  |
| TI | TMS417409ADJ-6 16M EDO RAM | Pass |  |
| ### MITSUBISHI | M5M418165CJ-6 8M EDO RAM | Pass |  |
| LGS | GM71C17403CJ6 16M EDO RAM | Pass |  |
| LGS | GM71C18163CJ6 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164BJC-60 8M EDO RAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On Board | C&T 65550 | Pass |  |
| TopView | S3 86C801 | Pass |  |
| TopView | S3 Virge | Pass |  |
| Cardex | ET-6000 | Fail | (BYPASS) |
| TopView | ET-4000 W32P | Fail | (BYPASS) |
| ### ISA Bus |  |  |  |
| TopView | ET-4000 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### On Board | ### Realtek RTL8029 | Pass |  |
| D-Link | DE 530 | Pass |  |
| 3COM | FAST ETHER LINK | Pass |  |
| Realtek | RTL8139 | Pass |  |
| ### Surecom | Digital 21140-AF | Pass |  |
| ISA Bus |  |  |  |
| 3COM | 3C509 | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### AHA | ### 2940UW | Pass |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MPEG Card |  |  |  |
| LEMEL Magic | MPEG Card | Pass |  |
| Sound Card |  |  |  |
| ### Creative | SB 16 Value PNP | Pass | (Require power voltage –12V) |
| Creative | AWE 64 PNP | Pass | (Require power voltage –12V) |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA Card |  |  |  |
| AAEON | PCD-894A | Pass |  |
| AAEON | PCD-895A | Pass |  |
| AAEON | PCD-896 | Pass |  |
| AAEON | RDM-1010A | Pass |  |
| AAEON | RDM-1020N | Pass |  |
| 104 Module |  |  |  |
| AAEON | PCM-3820/C | Pass |  |
| AAEON | PCM-3110B | Pass |  |
| AAEON | PCM-3111 | Pass |  |
| AAEON | PCM-3420 | Pass |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Quantum | ### ST 32A013 | Pass |  |
| Seagate | ST51270A | Pass |  |
| Maxtor | 83240D4 | Pass |  |
| Seagate | ST31230N SCSI | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### CD/ROM |  |  |  |
| VUGEO | 624A-004 | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 33.6 HIGH SPEED MODEM | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| ACER | 6311-TW | Pass |  |
| BTC | 5122 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |
| Logitech | First Mouse | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG 4.5 | Pass |  |

B. Booting Stability Testing

Test Condition: Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0﹪ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0﹪ |  |
| KB Reset 20 Times | 20 | 20 | 0﹪ |  |

C. Booting Stability Testing

Test Condition: Temp.  60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0﹪ |  |
| H/W Reset 1000 Times | 1000 | 999 | 0.1﹪ |  |

D. Burn-In Stability Testing

Test Condition: Burn-In Under Temp. -10℃~70℃ Burn-In 48 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAD 4.5 | Pass |  |

E. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. -10℃~70℃ Burn-In 24 Hours

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 42 Hours | 1000 | 1000 | 0﹪ |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | LGS GM71C17403CJ6 32MB X2 |
| --- | --- |
| VGA Card | OnBoard (C&T) |
| HDD TYPE | Quantum ST32A013 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=hing Files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AM5X86 P75-s 133 |  |  |  |  |  |  |  |  |  |  |  |  |  | Unit |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU |  | ### 447.05 |  |  |  |  |  |  |  |  |  |  |  |  |  | Mhz |
| FPU |  | 1139.52 |  |  |  |  |  |  |  |  |  |  |  |  |  | Mhz |
| VIDEO |  | 8747 |  |  |  |  |  |  |  |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 288.7 |  |  |  |  |  |  |  |  |  |  |  |  |  | ID |  |
| Power Meter 2.02 | 40.6 |  |  |  |  |  |  |  |  |  |  |  |  |  | Mips |  |
| PcBench 96 CPUmark16 | 105.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WinStone 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 32.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High-End Winstone | 32.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WinBench 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Graphic WinMark 96 | 10.4 |  |  |  |  |  |  |  |  |  |  |  |  |  | MP/s |  |
| ###### CPUmark 32 | 81.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPUmark 16 | 93.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10736 |  |  |  |  |  |  |  |  |  |  |  |  |  | KB/sec |  |
| Performance ID | 70.77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WINTECH 1.2 | 82.06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | Panasonic MN4117405CSJ-06 EDO RAM 32MBx2 |
| --- | --- |
| VGA Card | OnBoard C&T65550 |
| ### FLOPPY TYPE | Teac FD-235HF |
| VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Processor Type | Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # AM5X86 P75-S 133 | 2.6 | 1 | Amp/Max |

Note:

 1.Power supply voltage : ＋5V

   2.Suspend Model all function off

6. Margin Testing

A. Temperature Test (Software: AMIDIAG4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -10℃ |  |

B. Temperature Test (Boot Up)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -10℃ |  |

C. DC Voltage Test (Software: Qaplus/fe 5.29)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.3 V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4 V |  |

D. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device: HDD Quantum ST32A013

Slave Device: CD-ROM Vugeo 624A-004

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |