PCM-4898

## QE Compatibility Test Report

Release Date :Aug. 14, 2000

__________        __________     __________     __________

# Issue Stamp                     QA Manager               QE Manager               Test Engineer

Summary of contents :

Cover Sheet																		1

Summary of contents																2

1. Margin																					3

Includes temperature, DC voltage and IDE cable length tests.

2. Stability 																				4

Includes burn-in and booting tests .

A. Burn-In Testing(25℃/24hrs)															4

B. Burn-In Testing(Climatic Sequence/48hrs)												4

C. Booting Testing(25℃)																4

D. Booting Testing(Damp Heat,Cyclic/24hrs)												4

3. Performance																			5

Includes performance indexes using various performance test software programs .

4. Power consumption data																6

Includes +5V current rate .

5. Hardware compatibility																7

Includes CPU , DRAM , adapter card and HDD compatibility tests.

A. Component Compatibility- CPU														7

B. Component Compatibility- DRAM SIMM Module										7

C. Adapter Compatibility- VGA															7

D. Adapter Compatibility- LAN															8

E. Adapter Compatibility- SCSI Card														8

F. Adapter Compatibility- Other Card														8

G. Peripheral Compatibility- HDD Device													8

H. Peripheral Compatibility- Storage Device													9

I. Peripheral Compatibility- Others  														9

J. Peripheral Compatibility- Input Device													10

K. On_Board Network Hardware (physical) Compatibility										10

6. Software compatibility																12

Includes various OS and application software tests.

A. O.S. Compatibility- Windows 95														12

B. O.S. Compatibility- Windows 98														13

B-1. Microsoft Windows 98 Hardware Compatibility Test(HCT)									14

C. O.S. Compatibility- Windows NT R4.0													15

D. O.S. Compatibility- DOS and DOS Utility Compatibility Testing								17

E. Word Processing/Spread Sheet Compatibility Testing										18

F. Business Graphics/DTP/DB Compatibility Testing											20

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

Quality Engineering Department of AAEON TECHNOLOGY  INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Hsin-Tien City, Taipei, Taiwan , ROC

Tel: 886-2-29121234

Fax:886-2-29130792

886-2-29130774

1. Margin Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4516821AG-A10-9NF □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard NS Geode CS5530A |
| DOC Type | M-System DOC2000 □4MB □8MB ■16MB □24MB □___MB |

Test Result:

| High Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: Cyrix GXm-266P |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FE V5.29 |  |  |  |  | □ AMIDIAG V4.50 |  |  |  |  |
| □55℃ | □60℃ |  | □65℃ | ■70℃ |  | □ 75℃ |  | □ 80℃ | □ 85℃ |  |  |

| Low Temperature: 2 Hours |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing CPU: Cyrix GXm-266P |  |  |  |  | □ COM1 LB |  | □ COM2 LB |  |  | □ LPT LB |  |
| Test Program: |  | ■ QAPlus/FEV5.29 |  |  | □ AMIDIAG V4.50 |  | □5℃ |  |  | □0℃ |  |
| □-5℃ | ■-10℃ |  | □-15℃ | □-20℃ |  | □-25℃ |  | □-30℃ | □-35℃ |  | □-40℃ |

| High Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Cyrix GXm-266P |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| □A Grade: 5.20V | ■B grade: 5.25V | □C Grade: 5.30V | □D Grade: 5.35V | □E Grade: 5.4V |

| Low Voltage: 2 Hours/25℃ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Testing CPU: Cyrix GXm-266P |  | □ COM1 LB | □ COM2 LB | □ LPT LB |
| Test Program: | ■ QAPlus/FE V5.29 |  | □ AMIDIAG V4.50 |  |
| □A Grade: 4.80V | ■B grade: 4.75V | □C Grade: 4.70V | □D Grade: 4.65V | □E Grade: 4.6V |

2﹒Stability Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4516821AG-A10-9NF □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver:4.51PG |
| O.S. Environment | ■MSDOS6.22 □_______________ |
| VGA Card | ■OnBoard NS Geode CS5530A |
| DOC Type | M-System DOC2000 □4MB □8MB ■16MB □24MB □___MB |

A. Burn-In Testing (24Hours)

Test Condition : Under Room Temp. 25℃

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| QAPlus/FE V5.29 |  |  |  |

B. Burn-In Testing (48 Hours)

Test Condition : Burn-In Under Temp. 0℃〜60℃(IEC-68-2-61)

| Test Software | Pass | Fail | Note |
| --- | --- | --- | --- |
| QAPlus/FE V5.29 |  |  |  |

##### C. Booting Testing

Test Condition : Under Room Temp. 25℃

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0 % | 20 Sec | #### 5 Sec |  |
| KB Reset 20 Times | 20 | 20 | 0 % | 20 Sec | 5 Sec |  |

D. Burn-In Booting Testing (24 Hours)

Test Condition : Burn-In Under Temp. -0℃〜60℃(IEC-68-2-30)

| Test Method | Actual | Successful | Failure rate | On time | Off Time | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Power On/Off | 1365 | 1363 | 0.15% | 50 Sec | #### 50 Sec |  |

3﹒Performance Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V658020 TC-10 □8 □16 □32□64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 □Windows NT □ |
| VGA Card | ■OnBoard NS Geode CS5530A |
| VGA Memory | □256K □512K □1M ■3M □4M □ |
| HDD Type | □Max □Sea □Con ■IBM □Fuj : DPTA-372050 |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Testing Utility | Cyrix GXM | Cyrix GXM | NS Geode | NS Geode | Unit |
|  | GXm-233GP | GXm-266GP | GX1-300P 1.69k | GXLV-233 1k |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |
| CPU | 734.29 | 890.46 | 1030.41 | 743.21 | MHz |
| ### FPU | 1991.70 | 2381.12 | 2763.05 | 1980.64 | MHz |
| VIDEO | 13284 | 16115 | 18904 | 13284 | Chs/sec |
| ### Norton 8.0 |  |  |  |  |  |
| SI | 448.7 | 541.1 | 629.1 | 453.1 | ID |
| ### Power Meter 2.02 |  |  |  |  |  |
| MIPS Test | 67.7 | 83.6 | 94.8 | 71.1 | Mips |
| WinStone 99 v1.2 |  |  |  |  |  |
| Business Graphic Winstone99 | 7.11 | 7.99 | 8.85 | 7.11 |  |
| WinBench 99 |  |  |  |  |  |
| H.E. Graphic WinMark 99 | 76.2 | 88.6 | 99.9 | 76.8 | MP/s |
| Business Graphic WinMark99 | 26.4 | 30.4 | 34.5 | 26.6 | TB/s |
| CPUmark 99 | 7.24 | 8.41 | 9.37 | 7.26 |  |
| ### Coretest V3.02 |  |  |  |  |  |
| DTR (Buffered Read) | 5920 | 6256 | 6480 | 5936 | KB/Sec |
| Performance ID | 43.69 | 45.69 | 46.9 | 43.79 |  |
| ### Wintech V1.2 |  |  |  |  |  |
| Overall WinTach RPM | 116.55 | 134.70 | 151.71 | 115.64 |  |

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V658020 TC-10 □8 □16 □32□64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| O.S. Environment | ■MSDOS6.22 ■Windows 98 □Windows NT □ |
| VGA Card | ■OnBoard NS Geode CS5530A |
| VGA Memory | □256K □512K □1M ■3M □4M □ |
| HDD Type | □Max □Sea □Con ■IBM □Fuj : DPTA-372050 |
| VGA Testing Mode | 1024X768X256c (For WinStone & WinBench) |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Testing Utility | NS Geode | NS Geode |  |  | Unit |
|  | G1-266P 2.2k | G1-200P 1k+2.7k |  |  |  |
| ### LandMark Speed 2.0 |  |  |  |  |  |
| CPU | 888.09 | 616.26 |  |  | MHz |
| ### FPU | 2385.29 | 1655.47 |  |  | MHz |
| VIDEO | 16115 | 10922 |  |  | Chs/sec |
| ### Norton 8.0 |  |  |  |  |  |
| SI | 540.8 | 376.4 |  |  | ID |
| ### Power Meter 2.02 |  |  |  |  |  |
| MIPS Test | 83.6 | 58.0 |  |  | Mips |
| WinStone 99 v1.2 |  |  |  |  |  |
| Business Graphic Winstone99 | 8.01 | 6.43 |  |  |  |
| WinBench 99 |  |  |  |  |  |
| H.E. Graphic WinMark 99 | 88.4 | 70.5 |  |  | MP/s |
| Business Graphic WinMark99 | 30.3 | 24 |  |  | TB/s |
| CPUmark 99 | 8.39 | 6.43 |  |  |  |
| ### Coretest V3.02 |  |  |  |  |  |
| DTR (Buffered Read) | 6256 | 5696 |  |  | KB/Sec |
| Performance ID | 45.69 | 42.36 |  |  |  |
| ### Wintech V1.2 |  |  |  |  |  |
| Overall WinTach RPM | 135.24 | 107.04 |  |  |  |

4﹒Power Consumption

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A80-9JF 128MB |
| --- | --- |
| VGA Card | Onboard NS Geode CS5530A |
| DOC or Floppy | M-Systems DOC 16MB |
| O.S. | DOS6.22 |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

ATX Power

| Mode Type | CPU Type |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | Cyrix GXM GXm-233GP | Cyrix GXM GXm-266GP | NS Geode GX1-300P R393 : 1.69k | NS Geode GXLV-233 R393 : 1k |  |
| Normal Mode(+5V) | 2.12 | 1.94 | 1.78 | 2.06 | amp/max |
| Doze Mode(+5V) | 2.12 | 1.94 | 1.78 | 2.06 | amp/max |
| Standby Mode(+5V) | 1.01 | 1.01 | 1.03 | 1.01 | amp/max |

| Mode Type | CPU Type |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | NS Geode G1-266 R393 : 2.2k | NS Geode G1-200 R393 : 1k+2.7k |  |  |  |
| Normal Mode(+5V) | 1.61 | 1.40 |  |  | amp/max |
| Doze Mode(+5V) | 1.61 | 1.40 |  |  | amp/max |
| Standby Mode(+5V) | 1.02 | 0.97 |  |  | amp/max |

Normal Model is meassured while open system

Doze Model VGA controller is still enabled

STANDBY Model VGA controller is still enabled

5﹒Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Cyrix | ### GXm-233GP |  |  | 2.9V |
| Cyrix | GXm-266GP |  |  | 2.9V |
| NS Geode | GXLV-233P |  |  | 2.2V |
| NS Geode | G1-300P |  |  | 2V |
| NS Geode | G1-266P |  |  | 1.8V |
| #### NS Geode | G1-200P |  |  | 1.6V |

##### B. Component Compatibility- DRAM DIMM Module

DRAM DIMM Model List:

| Brand | Device Model | #### Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| HYNDAI | HY57V658020 TC-10 128MB |  |  |  |
| NEC | D4564841G5-A80-9JF 128MB |  |  |  |
| NEC | D4564841G5-A10-9JF 128MB |  |  |  |
| LGS | GM72V661641CT7J 64MB |  |  |  |
| HITACHI | 5264805DTTA60 PC100-222-60 64MB |  |  |  |
| HYNDAI | HY57V1291620 TC-10S 64MB |  |  |  |
| HYNDAI | HY57V129160 LTC-10S 64MB |  |  |  |
| NEC | D4564841G5-A10-9JF 64MB |  |  |  |
| NEC | D4564841G5-A60-9JF 64MB |  |  |  |
| NEC | D4564841G5-A80-9JF 64MB |  |  |  |
| HYNDAI | HY57V168010D TC-10S 32MB |  |  |  |
| NEC | D4516821AG-A10-9NF 32MB |  |  |  |
| MICRO | MT48LC2M8A1 TG –10S 32MB |  |  |  |
| SIEMENS | HYB39S16800A7 –10 32MB |  |  |  |
| MITSUBISHI | M2V64S40BTP 32MB |  |  |  |
| MITSUBISHI | M5M4V16S30DTP 32MB |  |  |  |
| HITACHI | HM5216805TT10H 16MB |  |  |  |
| NEC | D4516821AG5-A10-9NF 16MB |  |  |  |

C. Adapter Compatibility- VGA Card

VGA Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Onboard VGA | NS Geode CS5530A |  |  |  |

D. Adapter Compatibility- LAN Card

Action:

| 1. Load ODI Driver (Lsl,Ipxodi,Vlm Or Netx) | 2. Login To Server System |
| --- | --- |
| 3. Transfer Data Between SRV And WK(Mulity Task) | 4. logout |

LAN Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| #### Intel | 82557 |  |  |  |
| Intel | 82559 |  |  |  |
| Intel | EtherExpressTM PRO/100 (ISA) |  |  |  |
| D-Link | DFE-530TX |  |  |  |
| REL | RTL8139A |  |  |  |
| 3 COM | 3C905B-TX |  |  |  |
| 3 COM | 3C509B(ISA) |  |  |  |

E. Adapter Compatibility- SCSI Card

SCSI Adapter List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Adaptec | AHA-2940U/UW |  |  |  |
| Adaptec | AHA-1520 |  |  |  |
| Adaptec | AHA-2940U |  |  |  |
| Adaptec | AHA-2940U2W |  |  |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| MPEG Card |  |  |  |  |
| YUAN | MPG-200C DVD MPEG-2 |  |  |  |
| Modem Card (PCI) |  |  |  |  |
| SmartLink | 5634PEW 56000bps Win modem |  |  |  |

##### G. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Quantum | 3240AT 3.2GB |  |  |  |
| Quantum | 1700AT 1.7GB |  |  |  |
| Seagate | ST34520A 4GB |  |  |  |
| Seagate | ST51270A 1.2GB |  |  |  |
| Quantum | Bigfoot 4.0AT 4GB |  |  |  |
| Western Digital | WD Caviar 28400 8.4GB |  |  |  |
| Western Digital | WD Caviar 26400 6.4GB |  |  |  |
| FUJITSU | MPC 3032AT 3.2GB |  |  |  |
| #### IBM | DTLA-307045 40GB |  |  |  |
| #### IBM | DPTA-372050 20GB |  |  |  |
| #### Seagate | ST31230N 1.0GB SCSI |  |  |  |
| #### Seagate | ST34520LW 4.3GB SCSI |  |  |  |
| #### Seagate | ST34520W 4.3GB SCSI |  |  |  |

##### H.Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| CD/ROM |  |  |  |  |
| TEAC | CD-540E 40X |  |  |  |
| CREATIVE | CD2423E 24X |  |  |  |
| PIONEER | DR-A24X 24X |  |  |  |
| #### SONY | CDU-4811-81 48X |  |  |  |
| #### PIONEER | DVD-A03S |  |  |  |
| ZIP |  |  |  |  |
| Iomega | Z100P2 |  |  |  |
| LS-120 |  |  |  |  |
| Panasonic | LKM-F934-1 |  |  |  |
| Fax/Modem |  |  |  |  |
| #### Cyber Lander | Rock Modem E210 V.90 56K |  |  |  |
| LEMEL | MD336RSVD 56K |  |  |  |

I.Peripheral Compatibility- Others

Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| RS-485 Port |  |  |  |  |
| Adventech | Adam 401 |  |  |  |
| USB Port |  |  |  |  |
| WINTEK | AM-767-U USB Mouse |  |  |  |
| DEXIN | A2U800A USB Mouse |  |  |  |
| Solidtek | ACK-298 USB Keyboard |  |  |  |
| BTC | 7932 USB Keyboard |  |  |  |
| Vcam | CU-98 USB Camara |  |  |  |
| Tekom | TekCam P35U USB Camara |  |  |  |
| HP | ScanJet 4200C USB Scanner |  |  |  |
| EPSON | STYLUS COLOR740 USB Printer |  |  |  |
| HP | PhotoSmart P1000 |  |  |  |
| Iomega | Z100USB |  |  |  |
| Parallel Port |  |  |  |  |
| EPSON | STYLUS COLOR740 |  |  |  |
| #### HP | PhotoSmart P1000 |  |  |  |
| IR Port |  |  |  |  |
| ACTiSYS | ACT-IR210L |  |  |  |

##### J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |
| Acer | 6511-TW1C/B |  |  |  |
| Mouse |  |  |  |  |
| DEXIN | K3P800A |  |  |  |
| LOGITECH | M-M35 |  |  |  |

K. On_Board Network Hardware (physical) Compatibility

System Information(Client Site):

| Board System | ■ MB-668 |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D4564841G5-A80-9JF □8 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: □1■2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: |
| O.S. Environment | ■DOS6.22 ■WINNT____ □NOVELL_____ □__________ |
| VGA Card | ■OnBoard C&T 69000 |
| POWER SUPPLY | □MPU-250W□SSH-250G □SSH-350G ■ST-250AF |
| IDE HDD TYPE | □Max □Sea □Con □Qua □Fuj ■IBM : DHEA-34330 |
| LAN CARD | ■OnBoard: RTL 8139B |
| CD-ROM TYPE | □NEC □PANASONIC ■PIONEER □CREATIVE : DVD-A035 |
| Cable/Cable Length | CAT5 100M |

System Information(Server Site):

| Board System | ■ MB-668 |
| --- | --- |
| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: LGS GM72V661641CT7J □8 □16 □32 ■64MB□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: 4.51PG |
| VGA Card | ■OnBoard Cyrix CX5530 |
| O.S. Environment | ■DOS6.22 ■WINNT____ ■NOVELL 4.11 □__________ |
| IDE HDD TYPE | □Max □Sea □Con ■Qua □Fuj □IBM : Bigfoot 4.0AT |
| LAN CARD | ■OnBoard: RTL-8139B |
| ### CPU Type | ■I ntel □Cyrix□AMD : Celeron 366 MHz |
| CD-ROM TYPE | □NEC □PANASONIC □PIONEER ■CREATIVE : CD2423E |
| Cable/Cable Length | CAT5 100M |

HUB Type: D-Link DES1008 10/100 Fast Ethernet Switch

Test Result:

| SUBJECT | Network Hardware Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Transceiver |  |  |  |  |
| Receiver |  |  |  |  |
| CRC error |  |  |  |  |
| Collision back-off generation |  |  |  |  |
| Auto-negotiation |  |  |  |  |
| ### 100M Base-TX full duplex |  |  |  |  |
| 100M Base-TX half duplex |  |  |  |  |
| 10M Base-T full duplex |  |  |  |  |
| 10M Base-T half duplex |  |  |  |  |
| SUBJECT | Network Operation System Function Testing |  |  |  |
| Test Item |  | Pass | Fail | Note |
| NetWare 4.11 |  |  |  |  |
| Windows NT 4.0 |  |  |  |  |
| Linux Unix |  |  |  |  |
| SUBJECT | Network Performance Testing |  |  |  |
| Test Item |  | Result |  | Note |
| ### NWTEST 24HR |  | #### Pass |  |  |

| SUBJECT | Workstation Application Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| Dos 6.22 for IPXODI |  |  |  |  |
| Dos 6.22 for package driver |  |  |  |  |
| Windows 3.1 for IPXODI |  |  |  |  |
| Windows 3.1 for package driver |  |  |  |  |
| WFW 3.11 for NDIS 2.0/3.0 |  |  |  |  |
| WFW 3.11 for NDIS 3.0/ODI |  |  |  |  |
| Windows 95 for IPX/SPX |  |  |  |  |
| Windows 95 for NetBEUI |  |  |  |  |
| Windows 95 for TCP/IP |  |  |  |  |
| Windows 98 for IPX/SPX |  |  |  |  |
| Windows 98 for NetBEUI |  |  |  |  |
| Windows 98 for TCP/IP |  |  |  |  |
| Windows NT 4.0 for IPX/SPX |  |  |  |  |
| Windows NT 4.0 for NetBEUI |  |  |  |  |
| Windows NT 4.0 for TCP/IP |  |  |  |  |
| Transfer Data Between SRV And WK |  |  |  | Mulity Task 12 Hours(NOTE 1.) |

Note : 1.Mulity Task is that transfers files(copy/verify/delet) and VCD in the same time.

6﹒Software Compatibility

A. O.S. Compatibility-Windows 95

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:NEC D4564841G5-A80-9JF □8 □16 □32■64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: NS Geode CS5530A Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | □Intel■Cyrix □AMD: GXm-266P |
| HDD Type | □Max ■WD□Con □Qua □Fuj : Caviar26400 6.4GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1280X1024 | 256 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: GXm-266P |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

B. O.S. Compatibility-Windows 98

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand:NEC D4564841G5-A80-9JF □8 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Memory Capacity & Brand | Brand: Cyrix 5530 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | □Intel■Cyrix □AMD: GXm-266P |
| HDD Type | □Max ■WD□Con □Qua □Fuj : Caviar26400 6.4GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 16 | — | — |  |  |  |
|  | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| ### 1280X1024 | 256 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: Cyrix GXm-266P |
| Boot |  |  |  |
| Boot Normal |  |  |  |
| Boot Logged (\BOOTLOG.TXT) |  |  |  |
| Boot Safe Mode |  |  |  |
| Boot Safe Mode & Network |  |  |  |
| Boot Step By Step Confirmation |  |  |  |
| Boot Command Prompt Only |  |  |  |
| Boot Safe Mode Command Prompt |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| Alt+F4 Quit a Program |  |  |  |
| Ctrl+Esc Display the Start Menu |  |  |  |
| Alt+Tab Switch To Last Windows |  |  |  |
| Ctrl+Alt+Del Close Program |  |  |  |
| F8 Start up Boot Up Menu |  |  |  |
| F4 Boot Up from Previous |  |  |  |
| Shutdown |  |  |  |
| Shutdown the Computer |  |  |  |
| Shutdown Restart |  |  |  |
| Shutdown to MS-DOS Mode |  |  |  |
| Close all Prog. & Login Other User |  |  |  |

B-1.Microsoft Windows 98 Hardware Compatibility Test(HCT)

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: NEC D451228841G5-A10-9JF □32 □64■128□256MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□ 2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: Cyrix CX5530 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M 0■3M □4M |
| CPU Type | ■Intel □Cyrix□AMD: Cyrix GXm-266P |
| HDD Type | □Max ■WD □Con □Qua □Fuj : Caviar 26400 6.4GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Keyboard TYPE | Acer 6511-TW1C/B |
| FDD TYPE | TEAC FD-235HF |
| Mouse | LOGITECH M-CQ38 |
| LAN CARD | Intel 82559ER |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

Test Result:

| HCT 7.6 Features Test | Pass | Fail | ### Remark |
| --- | --- | --- | --- |
| Virtual Dos Mach |  |  |  |
| DMPS Test |  |  |  |
| Video Playback |  |  |  |
| Extended BIOS Parameter Block |  |  |  |
| MCI CD Test |  |  |  |
| BCT API Test |  |  |  |
| INT 13 Extension |  |  |  |
| Disk(Stress) Individual Drive |  |  |  |
| Disk –CD (Stress) |  |  |  |
| Disk (Stress) |  |  |  |
| Coprocessor(Stress) |  |  |  |
| Net Class Dynamic |  |  |  |
| PCI I/O Space |  |  |  |
| PCI 2.1 (BIOS) |  |  |  |
| PCI 2.1 (Card) |  |  |  |
| Unreported I/O |  |  |  |
| Unreported Memory |  |  |  |
| PnP BIOS |  |  |  |
| APM 1.2 |  |  |  |
| Media Class Dynamic Config |  |  |  |
| Wave Drift |  |  |  |

C. O.S. Compatibility-Windows NT R4.0

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V168010D TC-10S □16 ■32 □64□128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□ 2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: Cyrix CX5530 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | ■Intel □Cyrix □AMD : Cyrix GXm-266P |
| HDD Type | □Max ■Sea □Con □Qua □Fuj : ST34520A 4GB |
| CD-ROM Type | □NEC ■TEAC □Pioneer □Creative : CD-532E 32X |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

VGA Driver Testing:

| Resolution | Color | SF | LF | Pass | Fail | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 640X480 | 256 | — | — |  |  |  |
|  | 32K | — | — |  |  |  |
|  | 64K | — | — |  |  |  |
| 800X600 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
|  | 64K |  |  |  |  |  |
| 1024X768 | 256 |  |  |  |  |  |
|  | 32K |  |  |  |  |  |
| 1280X1024 | 256 |  |  |  |  |  |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From CD/ROM |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: Cyrix GXm-266P |
| Boot |  |  |  |
| Boot to Windows NT |  |  |  |
| Boot to MS-DOS |  |  |  |
| Last known good configuration |  |  |  |
| Original Configuration |  |  |  |
| Shutdown |  |  |  |
| Shutdown & Restart |  |  |  |
| CPU/Memory/File System |  |  |  |
| Memory size less than 64MB |  |  |  |
| Memory size Larger than 64MB |  |  |  |
| Virtual memory |  |  |  |
| System tasking settings |  |  |  |
| FAT |  |  |  |
| NTFS |  |  |  |
| Nwtwork |  |  |  |
| System clock |  |  |  |
| Task switching |  |  |  |
| Function Test |  |  |  |
| Serial Mouse |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| PS/2 Mouse |  |  |  |
| System Clock |  |  |  |
| Keyboard Function Test |  |  |  |
| CapLock,NumLock,ScrLock |  |  |  |
| Ctrl+Alt+Del |  |  |  |
| Ctrl+Esc,Alt+Tab/Esc/Space |  |  |  |
| Alt+Tab,Alt+Esc,Alt+Space |  |  |  |
| Ctrl+Alt+Pause/Print Screen |  |  |  |
| Ctrl+Alt+F1~F12 |  |  |  |
| COM Port |  |  |  |
| Serial Mouse |  |  |  |
| Security/Audit/Control |  |  |  |
| User manager |  |  |  |
| Server manager |  |  |  |
| Disk administrator |  |  |  |
| Performance monitor |  |  |  |
| User profile editor |  |  |  |
| Event Viewer |  |  |  |
| Windows NT Diagnostics |  |  |  |

D. O.S. Compatibility-DOS and DOS Utility Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V658020 TC-10 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1□ 2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: Cyrix CX5530 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | □Intel■Cyrix □AMD: Cyrix GXm-266P |
| HDD Type | □Max □Sea □Con ■Qua □Fuj : 1700AT 1.7GB |
| CD-ROM Type | □NEC □Panasonic ■Pioneer □Creative : DVD-A03S |
| Driver or Utility | □VGA □HDD ■Others : Intel 82559ER LAN |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Installation From FDD |  |  |  |
| Install to IDE HDD |  |  | ###### Testing CPU: Cyrix GXm-266P |
| Function Test |  |  |  |
| Sysinfo Check (Before DOS Initialed ) |  |  |  |
| System Clock(Compare to RTC CLK) |  |  |  |
| DOS Prompt |  |  |  |
| FDD Format/Copy/Read/Write |  |  |  |
| Serial Mouse |  |  |  |
| DOS Commander |  |  |  |
| MSD.COM |  |  |  |
| EDIT.COM |  |  |  |
| EMM386.EXE |  |  |  |
| HIMEN.SYS |  |  |  |
| Norton Commander |  |  |  |
| Key F1~F10 Function |  |  |  |
| Key Alt+F1~F10 Function |  |  |  |
| Key Ctrl+F1~F10 Function |  |  |  |
| Telix |  |  |  |
| Setup Function |  |  |  |
| Connect with E288MX Fax |  |  |  |
| Dialing Function |  |  |  |
| File Upload Function |  |  |  |
| File Download Function |  |  |  |
| ET Ver3.53 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |
| ET 2000 |  |  |  |
| Installation Testing |  |  |  |
| 16 Font Mode |  |  |  |
| 24 Font Mode |  |  |  |
| Key: Ctrl+Esc Function |  |  |  |
| Sethrc Function |  |  |  |
| ETFree Function |  |  |  |

E. Word Processing/Spread Sheet Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V658020 TC-10 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: C&T69000 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | □Intel■Cyrix □AMD: Cyrix GXm-266P |
| HDD Type | □Max ■WD □Con □Qua □Fuj : Caviar 26400 6.4GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| Ami Pro 3.1 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Style Function |  |  |  |
| Page Function |  |  |  |
| Frame Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Word 6.0c |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Table Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| WordPerfect 6.1 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Table Function |  |  |  |
| Graphics Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Works 3.0b |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Lotus 1-2-3 R5 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Style Function |  |  |  |
| Tools Function |  |  |  |
| Range Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| MS Excel 5.0c |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Data Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Quattro Pro 6.01 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Block Function |  |  |  |
| Notebook Function |  |  |  |
| Graphics Function |  |  |  |
| Tools Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |

F. Business Graphics/DTP/DB Compatibility Testing

System Information:

| DRAM/SDRAM Capacity & BRAND(SIMM Qty) | Brand: HYNDAI HY57V658020 TC-10 □16 □32 □64■128MB □DRAM■SDRAM□SODIMM □________ Qty: ■1 □2 □3 □4 |
| --- | --- |
| System BIOS Version | ■Award □AMI Ver: 4.51PG |
| VGA Card & BIOS | ■OnBoard Cyrix CX5530 |
| VGA Memory Capacity & Brand | Brand: Cyrix CX5530 Built-in □60ns □70ns □ ns □DRAM □VRAM □256K □512K □1M ■3M □4M |
| CPU Type | □Intel■Cyrix □AMD: Cyrix GXm-266P |
| HDD Type | □Max ■WD □Con □Qua □Fuj : Caviar 26400 6.4GB |
| CD-ROM Type | □NEC □Panasonic □Pioneer ■Creative : CD2423E |
| Driver or Utility | ■VGA □HDD ■Others : Intel 82559ER LAN |

Test Results:

| Test Item | Pass | Fail | Note |
| --- | --- | --- | --- |
| PageMaker 5.0a |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| Utilities Function |  |  |  |
| Layout Function |  |  |  |
| Type Function |  |  |  |
| Element Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| CorelDraw 5.0E2 |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Layout Function |  |  |  |
| Arrange Function |  |  |  |
| Effects Function |  |  |  |
| Text Function |  |  |  |
| Special Function |  |  |  |
| Help Function |  |  |  |
| PowerPoint 4.0c |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Insert Function |  |  |  |
| Format Function |  |  |  |
| Tools Function |  |  |  |
| Draw Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| DBase 5.0 |  |  |  |
| ### File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Report Function |  |  |  |
| Program Function |  |  |  |
| Table Function |  |  |  |
| Properties Function |  |  |  |
| Help Function |  |  |  |
| Paradox 5.0 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Table Function |  |  |  |
| Query Function |  |  |  |
| Properties Function |  |  |  |
| Tools Function |  |  |  |
| Record Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |
| Access 2.0 |  |  |  |
| File Function |  |  |  |
| Edit Function |  |  |  |
| View Function |  |  |  |
| Security Function |  |  |  |
| Format Function |  |  |  |
| Record Function |  |  |  |
| Windows Function |  |  |  |
| Help Function |  |  |  |