SBC-355N

QE Compatibility Test Report

Release Date: Aug. 23, 1999

__________   __________   __________   __________

# Issue Stamp                 QA Manager               QE Manager               Test Engineer

Contents

1. Margin ------------------------------------------------------------------------------------------------------- 3

Includes temperature, DC voltage and IDE cable length tests.

2. Stability ------------------------------------------------------------------------------------------------------ 4

Includes burn-in and booting tests.

3. Performance ------------------------------------------------------------------------------------------------ 5

Includes performance indexes using various performance test software programs.

4. Hardware compatibility ---------------------------------------------------------------------------------- 6

Includes CPU, DRAM, adapter card and HDD compatibility tests.

5. Software compatibility ----------------------------------------------------------------------------------- 8

Includes various OS and application software tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

1. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 75℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -35℃ |  |

B. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.6V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4V |  |

C. IDE Drive Capability

### Test Equipment:

### Master Device: HDD Quantum FB1280A

Slave Device: CD-ROM  Creative CD620E

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 7.5CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |

2. Stability Testing

### A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

### B. Booting Stability Testing

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0％ |  |
| KB Reset 20 Times | 20 | 20 | 0％ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0％ |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000Times | 1000 | 1000 | 0％ |  |
| H/W Reset 1000Times | 1000 | 1000 | 0％ |  |

3. Performance Testing

System Information:

| DRAM Type & Capacity | 16 MB |
| --- | --- |
| VGA Card | PCM-3520 A1 (CHIPS F65535 A) |
| HDD TYPE | Seagate ST31275A |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | #### ALI 6117B |  |  |  |  | #### Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 36.85 |  |  |  |  | Mhz |
| FPU | N/A |  |  |  |  | Mhz |
| VIDEO | 1837 |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 32.9 |  |  |  |  | ID |
| Power Meter 2.02 | 4.8 |  |  |  |  | Mips |
| PcBench 9.0 CPUmark16 | 20.25 |  |  |  |  |  |
| H.E. WinStone 96 1.0 | 7.3 |  |  |  |  |  |
| WinBench 96 Ver 1.0 |  |  |  |  |  |  |
| Graphic WinMark 96 | 1.14 |  |  |  |  | MP/s |
| CPUmark 16 | 20.8 |  |  |  |  |  |
| CPUmark 32 | 23.9 |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |
| DTR (Buffered Read) | 2528 |  |  |  |  | KB/sec |
| Performance ID | 19.27 |  |  |  |  |  |
| WINTECH 1.2 | 1.91 |  |  |  |  |  |
| Power consumption (+5V in normal mode) | 6.8 |  |  |  |  | A |
| NOTE |  |  |  |  |  |  |

4. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| # Brand | # Device Model | # Result | # Note |
| --- | --- | --- | --- |
| ALI | 6117B | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| NEC | 4217400-60 16M FPM | Pass |  |
| NEC | 424100-70 16M FPM | Pass |  |
| NEC | D424400-70L 4M(X2) FPM | Pass |  |
| SIEMENS | HYB5117400JC-70 16M RAM FPM | Pass |  |
| SAMSUNG | KM44C4000AK-6 16M RAM FPM | Pass |  |
| MITSUBISH | M5M417400CJ 16M RAM FPM | Pass |  |

### C. Adapter Compatibility- VGA

VGA Adapter List:

| # Brand | # Device Model | # Result | # Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| Topview | Et 4000 ISA | Fail |  |
| AAEON | Pca-6642 (ET4000W32I) | ## Pass |  |
| # On Board | # Chips 65545 | ## Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| # ISA BUS |  |  |  |
| 3COM | 3C509 | Pass |  |
| Novell | NE2000 | Pass |  |

### E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| # ISA BUS |  |  |  |
| AHA | 1542CF | Pass |  |

F. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| # 104 Module |  |  |  |
| AAEON | PCM-3810 (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) | Pass |  |

### G. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ###### Conner | ###### CFS425A 425M | ###### Pass |  |
| WD | 22100 | Pass |  |
| Fujitsu | M1638TAU | Pass |  |
| Quantum | 850AT | Pass |  |
| Quantum | TB540S SCSI | Pass |  |
| Maxtor | 7540AV 540M | Pass |  |
| Segate | 5151270A | Pass |  |
| Segate | ST32132A | Pass |  |

### H. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| PHILIPS | PCA123CD | Pass |  |
| WUEGD | 612A-00 PX | Pass |  |

I. Peripheral Compatibility- Input Device

Input Device List:

| # Brand | # Device Model | # Result | # Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| ###### FOCUS | ###### 101KB | ###### Pass |  |
| Mouse |  |  |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

5. Software Compatibility

### A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  | Shutdown |  | Function |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| MS-DOS 6.22 |  | ü |  | ü |  | ü |  | ü |  | ü |  |
| QNX 4.22 & Wins |  | ü |  | ü |  | ü |  | ü |  | ü |  |
| NOVELL 3.12 |  | ü |  | ü |  | ü |  | ü |  | ü |  |

### B. Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | K/B Function |  | VGA Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.1 | ü |  | ü |  | ü |  | ü |  | ü |  | N/A |  |
| Windows 95 | ü |  | ü |  | ü |  | ü |  | ü |  | N/A |  |

### C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Result | Note |  | Result | Note |
| Ami Pro 3.1 | Pass |  |  | Pass |  |
| MS Word 6.0c | Pass |  |  | Pass |  |
| WordPerfect 6.1 | Pass |  |  | Pass |  |
| MS Works 3.0b | Pass |  |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  |  | Pass |  |
| MS Excel 5.0c | Pass |  |  | Pass |  |
| Quattro Pro 6.01 | Pass |  |  | Pass |  |
| PageMaker 5.0a | Pass |  |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  |  | Pass |  |
| PowerPoint 4.0c | Pass |  |  | Pass |  |
| Dbase 5.0 | Pass |  |  | Pass |  |
| Paradox 5.0 | Pass |  |  | Pass |  |
| Access 2.0 | Pass |  |  | Pass |  |

### D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- |
| ###### Norton Utility 8.0 |  | ###### Pass |  |  |  |  |
| ET 3.53 | 16 * 15 Font Mode |  |  | Pass |  |  |
|  | 24 * 24 Font Mode |  |  | N/A |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode |  |  | Pass |  |  |
|  | 24 * 24 Font Mode |  |  | N/A |  |  |

### E. Diagnostics Software Testing

| # Test Item / Software | # QAFE5.29 |  | # AMIDIAG4.5/5.1 |  | # CHECKIT3.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note | Result | Note |  |  |
| Main Component | Pass |  | Pass |  | Pass |  |  |  |
| Memory | Pass |  | Pass |  | Fail |  |  |  |
| Video Adapter | Pass |  | Pass |  | Pass |  |  |  |
| Floppy Drive | Pass |  | Pass |  | Pass |  |  |  |
| Hard Drive | Pass |  | Pass |  | Pass |  |  |  |
| Keyboard | Pass |  | Pass |  | Pass |  |  |  |
| LPT Port | Pass |  | Pass |  | Pass |  |  |  |