AAEON

### ISO-9001/ISO-14001 Certified

### Industrial Automation PCs

SBC-456E

QE Compatibility Test Report

Release Date: Jun. 08, 1999

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility --------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------------- 8

Includes burn-in and booting tests.

4. Performance --------------------------------------------------------------------------------------------------------- 9

Includes performance indexes using various performance test software programs.

5. Power Consumption Data ----------------------------------------------------------------------------------------- 10

Includes +5V current rate.

6. Margin ---------------------------------------------------------------------------------------------------------------- 11

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

- Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| WINDOWS 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS NT 4.0 W/S |  |  |  |  |  |  |  |  |  |  |  |  |
| WINDOWS NT 4.0 SRV |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| Norton Commander 5.0 |  |  | Pass |  |
| Telix 3.2x |  |  | Pass |  |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | #### Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Qaplus5.5 |  |  | Amidiag4.5 |  |  | Amidiag5.11 |  |  | Pccheck3.02 |  |  | Checkit3.0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Memory |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Floppy Drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hard Drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Com Port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LPT Port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plug & play | N/A |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

486 CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| AMD | ON BOARD 5X86-133 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PANASONIC | MN4117400BSJ-06 32M FPM RAM | Pass |  |
| MITSUBISH | M5M417400CT-7 16M FPM RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| SAMSUNG | KM44C4000AK-6 16M FPM RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| HYNDAI | HY5117400AJ-70 16M FPM RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| SAMSUNG | KM41C16000BK-6 64M FPM RAM | Pass |  |
| PANASONIC | PP716M66 16M W/PARITY | Pass |  |
| NEC | NX716-6 16M EDO RAM | Pass |  |
| SIEMENS | HYB5118165BSJ-60 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BSJ-60 8M EDO RAM | Pass |  |
| PANASONIC | MN414400BSJ-07 16M FPM RAM | Pass |  |
| LGS | GM71C17403CJ6 32M EDO RAM | Pass |  |
| NEC | 421865-60 8M EDO RAM | Pass |  |
| MICON | MT4C4M4E8DJ-6 16M EDO RAM | Pass |  |
| TI | TMS417409ADJ-6 16M EDO RAM | Pass |  |
| ### MITSUBISHI | ### M5M418165CJ 8M EDO RAM | Pass |  |
| LGS | GM71C17403CJ6 16M EDO RAM | Pass |  |
| LGS | GM71C18163CJ6 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164B 8M EDO RAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| ### On_board | CHIPS 65550 | #### Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| On-board | RTL-8029 | Pass | SBC-456E only |
| ISA BUS |  |  |  |
| 3COM | 3C509 | Pass |  |
| Novell | NE2000 | Pass |  |
| Novell | NE2000 + | Pass |  |
| SVAC | ETHER 16 | Pass |  |
| On-board |  | #### N/A |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| AHA | 1520K ISA | Pass |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### Sound Card |  |  |  |
| Creative | AWE 32 | Pass |  |
| Creative | SB 16 Value PNP | Pass |  |
| ON BOARD |  | N/A |  |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA CARD |  |  |  |
| AAEON | RDM-1020N | Pass |  |
| AAEON | RDM-1010A | Pass |  |
| 104 Module |  |  |  |
| AAEON | PCM-3820C (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3420 (SCSI) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCD-895 | Pass |  |
| AAEON | PCD-896 | Pass |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST33232A | Pass |  |
| Seagate | ST32132A | Pass |  |
| Seagate | ST51270A | Pass |  |
| ### IBM | DHEA-34330 | Pass |  |
| WD | WDAC33100 | Pass |  |
| Quantum | TM17A012 | Pass |  |
| Quantum | TM1700A | Pass |  |
| Maxtor | 83240D4 | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | Pass |  |
| Philips | PCA123CD | Pass |  |
| VUGEO | 624A-004 | Pass |  |
| Panasonic | ### CR-585-B | Pass |  |
| Pioneer | DR-A24X | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 急速33.6 HIGH SPEED MODEM | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | Pass |  |
| ACER | 6511 | Pass |  |
| ### Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse SUPER | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

B. Booting Stability Testing

Test Condition: Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0 |  |
| KB Reset 20 Times | 20 | 20 | 0 |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0 |  |

C. Burn-In Stability Testing

Test Condition: Burn-In Under Temp. 60℃ 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

D. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | ###### Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0 |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0 |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | NEC 4217405 16MX2 60NS |
| --- | --- |
| VGA Card | OnBoard (C&T 65550) |
| HDD TYPE | Quantum(Mode4) |
| VGA Testing Mode | 640X480X16 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |
| --- | --- | --- | --- | --- |
| Testing Utility | AMD 5X86-133 |  |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |
| CPU | 447.06 |  |  | Mhz |
| FPU | 1139.58 |  |  | Mhz |
| VIDEO | 7504 |  |  | Chs/sec |
| Norton 8.0 SI | 288 |  |  | ID |
| Power Meter 2.02 | 41.8 |  |  | Mips |
| PcBench 9.0 CPUmark16 | 103.62 |  |  |  |
| WinStone 96 1.0 | 31.5 |  |  |  |
| WinBench 96 |  |  |  |  |
| Graphic WinMark 96 | 10.6 |  |  | MP/s |
| ### CPU mark 16 | 77 |  |  |  |
| CPU mark 32 | 71.8 |  |  |  |
| Coretest 3.02 |  |  |  |  |
| DTR (Buffered Read) | 1360 |  |  | KB/sec |
| Performance ID | 12.9 |  |  |  |
| WINTECH 1.2 | 83.68 |  |  |  |
| Note |  |  |  |  |

5. Power Consumption Data

System Information:

| ### DRAM Type & Capacity | Siemens HY85117405BJ-60 16MB*2 |
| --- | --- |
| VGA Card | Onboard C&T F65550 |
| DOC Type | M-Systems Md-2800-D08 |
| O.S. | DOS V 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=C:\Dos\Himem.Sys Dos=High Files=35 Buffers=40 | Prompt $P$G C:\Dos\Smartdrv.Exe /X |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Processor Type | Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # AMD 5X86-133 | 2.6 | 0.9 | Amp/Max |

Note :

        1.Power supply voltage : ＋5V.

        2.Normal Model Is measured while open system.

        3.Suspend Model all function off.

6. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -40℃ |  |

B. Temperature Test (BOOT UP)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -40℃ |  |

C. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.30V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.55V |  |

D. IDE Drive Capability

Test Equipment

Primary IDE Port:

### Master Device: W/D 22100

Slave Device: CD-ROM/VUGEO-624A-004

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |