SBC-492

QE Compatibility Test Report

Release Date: Aug. 11, 1999

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility ------------------------------------------------------------------------------------------------------ 3

Includes various OS and application software tests.

2. Hardware compatibility ---------------------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability ------------------------------------------------------------------------------------------------------------------------ 8

Includes burn-in and booting tests.

4. Performance ------------------------------------------------------------------------------------------------------------------ 9

Includes performance indexes using various performance test software programs.

5. Power Consumption Data ------------------------------------------------------------------------------------------------- 10

Includes performance indexes using various performance test software programs.

6. Margin ------------------------------------------------------------------------------------------------------------------------ 11

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

- Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2/V (Japan) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM OS/ 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.1 |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows 95 (English) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 (Japan) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Windows Nt4.0w/S (English) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Windows Nt4.0w/S (Japan) |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Norton Utility 8.0 |  | Pass |  |
| Norton Commander 5.0 |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Amidiag4.5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Video Adapter | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| COM Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  |  |  |  |  |  |  |  |  |  |
| Note : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| AMD | AM486 DX4-100 | Pass | On Board |
| AMD | Am5x86 -133 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| NEC | 4218165-60 8M EDO RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| SIEMENS | HYB5118165BSJ-60 8M EDO RAM | Pass |  |
| SAMSUNG | KM41C16000BK-6 64M FPM RAM | Pass |  |
| PANASONIC | MN414400BSJ-07 8M FPM RAM | Pass |  |
| PANASONIC | MN4117400BSJ-06 32M FPM RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164BJC-60 8M EDO RAM | Pass |  |
| MITSUBISH | M5M417400CJ-7 16M FPM RAM | Pass |  |
| ### MITSUBISHI | M5M418165CJ 8M EDO RAM | Pass |  |
| LGS | GM71C17403CJ6 16M EDO RAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| On Board | Trident TGUI9400-1 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### D-Link | DE 530 | Pass |  |
| D-Link | DEF 500TX | Pass |  |
| 3COM | FAST ETHER LINK | Pass |  |
| Realtek | RTL8139 | Pass |  |
| ### Intel | 82557 - Based | Pass |  |
| ### SVEC | AM79C970AKC | Pass |  |
| ### ISA Bus |  |  |  |
| Intel | Ether Express PRO/100B | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| ### AHA | 2940UW | Pass |  |
| ISA Bus |  |  |  |
| ### AHA | 1520K | Pass |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MPEG Card |  |  |  |
| LEMEL Magic | MPEG Card | Pass |  |
| Sound Card |  |  |  |
| ### Creative | SB 16 Value PnP | Pass |  |
| Creative | AWE 64 PnP | Pass |  |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA Card |  |  |  |
| AAEON | RDM-1010A | Pass |  |
| AAEON | RDM-1020N | Pass |  |
| AAEON | RDM-211A | Pass |  |
| 104 Module |  |  |  |
| AAEON | PCM-3820C (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3420 (SCSI) | Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCD-896 (SSD) | Pass |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST3123ON SCSI | Pass |  |
| Seagate | ST32155W/P SCSI | Pass |  |
| Seagate | ST51270A | Pass |  |
| Seagate | ST32132A | Pass | HDD has set to delay 5 Sec. |
| WD | WDAC22100 | Pass |  |
| Quantum | ST 3240 | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| VUGEO | 624A-004 | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 33.6 HIGH SPEED MODEM | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| ACER | 6311-TW | Pass |  |
| QTRONIX | QX-301H | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |
| Logitech | First Mouse | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| QAPLUS/FE 5.29 | Pass |  |

B. Burn-In Stability Testing

Test Condition: Burn-In Under -40℃~85℃ Burn-In 30 Hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAD 4.5 | Pass |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0﹪ | -40℃~85℃ |
| H/W Reset 1000 Times | 1000 | 1000 | 0﹪ | - 5℃~65℃ |

4. Performance Testing

SYSTEM INFORMATION:

| DRAM Type & Capacity | Siemens 32MB X2 |
| --- | --- |
| VGA Card | On Board (Trident) |
| HDD TYPE | Quantum ST3240 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone 96& WinBench 96) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=hing Files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AM486 DX4-100 |  | Am5X86 133M |  |  |  |  |  |  |  |  |  |  |  | Unit |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU |  | ### 333.01 |  | 444.02 |  |  |  |  |  |  |  |  |  |  |  | Mhz |
| FPU |  | 878.43 |  | 1085.35 |  |  |  |  |  |  |  |  |  |  |  | Mhz |
| VIDEO |  | 10685 |  | 10685 |  |  |  |  |  |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 215.0 |  | 286.7 |  |  |  |  |  |  |  |  |  |  |  | ID |  |
| Power Meter 2.02 | 30.9 |  | 40.6 |  |  |  |  |  |  |  |  |  |  |  | Mips |  |
| PcBench 96 CPUmark16 | 90.12 |  | 103.61 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WinStone 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 27.3 |  | 29.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WinBench 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Graphic WinMark | 4.05 |  | 8.63 |  |  |  |  |  |  |  |  |  |  |  | MP/s |  |
| ###### CPUmark 32 | 70.2 |  | 72.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 1504 |  | 1168 |  |  |  |  |  |  |  |  |  |  |  | KB/sec |  |
| Performance ID | 15.74 |  | 14.087 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power consumption(+5V in normal mode) | 1.80 |  | 2.4 |  |  |  |  |  |  |  |  |  |  |  | A |  |
| Note |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5. Power Consumption

System Information:

| DRAM Type & Capacity | HY5117400AJ –70 16M*2 |
| --- | --- |
| VGA Card | On Board (Trident) |
| DiskOnChip | MD2200-D08 |
| O.S. | DOS Mode |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

| Mode Type | CPU Type |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Am5X86 133M |  |  |  |  |  |  |
| Normal Mode(+5V) | 2.4 |  |  |  |  | amp/max |  |
| Suspend Mode(+5V) | 0.9 |  |  |  |  | amp/max |  |

Note:

    1.Suspend Model all function off.

2.Power supply voltage: +5V

6. Margin Testing

A. Temperature Test (Software: AMIDIAG4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -40℃ |  |

B. Temperature Test (Boot Up)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 85℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -40℃ |  |

C. DC Voltage Test (Software: Qaplus/fe 5.29)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.30 V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.46 V |  |

D. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device: HDD Seagate ST51270A

Slave Device: CD-ROM Vugeo 624A-004

Secondary IDE Port:

Master Device: HDD Quantum ST3240

Slave Device: CD-ROM Vugeo 624A-004

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | 75CM |  |