AAEON

ISO-9001/ISO-14001 Certified

Industrial Automation PCs

SBC-557

QE Compatibility Test Report

Release Date: May. 12, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility -------------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests .

2. Hardware compatibility ------------------------------------------------------------------------------------------------ 5

Includes CPU , DRAM , adapter card and HDD compatibility tests.

3. Stability -------------------------------------------------------------------------------------------------------------------- 8

Includes burn-in and booting tests .

4. Performance -------------------------------------------------------------------------------------------------------------- 9

Includes performance indexes using various performance test software programs

5. Power consumption data------------------------------------------------------------------------------------------------11

Includes +5V current rate .

6. Margin -------------------------------------------------------------------------------------------------------------------- 12

Includes temperature , DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | KB Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LINUX slakware4.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | KB Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 95(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0/ W(E) |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result | Result | Note |
| --- | --- | --- |
| NORTON UTILITY 8.0 | Pass |  |
| NORTON COMMANDER 5.0 | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qaplus5.29 |  |  | AMIDIAG5.11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Resul |  | Note |  |  |  |  |  |  |
| ##### MAIN COMPONENT | Fail |  | Fail |  |  |  |  |  |  |  |  |
| MEMORY | Pass |  | Pass |  |  |  |  |  |  |  |  |
| ##### VIDEO ADAPTER | Pass |  | Pass |  |  |  |  |  |  |  |  |
| FLOPPY DRIVE | Pass |  | Pass |  |  |  |  |  |  |  |  |
| HARD DRIVE | Pass |  | Pass |  |  |  |  |  |  |  |  |
| KEYBOARD | Pass |  | Pass |  |  |  |  |  |  |  |  |
| COM PORT | Pass |  | Pass |  |  |  |  |  |  |  |  |
| LPT PORT | Pass |  | Pass |  |  |  |  |  |  |  |  |
| Plug & play | Pass |  | Pass |  |  |  |  |  |  |  |  |
| Note :QAPLUS5.29 interval timer fail AMIDIAG5.11 RTC periodic interrupt not coming AMIDIAG4.5 Calibrate timer fail |  |  |  |  |  |  |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | ### Pentium MMX 200 | #### Pass |  |
| Intel | ### Pentium MMX 233 | Pass |  |
| Intel | ### Pentium Tillamook 266 | Pass |  |
| AMD | K6-233 | Fail | Run winstone99 hi-end fail |
| AMD | K6-266 | #### Pass |  |
| AMD | ### K6-300 | Fail | Run winstone99 hi-end fail |
| AMD | K6/2 300 | Pass | 100x3 |
| AMD | K6/2 400 | Pass | 100x4 |
| AMD | K6/3 450 | Pass | 100x4.5 |
| Cyrix | M2 PR200 | #### Fail | Run winstone99 hi-end fail |
| Cyrix | M2 PR233 | Pass |  |
| Cyrix | ### M2 PR300 | Pass |  |
| IDT | Winchip 225 | Pass |  |
| IDT | Winchip 240 | Fail | Install95,98,NT fail |
| Rise | mP266 | Fail | install NT fail |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MT | 48LC2M8A1TG-10S 32MB | Pass |  |
| NEC | D4564841G5-A10-9JF 64MB | Pass |  |
| NEC | D4564841G5-A10-9JF 128MB | Pass |  |
| LGS | ### GM72V661641CT7J 64MB | Pass |  |
| HYNDAI | HY57V658020TC-10 128MB | Pass |  |
| HYNDAI | HY57V168010D TC-10S 32MB | Pass |  |
| NEC | D4564841G5-A80-9JF 64MB | Pass |  |
| NEC | D4516821AG5-A10-9NF 32MB | Pass |  |
| NEC | D4564841G5-A80-9JF 128MB | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On_board | C&T69000 | Pass |  |
| ISA Bus |  |  |  |
| Tseng | ET4000S/W32 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### ISA Bus |  |  |  |
| Intel | Ether Express TM PRO/100 | Pass |  |
| 3com | Etherlink III ISA 3c509b | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### ISA Bus |  |  |  |
| AHA | 1520B | Pass |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Sound Card(ISA Bus) |  |  |  |
| Creative | ### AWE64 | Pass |  |
| Creative | SB16 | Pass |  |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| AAEON | ### PCM-3820-2 | Pass | SSD 2M |
| AAEON | PCM-3820-4 | Pass | SSD 4M |
| AAEON | PCM-3110 | Pass | PCMCIA |
| AAEON | PCM-3115B | Pass | PCMCIA |
| AAEON | PCM-3200 | Pass | Sound |
| AAEON | PCM-3600 | Pass | Fax/modem |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Hitachi | DK226A-21U | Pass |  |
| Seagate | ST34520A | Pass |  |
| Fujitsu | ### MPC3032AT | Pass |  |
| Mxtor | 90648D3 | Pass |  |
| Quantiun | Bigfoot TX4.0 | Pass |  |
| Quantium | Fireball TM1700A | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| TEAC | ### 532E | Pass |  |
| SAMPO | CDM-T631 | Pass |  |
| NEC | 282 | Pass |  |
| FAX/Modem |  |  |  |
| ### LEMEL | 33.6 FAX/MODEM | Pass |  |
| Rockware | Cyber lander 2000 56K | Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand |  | Device Model | Result |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- |
| Keyboard |  |  |  |  |  |  |
| BTC |  | 5140 | Pass |  |  |  |
| ACER |  | 6511-TW1C/B | Pass |  |  |  |
| USB Keyboard |  |  |  |  |  |  |
| BTC |  | 7932 | Pass |  |  |  |
| Mouse |  |  |  |  |  |  |
| Microsoft |  | ### Serial Mouse | Pass |  |  |  |
| IBM |  | PS/2 Mouse | Pass |  |  |  |
| RS485 |  |  |  |  |  |  |
| Advantech |  | ADAM 4011 | Pass |  | COM2/RS-485 |  |
| USB Mouse |  |  |  |  |  |  |
| DEXIN |  | A2U800A | Pass |  |  |  |
| PRINT |  |  |  |  |  |  |
| EPSON |  | ### Style color 740 | Pass |  | USB |  |
| PC CAMERA |  |  |  |  |  |  |
| ### Vcam | CU-98 |  |  | Pass |  | USB |
| Tekcam | P35U |  |  | Pass |  | USB |
| ### ZIP |  |  |  |  |  |  |
| IOMEGA |  | ZIP 100 | Pass |  | Parallel port |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition : Burn-In Under Temp.-10℃〜70℃( (IEC-68-2-61)

| Test Software | Result | Note |
| --- | --- | --- |
| ### Qaplus529 | Pass | K6/2 300 |

B. Booting Stability Testing

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-30)

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1433 | 1433 | 0% | K6/2 300 |

C. Booting Stability Testing

Test Condition : Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1433 | 1433 | 0% | K6/2 300 |
| KB Reset 20 Times | 20 | 20 | 0% |  |
| H/W Reset 1000 Times | 1868 | 1868 | 0% | K6/2 400 |

4. Performance Testing

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A10-9JF 64MB |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| HDD TYPE | ### Quantum bigfootTX4.0 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS 6.22 , Windows 98 and Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high,umb Files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  |  | CPU Configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Pentium MMX-200 |  | Pentium MMX-233 | Tilamook 266 | AMD K6-233 | AMD K6-266 | AMD K6-300 | AMD K6/2 300 | AMD K6/2 400 |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |
| CPU | 1131.76 |  | 1530.39 | 1749.03 | 2399.12 | 2741.86 | 3084.56 | 3084.56 | 4111.11 | Mhz |
| FPU | 4010.86 |  | 4679.23 | 5247.79 | 3829.08 | 4376.09 | 4923.15 | 4923.15 | 6626.51 | Mhz |
| VIDEO | 11843 |  | 11843 | 11843 | 10922 | 10922 | 10922 | 10922 | 20915 | Chs/sec |
| Norton 8.0 SI | 680.6 |  | 794 | 907.4 | 1388.7 | 1587.1 | N/A | N/A | 484.7 | ID |
| Power Meter 2.02 | 105.3 |  | 123.6 | 135.4 | 167.3 | 189.6 | 218.8 | 218.8 | 284.4 | Mips |
| WinStone 99 |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 13.6 |  | 13.7 | 14.9 | 15.8 | 15.4 | 16.6 | 17.5 | 19.2 |  |
| High-End Winstone | 11.8 |  | 12.1 | 13.6 | Fail | 13.4 | Fail | 15.5 | 17.7 |  |
| WinBench 99 |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 168 |  | 185 | 201 | 194 | 214 | 227 | 248 | 327 | MP/s |
| ###### Business Graphic WinMark | 60.5 |  | 65.1 | 69.8 | 67.3 | 73.9 | 76.9 | 84.4 | 104 |  |
| CPUmark 32 | 433 |  | 470 | 484 | 572 | 620 | 600 | 785 | 803 |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 9888 |  | 9888 | 9856 | 9744 | 9744 | 9744 | 9824 | 9680 | KB/sec |
| Performance ID | 63.43 |  | 63.47 | 63.21 | 62.70 | 62.74 | 62.70 | 63.17 | 62.56 |  |
| ### Notes |  |  |  |  |  |  |  |  |  |  |

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A10-9JF 64MB |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| HDD TYPE | Quantum bigfootTX4.0 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS 6.22 , Windows 98 and Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Cyrix M2 PR 200 |  | Cyrix M2 PR 233 | Cyrix M2 PR 300 |  | Winchip 225 |  | Winchip 240 |  | AMD K6-3 450 |  | AMD K6-2 450 |  |  | AMD K6-2 500 |  |  |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### CPU | 2072.93 |  | 2323.71 | 2901 |  | 1068.82 |  | 1141.95 |  | 4625.49 |  | 4625.49 |  |  |  | 5139.43 | Mhz |  |
| FPU | 2396.43 |  | 2665.90 | 3353.88 |  | 2192.82 |  | 2342.85 |  | 7454.71 |  | 7393.38 |  |  |  | 8214.87 | Mhz |  |
| VIDEO | 17873 |  | 17554 | 18204 |  | 11170 |  | 11170 |  | 20915 |  | 10922 |  |  |  | 10922 | Chs/sec |  |
| Norton 8.0 SI | 1133.9 |  | 1271.4 | 1235.1 |  | 485.7 |  | 518.9 |  | N/A |  | 785.9 |  |  |  | N/A | ID |  |
| Power Meter 2.02 | 113.8 |  | 129.3 | 167.3 |  | 94.8 |  | 101.6 |  | 355.5 |  | 355.5 |  |  |  | 406.3 | Mips |  |
| WinStone 99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | Fail |  | 8.12 | 8.78 |  | 8.25 |  | Fail |  | 23.1 |  | 21.7 |  |  |  | 22.6 |  |  |
| High-End Winstone | Fail |  | 8.05 | 9.1 |  | 8.66 |  | Fail |  | 20.8 |  | 20.9 |  |  |  | 21.9 |  |  |
| WinBench 99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 197 |  | 202 | 248 |  | 198 |  | 203 |  | 410 |  | 301 |  |  |  | 324 | MP/s |  |
| ###### Business Graphic WinMark | 67.6 |  | 68.6 | 81.6 |  | 71.7 |  | 72.6 |  | 129 |  | 98.9 |  |  |  | 103 |  |  |
| CPUmark 32 | 449 |  | 474 | 558 |  | 474 |  | 463 |  | 1340 |  | 914 |  |  |  | 965 |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10064 | 8992 |  |  | 10080 |  | 8992 |  | 9056 |  | 9664 |  | 10288 | 10288 |  |  |  | KB/sec |
| Performance ID | 64.64 | 58.22 |  |  | 64.74 |  | 58.22 |  | 58.60 |  | 62.43 |  | 69.44 | 69.56 |  |  |  |  |
| ### Notes : CPU type of AMD K6-2 450 & / K6-2 500 HDD is IBM DPTA-372050 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | KOREA HY57V168010D 32MB |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| FLOPPY | TEAC FD-235HF |
| ### VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Process Type | Normal Mode | # Doze Mode | Standby Mode | Suspend Mode | Unit |
| --- | --- | --- | --- | --- | --- |
| # PENTIUM MMX-200 | 4 | 3.2 | 2 | 1.6 | Amp/Max |
| PENTIUM MMX-233 | 4.4 | 3.4 | 2.2 | 1.6 | Amp/Max |
| PENTIUM Tillamook 266 | 2.8 | 2.4 | 1.7 | 1.4 | Amp/Max |
| AMD K6-233 | 8.4 | 5.6 | 2.8 | 1.4 | Amp/Max |
| AMD K6-266 | 3.4 | 2.6 | 1.8 | 1.4 | Amp/Max |
| AMD K6-300 | 3.2 | 2.6 | 1.7 | 1.4 | Amp/Max |
| AMD K6/2 300 | 4.3 | 3.2 | 2 | 1.4 | Amp/Max |
| AMD K6/2 400 | 5.2 | 3.8 | 2.2 | 1.4 | Amp/Max |
| AMD K6/2 450 | 5.6 | 4.1 | 2.5 | 1.6 | Amp/Max |
| AMD K6/2 500 | 6.4 | 4.5 | 2.8 | 1.7 | Amp/Max |
| AMD K6/3 450 | 6.4 | 4.2 | 2.8 | 1.8 | Amp/Max |
| Cyrix M2 PR-200 | 6.4 | 4.4 | 2.4 | 1.4 | Amp/Max |
| Cyrix M2 PR-233 | 6.8 | 4.8 | 2.6 | 1.4 | Amp/Max |
| Cyrix M2 PR-300 | 8 | 5.8 | 2.8 | 1.2 | Amp/Max |
| IDT Winchip 225 | 3.2 | 2.8 | 2 | 1.8 | Amp/Max |
| IDT Winchip 240 | 3.5 | 3 | 2.2 | 1.8 | Amp/Max |

Note :

 1.Power supply voltage : ＋5V

   2.Suspend Model all function off

6. Margin Testing

A. Temperature Test (Software: AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -10℃ |  |

B. DC Voltage Test (Software: AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | Pass |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | Pass |  |

C. IDE Drive Capability

Test Equipment

Primary IDE Port:

Master Device: HD Quantum bigfootTX4.0

Slave Device: CD-ROM NEC CDR-1901A

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | N/A |  |