SBC-590

###### QE Compatibility Test Report

Release Date: Aug. 20, 1999

__________     __________   __________   __________

# Issue Stamp             QA Manager          QE Manager         Test Engineer

Contents

- 1 Software compatibility ------------------------------------------------------------------------------------ 3

Includes various OS and application software tests.

- 2. Hardware compatibility ---------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

- 3. Stability ------------------------------------------------------------------------------------------------------ 8

Includes burn-in and booting tests.

- 4. Performance ------------------------------------------------------------------------------------------------ 9

Includes performance indexes using various performance test software programs.

- 5. Power consumption data----------------------------------------------------------------------------------10

Includes +5V current rate.

- 6. Margin ------------------------------------------------------------------------------------------------------- 11

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

- 1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | Keyboard Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| Ms-Dos 6.22 | ## Pass |  | ## Pass |  |  | ## Pass |  |  | ## Pass |  |  | ## Pass |  |  |

B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.1 | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  |
| Windows 95 | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  |
| Windows NT 4.0 W/S | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | N/A |  |
| Windows NT 4.0 SRV | ## Pass |  | ## Pass |  | ## Pass |  | ## N/A |  | N/A |  | N/A |  |
| OS/2 Warp 4.0 | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | ## Pass |  | N/A |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | ## Pass |  | ## Pass |  |
| Ms Word 6.0c | ## Pass |  | ## Pass |  |
| Wordperfect 6.1 | ## Pass |  | ## Pass |  |
| Ms Works 3.0b | ## Pass |  | ## Pass |  |
| Lotus 1-2-3 R5 | ## Pass |  | ## Pass |  |
| Ms Excel 5.0c | ## Pass |  | ## Pass |  |
| Quattro Pro 6.01 | ## Pass |  | ## Pass |  |
| Pagemaker 5.0a | ## Pass |  | ## Pass |  |
| Coreldraw 5.0e2 | ## Pass |  | ## Pass |  |
| Powerpoint 4.0c | ## Pass |  | ## Pass |  |
| Dbase 5.0 | ## Pass |  | ## Pass |  |
| Paradox 5.0 | ## Pass |  | ## Pass |  |
| Access 2.0 | ## Pass |  | ## Pass |  |

- D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| ET 3.53 | 16 * 15 Font Mode | ## Pass |  |  |
|  | 24 * 24 Font Mode | ## Pass |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | ## Pass |  |  |
|  | 24 * 24 Font Mode | ## Pass |  |  |

E. Diagnostics Software Testing

| Test Item / Software | QAFE5.29 |  | AMIDIAG4.5/5.1 |  |  |  | CHECKIT3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note |  | Result | Note | Result |  | Note |  |  |  |
| Main Component | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| Memory | ## Pass |  |  | ## Pass |  | N/A |  |  |  |  |  |
| Video Adapter | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| Floppy Drive | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| Hard Drive | N/A |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| Keyboard | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| COM Port | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |
| LPT Port | ## Pass |  |  | ## Pass |  | ## Pass |  |  |  |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

- 586 CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| AMD | K5-100 | #### Pass |  |
| INTEL | PENTIUM-133 | Pass |  |
| INTEL | PENTIUM-166 | Pass |  |
| INTEL | PENTIUM-200 | Pass |  |

- B. Component Compatibility- DRAM SIMM Module

- DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | # Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| VANGUARD | VG264400BJ-6 8M | Pass |  |
| NEC | 424100-70 16M | Pass |  |
| NEC | 424400-70L 8M/NEC 421000-7PR | Pass |  |
| NEC | 424400-70 8M(*4)EDP RAM | Pass |  |
| TOSHIBA | TC514256AJJ-70 1M | Pass |  |

- C. Adapter Compatibility- VGA

- VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| TopView | ET 4000 W32P | Pass |  |
| TopView | S3 86801 | Pass |  |
| On_board |  | Pass |  |
| ISA BUS |  |  |  |
| TopView | ET 4000 ISA | Pass |  |

- D. Adapter Compatibility- LAN

- LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA |  |  |  |
| 3COM | 3C509 | # Pass |  |
| Novell | NE2000 | Pass |  |

- E. Adapter Compatibility- SCSI CARD

- SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA BUS |  |  |  |
| AHA | 1542CF | # Pass |  |

F. Adapter Compatibility- Industry Control Card

- Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| 104 Module |  |  |  |
| AAEON | PCM-3820 rev. C (SSD) | #### Pass |  |
| AAEON | PCM-3820C (SSD) | Pass |  |
| AAEON | PCM-3110 (PCMCIA MASTER) | Pass |  |
| AAEON | PCM-3200(SOUND) | # Pass |  |
| AAEON | PCM-3600(FAX/MODEM) | Pass |  |
| AAEON | PCM-3810 | Pass |  |
| AAEON | PCM-3115(PCMCIA 2 SLOTS) | Pass |  |

G. Peripheral Compatibility-HDD Device

- HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Maxtor | 7850AV 850M | # Pass |  |
| Seagate | ST31276R | Pass |  |
| CONNER | CFS425A 425M | Pass |  |
| Seagate | ST32123A | ### Pass |  |
| Quantum | 850AT | Pass |  |

H. Peripheral Compatibility- Storage Device

- Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Philips | PCA123CD | ### Pass |  |
| Acer | 8X SPEED | # Pass |  |
| Tape Streamer |  |  |  |
| HP | LaserJet III | ##### Pass |  |

- I. Peripheral Compatibility- Input Device

- Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | # Pass |  |
| ACER | 6511 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | #### Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | MouseMan (PS/2,Serial) | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

- Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | # Pass |  |

B. Booting Stability Testing

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0% |  |
| KB Reset 20 Times | 20 | 20 | 0% |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0% |  |

C. Continuous Burn-In Booting Stability Testing

- Test Condition: Burn-In Under Temp. 60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 999 | 0.1% |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0% |  |

4. Performance Testing

- System Information:

| DRAM Type & Capacity | Panassonic 32MBX2 |
| --- | --- |
| VGA Card | OnBoard (ALG 2032) |
| HDD Type | Segate ST51270A(Mode4) |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

- Testing Software

|  | CPU Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AMD K5-100 | INTEL P-133 | ### INTEL P-166 | INTEL P-200 |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |
| CPU | 896.28 | 770.01 | 962.44 | 1155.24 |  | Mhz |
| FPU | 1141.26 | 2584.86 | 3230.58 | 3878.06 |  | Mhz |
| VIDEO | 5748 | 8936 | 8936 | 8936 |  | Chs/sec |
| Norton 8.0 SI | 3963 | 422.8 | 528.4 | 634.2 |  | ID |
| Power Meter 2.02 | 50.8 | 63.2 | 74.8 | 81.3 |  | Mips |
| PcBench 9.0 CPUmark16 | 165.25 | 180.53 | 123.87 | 355.85 |  |  |
| WinStone 96 1.0 | 37.3 | 24.4 | 42.3 | 43.3 |  |  |
| WinBench 96 |  |  |  |  |  |  |
| Graphic WinMark 96 | 4.67 | 4.11 | 5.92 | 6.12 |  | MP/s |
| CPUmark 16 | 1.37 | 138 | 197 | 221 |  |  |
| CPUmark 32 | 1.39 | 130 | 171 | 183 |  |  |
| CORTST3.0 |  |  |  |  |  |  |
| DTR (Buffered Read) | 9216 | 6000 | 11472 | 14848 |  | KB/sec |
| Performance ID | 60.19 | 39.63 | 73.47 | 93.66 |  |  |
| WINTECH 1.2 | 11.83 | 7.37 | 6.56 | 15.22 |  |  |
| Note |  |  |  |  |  |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | Panasonic MN4117405CSJ-06 EDO RAM 32MBx2 |
| --- | --- |
| VGA Card | OnBoard C&T65550 |
| Floppy Type | Teac FD-235HF |
| VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Processor Type | Normal Mode | Suspend Mode | Unit |
| --- | --- | --- | --- |
| # AMD K5 100 | 4 | 1 | Amp/Max |
| Intel Pentium -133 | 3.5 | 1.2 | Amp/Max |
| Intel Pentium -166 | 3.8 | 1.2 | Amp/Max |
| Intel Pentium -200 | 4.4 | 1.4 | Amp/Max |

Note:

 1.Power supply voltage : ＋5V

   2.Suspend Model all function off

6. Margin Testing

- A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High temperature Margin (Burn-In 2 Hours) | 70℃ |  |
| Low temperature Margin (Burn-In 2 Hours) | -35℃ |  |

- B. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | 5.6V |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | 4.4V |  |

C. IDE Drive Capability

- Test Equipment:

- Primary IDE Port:

Master Device: Segate ST51270A

Slave Device: CD-ROM SAMPO-36X

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | ## 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | 75CM |  |