SBC-597

QE Compatibility Test Report

Release Date: May. 12, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility --------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------- 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability --------------------------------------------------------------------------------------------------------- 8

Includes burn-in and booting tests.

4. Performance --------------------------------------------------------------------------------------------------- 9

Includes performance indexes using various performance test software programs

5. Power consumption data--------------------------------------------------------------------------------------11

Includes +5V current rate.

6. Margin -----------------------------------------------------------------------------------------------------------12

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | KB Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MS-DOS 6.2V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LINUX Slakware4.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B Microsoft Windows & IBM O/S 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | KB Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0/ W |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |
| Windows 95(J) |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows NT 4.0 W(J) |  |  |  |  |  |  |  |  |  |  |  |  |

(J): Japanese

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDraw 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| Norton Commander 5.0 |  |  | Pass |  |

E. Diagnostics Software Testing

| Test Item / Software | Qaplus5.29 |  |  | Amidiag4.5 |  |  | Amidiag5.11 |  |  | Winplus6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note |  |  |  |
| ##### Main Component | Fail |  | Fail |  |  | Fail |  |  | Pass |  |  |  |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| ##### Video Adapter | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| Hard Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| Keyboard | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| COM Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| LPT Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| Plug & play | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  |  |  |  |
| Note :QAPLUS5.29 interval timer fail AMIDIAG4.5 Calibrate timer fail AMIDIAG5.11 RTC periodic interrupt not coming |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

F. N.O.S. & Application Software Compatibility (On_Board Network)

System Information:

| DRAM Capacity & BRAND(SIMM Qty) | Brand: LGX PC100 □4□8□16□32■64□128M Qty:■1□2 □3 □4 |
| --- | --- |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver:2.0 |
| KB BIOS Version | □AMI- □VIA- ■Internal |
| BACKPLANE | □05P2 □08P3 □14P3 □ 03 □08 □14 ■N/A |
| O.S. Environment | □DOS5.0 □DOS6.0 ■DOS6.22 |
| VGA Card & BIOS | ■OnBoard: C&T69000 □ □BIOS: |
| POWER SUPPLY | □MPU-250W■SSG-250G □SSH-350G □ST-250AF |
| IDE HDD TYPE | □Max □Sea □Con □Qua ■Fuj □IBM : MPC3032AT |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| ### LAN CARD | ■OnBoard: RTL-8139 BIOS: |
| ### CPU Type | K6-2 300MHZ |
| CD-ROM TYPE | □NEC ■PANASONIC □PIONEER □CREATIVE : |
| Driver or Utility | ■VGA □HDD ■Others : RTL-8139 |

Test Result:

| ### SUBJECT | Network Operation System Function Testing |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  | Pass | Fail | Note |
| NetWare 4.11 |  | ˇ |  |  |
| Windows NT 4.0 |  | ˇ |  |  |
| SUBJECT | Workstation Application Function Testing |  |  |  |
| Windows 95 for IPX/SPX |  | ˇ |  |  |
| Windows 95 for NetBEUI |  | ˇ |  |  |
| Windows 95 for TCP/IP |  | ˇ |  |  |
| Windows 98 for IPX/SPX |  | ˇ |  |  |
| Windows 98 for NetBEUI |  | ˇ |  |  |
| ### Windows 98 for TCP/IP |  | ˇ |  |  |
| Windows NT 4.0 for IPX/SPX |  | ˇ |  |  |
| Windows NT 4.0 for NetBEUI |  | ˇ |  |  |
| Windows NT 4.0 for TCP/IP |  | ˇ |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | ### Pentium MMX 200 | #### Pass |  |
| Intel | ### Pentium MMX 233 | Pass |  |
| Intel | ### Tillamook 266 | Pass |  |
| AMD | K6-233 | Pass |  |
| AMD | K6-266 | #### Pass |  |
| AMD | K6-300 | Pass |  |
| AMD | K6/2 300 | Pass | 100x3 |
| AMD | ### K6/2 400 | Pass | 100x4,Install 95 protect error, but98 NT pass |
| Cyrix | M2 PR200 | #### Pass |  |
| Cyrix | M2 PR233 | Pass |  |
| Cyrix | M2 PR300 | Pass |  |
| IDT | Winchip 225 | Pass |  |
| IDT | Winchip 240 | Pass |  |
| Rise | MP6-266 | Fail | Install 95,98,NT fail |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MT | 48LC2M8A1TG-10S 32MB | Pass |  |
| NEC | D4564841G5-A10-9JF 64MB | Pass |  |
| NEC | D4564841G5-A10-9JF 32MB | Pass |  |
| SIEMENS | HYB39S16800AT-10 32MB | Pass |  |
| KOREA | HY57V168010D TC-10S 32MB | Pass |  |
| LGS | GM72V661641CT7J 64MB | Pass |  |
| KOREA | HY57V658020TC-10 128MB | Pass |  |
| HITACHI | ### HM5216805TT10H 16MB | Pass |  |
| KINGMAX | KSV884T4A0-08 8C0032 64MB | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ### PCI Bus |  |  |  |
| On_board | C&T69000 | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| On-board | RTL8139 | Pass |  |
| D-LINK | 530-TX | Pass |  |
| 3COM | 905B | Pass |  |
| INTEL | ### SB82558B | Pass |  |
| ISA Bus |  |  |  |
| Intel | Ether Express TM PRO/100 | Pass |  |

E. On_Board Network Hardware (physical) Compatibility

System Information:

| DRAM Capacity & BRAND(SIMM Qty) | Brand:LGX PC100 □4□8□16□32■64□32M __ Qty: ■1 □2 □3 □4 |
| --- | --- |
| SYSTEM BIOS Ver. | ■AWARD □AMI Ver: |
| KB BIOS Version | □AMI- □VIA- ■Internal |
| BACKPLANE PCA-61xx | □05P2 □08P3 ■14P3 □ 03 □08 □14 □ N/A |
| O.S. Environment | □DOS5.0 □DOS6.0 ■DOS6.22 |
| VGA Card & BIOS | ■OnBoard □ □BIOS: |
| POWER SUPPLY | ■MPU-250W□SSG-250G □SSH-350G □ST-250AF |
| IDE HDD TYPE | □Max □Sea □Con □Qua ■Fuj □IBM :MPC3032AT |
| SCSI HDD TYPE | □Max □Sea □Con □Qua □Fuj □IBM: |
| LAN CARD | ■OnBoard: RTL8139 BIOS: |
| CD-ROM TYPE | □NEC □PANASONIC ■TEAC □CREATIVE |
| Driver or Utility | □VGA □HDD ■Others : RTL-8139 |

Test Result:

| Test Item |  | Pass | Fail | Note |
| --- | --- | --- | --- | --- |
| SUBJECT | Network Hardware Function Testing |  |  |  |
| Transceiver |  | ˇ |  |  |
| Receiver |  | ˇ |  |  |
| Auto-negotiation |  | ˇ |  |  |
| ### 100M Base-TX full duplex |  | ˇ |  |  |
| 100M Base-TX half duplex |  | ˇ |  |  |
| 10M Base-T full duplex |  | ˇ |  |  |
| 10M Base-T half duplex |  | ˇ |  |  |

F. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI Bus |  |  |  |
| AHA | 2940UW | #### Pass |  |
| ISA Bus |  |  |  |
| AHA | ### 1520B | Pass |  |

G. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Sound Card(ISA Bus) |  |  |  |
| Creative | AWE64 | Pass |  |
| Creative | SB16 | Pass |  |

I. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| HITACHI | DK226A-21U | Pass |  |
| Seagate | ST34520A | Pass |  |
| FUJITSU | MPC3032AT | Pass |  |
| Mxtor | 90648D3 | Pass |  |

J. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| TEAC | 532E | Pass |  |
| SAMPO | CDM-T631 | Pass |  |
| ### FAX/Modem |  |  |  |
| ### LEMEL | 33.6 FAX/MODEM | Pass |  |

K. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | Pass |  |
| USB Keyboard |  |  |  |
| BTC | 7932 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| RS485 |  |  |  |
| Adventech | ADAM 4011 | Pass |  |
| USB Mouse |  |  |  |
| DEXIN | A2U800A | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition : Burn-In Under Temp.-10℃〜70℃( (IEC-68-2-61)

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | Pass |  |

B. Booting Stability Testing

Test Condition : Burn-In Under Temp. -10℃〜70℃(IEC-68-2-30)

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0% |  |

C. Booting Stability Testing

Test Condition : Room Temp.

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0% |  |
| KB Reset 20 Times | 20 | 20 | 0% |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0% |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | KOREA HY57V168010D 32M |
| --- | --- |
| ### VGA Card | OnBoard C&T 69000 |
| HDD TYPE | FUJITSU MPC3032AT |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench9) |
| O.S. | DOS 6.22 , Windows 95 and Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high,umb Files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  |  | CPU Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Pentium MMX-200 |  | Pentium MMX-233 | Tillamook 266 | AMD K6-233 | AMD K6-266 | AMD K6-300 | AMD K6-2 300 | AMD K6-2 400 |  |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |  |  |
| CPU | 1311.77 |  | 1530.40 | 1749.01 | 2399.10 | 2741.84 | 3084.56 | 3084.59 |  | 4111.55 | Mhz |
| FPU | 4010.86 |  | 4679.38 | 5347.74 | 3829.04 | 4376.09 | 4923.15 | 4923.60 |  | 6626.45 | Mhz |
| VIDEO | 14043 |  | 14043 | 14043 | 12288 | 12288 | 12288 | 33 |  | 12288 | Chs/sec |
| Norton 8.0 SI | 680.6 |  | 794 | 907.4 | 1388.7 | 1587.1 | 5062.3 | N/A |  | 484.7 | ID |
| Power Meter 2.02 | 105.3 |  | 123.6 | 135.4 | 167.3 | 189.6 | 218.8 | 218.8 |  | 284.4 | Mips |
| WinStone 98 |  |  |  |  |  |  |  |  |  |  |  |
| Business Winstone | 17.7 |  | 18.6 | 16.4 | 19.8 | 18.8 | 19.2 | 20.5 |  | 23.7 |  |
| High-End Winstone | 17.6 |  | 18.7 | 17.1 | 19 | 20 | 17.8 | 18.8 |  | 22.7 |  |
| WinBench 98 |  |  |  |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 112 |  | 124 | 104 | 125 | 136 | 142 | 158 |  | 186 | MP/s |
| ###### Business Graphic WinMark | 86.7 |  | 94.6 | 76.8 | 99.7 | 108 | 111 | 126 |  | 144 |  |
| CPUmark 32 | 420 |  | 459 | 381 | 555 | 624 | 617 | 709 |  | 879 |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10032 |  | 10016 | 10032 | 9744 | 9792 | 9808 | 9856 |  | 9952 | KB/sec |
| Performance ID | 64.41 |  | 64.31 | 66.18 | 62.74 | 62.98 | 63.08 | 63.36 |  | 65.70 |  |

System Information:

| DRAM Type & Capacity | KOREA HY57V168101D 32M |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| HDD TYPE | FUJITSU MPC3032AT |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS 6.22 , Windows 95 and Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high,umb Files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  |  | CPU Configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Cyrix M2 PR 200 |  | Cyrix M2 PR 233 | Cyrix M2 PR 300 | IDT WinChip 225 | IDT WinChip 240 | AMD K6/3 450 |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |
| ### CPU | 2072.81 |  | 2323.71 | 2900.92 | 1068.82 | 1141.96 | 4625.44 | Mhz |
| FPU | 2398.89 |  | 2669.09 | 3343.54 | 2192.84 | 2342.85 | 7454.79 | Mhz |
| VIDEO | 192.75 |  | 192.75 | 192.75 | 13107 | 13284 | 12288 | Chs/sec |
| Norton 8.0 SI | 1133.9 |  | 1271.4 | 1235.1 | 485.7 | 518.9 | N/A | ID |
| Power Meter 2.02 | 113.8 |  | 129.3 | 167.3 | 94.8 | 101.6 | 355.5 | Mips |
| WinStone 98 |  |  |  |  |  |  |  |  |
| Business Winstone | 17.9 |  | 18.1 | 18.8 | 20.5 | 19.4 | 21.3 |  |
| High-End Winstone | 17.3 |  | 17.7 | 17.7 | 22 | 20.4 | Fail |  |
| WinBench 98 |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 128 |  | 127 | 161 | 130 | 134 | 238 | MP/s |
| ###### Business Graphic WinMark | 99.7 |  | 100 | 123 | 105 | 106 | 190 |  |
| CPUmark 32 | 438 |  | 445 | 527 | 459 | 466 | 1480 |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10080 |  | 8992 | 10064 | 9088 | 9104 | 9936 | KB/sec |
| Performance ID | 64.74 |  | 58.26 | 64.6 | 60.56 | 60.66 | 65.61 |  |

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A10-9JF 64M |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| HDD TYPE | DPTA-372050 |
| VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS 6.22 , Windows 98 and Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high,umb Files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Testing Software

|  |  | CPU Configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AMD K6-2 450 |  | AMD K6-2 500 | AMD K6-2 550 |  |  |  |  |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |
| ### CPU | 4625.49 |  | 5139.43 | 5653.36 |  |  |  | Mhz |
| FPU | 7393.43 |  | 8214.95 | 9036.41 |  |  |  | Mhz |
| VIDEO | 11988 |  | 11988 | 12443 |  |  |  | Chs/sec |
| Norton 8.0 SI | 781.7 |  | 1081.9 | 1381.1 |  |  |  | ID |
| Power Meter 2.02 | 355.5 |  | 406.3 | 406.3 |  |  |  | Mips |
| WinStone 98 |  |  |  |  |  |  |  |  |
| Business Winstone | 21.4 |  | 22.2 | 23.1 |  |  |  |  |
| High-End Winstone | 20.8 |  | 21.5 | 22.5 |  |  |  |  |
| WinBench 98 |  |  |  |  |  |  |  |  |
| ###### High-End Graphic WinMark | 314 |  | 335 | 345 |  |  |  | MP/s |
| ###### Business Graphic WinMark | 94.2 |  | 97.3 | 100 |  |  |  |  |
| CPUmark 32 | 905 |  | 961 | 987 |  |  |  |  |
| Coretest 3.02 |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10240 |  | 10272 | 10272 |  |  |  | KB/sec |
| Performance ID | 69.03 |  | 69.22 | 69.22 |  |  |  |  |

5. Power Consumption Data

System Information:

| DRAM Type & Capacity | KOREA HY57V168010D 32MB SDRAM |
| --- | --- |
| VGA Card | OnBoard C&T 69000 |
| FLOPPY | Teac FD-235HF |
| HDD TYPE | N/A |
| VGA Testing Mode | 640x480x16 colors |
| O.S. | DOS 6.22 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys Dos=high,umb Files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

The Application of Software

| #### Brand | #### Landmark System Speed Test Version 2.0 |
| --- | --- |

Test Result

| Mode Type ## Processor Type | Normal Mode | # Doze Mode | Standby Mode | Suspend Mode | Unit |
| --- | --- | --- | --- | --- | --- |
| # PENTIUM MMX-200 | 4 | 3.4 | 2.8 | 2 | Amp/Max |
| PENTIUM MMX-233 | 4 | 3.4 | 2.8 | 2 | Amp/Max |
| PENTIUM Tillamook 266 | 3 | 2.6 | 2.4 | 2 | Amp/Max |
| AMD K6-233 | 7 | 5.4 | 3.6 | 2 | Amp/Max |
| AMD K6-266 | 3.2 | 2.8 | 2.2 | 1.6 | Amp/Max |
| AMD K6-300 | 4 | 3.4 | 2.8 | 2 | Amp/Max |
| AMD K6/2 300 | 4.2 | 3.4 | 2.8 | 2 | Amp/Max |
| AMD K6/2 400 | 6.4 | 4.8 | 3.2 | 2 | Amp/Max |
| AMD K6/2 450 | 6.4 | 4.6 | 3.1 | 2.3 | Amp/Max |
| AMD K6/2 500 | 6.8 | 4.9 | 3.2 | 2.3 | Amp/Max |
| AMD K6/2 550 | 8.2 | 5.9 | 4 | 2.6 | Amp/Max |
| AMD K6/3 450 | 6.8 | 5.4 | 3.2 | 2 | Amp/Max |
| Cyrix M2 PR-200 | 5.8 | 4.5 | 3 | 2 | Amp/Max |
| Cyrix M2 PR-233 | 5.8 | 4.6 | 3 | 2 | Amp/Max |
| Cyrix M2 PR-300 | 6 | 4.9 | 3.2 | 2 | Amp/Max |
| IDT Winchip 225 | 4 | 3.6 | 3 | 2.5 | Amp/Max |
| IDT Winchip 240 | 4 | 3.6 | 3 | 2.4 | Amp/Max |

Note: AMD K6-2 450/ AMD K6-2 500/ AMD K6-2 550. FDD type : M-System DOC 16MB

 1.Power supply voltage : ＋5V

 2.Suspend Model all function off

6. Margin Testing

A. Temperature Test (Software: AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 70∘C |  |
| Low Temperature Margin (Burn-In 2 Hours) | -10∘C |  |

B. DC Voltage Test (Software: Qaplus5.29)

| Testing Condition | Result | Note |
| --- | --- | --- |
| VDC High Voltage Margin (Burn-In 2 Hours) | Pass |  |
| VDC Low Voltage Margin (Burn-In 2 Hours) | Pass |  |

C. IDE Drive Capability

Test Equipment

Primary IDE Port

Master Device: HD FUJITSU MPC3032AT

Slave Device:  N/A

Secondary IDE Port

Master Device: SAMPO-36X CDM-T631

Slave Device: N/A

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 75CM |  |
| Secondary IDE Port Flat-Cable Length Margin | 75CM |  |