AAEON

### ISO-9001/ISO-14001 Certified

### Industrial Automation PCs

SBC-598

QE Compatibility Test Report

Release Date: May. 12, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager          QE Manager         Test Engineer

Contents

1. Software compatibility ------------------------------------------------------------------------------------------------- 3

Includes various OS and application software tests.

2. Hardware compatibility ------------------------------------------------------------------------------------------------ 5

Includes CPU, DRAM, adapter card and HDD compatibility tests.

3. Stability -------------------------------------------------------------------------------------------------------------------- 9

Includes burn-in and booting tests.

4. Performance -------------------------------------------------------------------------------------------------------------- 10

Includes performance indexes using various performance test software programs

5. Power Consumption Data --------------------------------------------------------------------------------------------- 11

Includes +5V current rate.

6. Margin --------------------------------------------------------------------------------------------------------------------- 12

Includes temperature, DC voltage and IDE cable length tests.

There are 3 test results in this document; they are described as follows:

Pass: It worked perfectly

Fail: Test failed, and to be modified in the next version

N/A: Outside of specifications or no test equipment

The test report would be tested and updated after version changed.

AAEON TECHNOLOGY INC.

1F, No.6,Alley 6, Lane 45,Pao-Hsin Rd,

Shing-Tien City, Taipei, Taiwan, ROC

Tel:  886-29121234

Fax: 886-29130792

886-29130774

886-2-9130774

1. Software Compatibility

A. O.S. Compatibility

| O.S. / Test Item | Install |  |  | Boot |  |  | Shutdown |  |  | Function |  |  | K/B Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail | Pass |  | Fail |
| MS-DOS 6.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QNX 4.22 & Wins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOVELL 4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B. Microsoft Windows & IBM OS/ 2 Compatibility

| Wins / Test Item | Install |  | Boot |  | Shutdown |  | Function |  | Keyboard Function |  | Display Driver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail | Pass | Fail |
| Windows 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows 95 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows Nt 4.0 W/S |  |  |  |  |  |  |  |  |  |  |  |  |
| Windows Nt 4.0 Server |  |  |  |  |  |  |  |  |  |  |  |  |
| OS/2 Warp 4.0 |  |  |  |  |  |  |  |  |  |  | N/A |  |

C. Windows Application

| Application / Test Item | Installation |  | Function Test |  |
| --- | --- | --- | --- | --- |
|  | Result | Note | Result | Note |
| Ami Pro 3.1 | Pass |  | Pass |  |
| MS Word 6.0c | Pass |  | Pass |  |
| WordPerfect 6.1 | Pass |  | Pass |  |
| MS Works 3.0b | Pass |  | Pass |  |
| Lotus 1-2-3 R5 | Pass |  | Pass |  |
| MS Excel 5.0c | Pass |  | Pass |  |
| Quattro Pro 6.01 | Pass |  | Pass |  |
| PageMaker 5.0a | Pass |  | Pass |  |
| CorelDRAW 5.0E2 | Pass |  | Pass |  |
| PowerPoint 4.0c | Pass |  | Pass |  |
| Dbase 5.0 | Pass |  | Pass |  |
| Paradox 5.0 | Pass |  | Pass |  |
| Access 2.0 | Pass |  | Pass |  |

D. DOS Utility Compatibility Testing

| Application / Test Result |  |  | Result | Note |
| --- | --- | --- | --- | --- |
| Norton Utility 8.0 |  |  | Pass |  |
| Norton Commander 5.0 |  |  | Pass |  |
| Telix 3.2x |  |  | Pass |  |
| ET 3.53 | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |
| ET 2000 (For DOS) | 16 * 15 Font Mode | Pass |  |  |
|  | 24 * 24 Font Mode | Pass |  |  |

E. Diagnostics Software Testing

| Test Item / Software | Qafe5.29 |  |  | Qaplus5.5 |  |  | Amidiag4.5 |  |  | Amidiag5.11 |  |  | Pccheck3.02 |  |  | Checkit3.0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Result | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note | Result |  | Note |
| Main Component | Fail | Note1 | Fail |  | Note2 | Fail |  | Note3 | Fail |  | Note4 | Fail |  | Note5 | Pass |  |  |
| Memory | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Video Adapter | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Floppy Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Hard Drive | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Keyboard | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Com Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Lpt Port | Pass |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  | Pass |  |  |
| Plug & play | Pass | Ver.1.0 Nodes=14 | #### Pass |  | Ver.1.0 Nodes=14 | - |  |  | - |  |  | - |  |  | - |  |  |
| Note: 1-2. RTC Clock / Interrupt Ctrl Register/ Interval Timer Test --- N/A. 3-4. Timer Test/ Real Timer Clock Test---N/A. 5. Alarm / CMOS RAM --- N/A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

F. Windows NT 4.0 HCT (Hardware Compatibility Test)

| Test Item / Test Result |  | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  | #### Pass |  |
| Mouse |  | Pass |  |
| Parallel port |  | Pass |  |
| Sound (PC speaker) |  | Pass |  |
| Serial Port (COMx) |  | Pass |  |
| Coprocessor |  | Pass |  |
| Disk (FAT tile I/O) |  | Pass |  |
| Disk (mapped I/O) |  | Pass |  |
| Disk (NTFS file I/O) |  | Pass |  |
| Floppy I/O |  | Pass |  |
| Video (GDI w/poly.clip.) | Pass |  |  |
| Video (VDM VGA) | Pass |  |  |
| Virtual DOS mach | Pass |  |  |
| Coproc. (stress) | Pass |  |  |
| Disk (stress) | Pass |  |  |
| System stress | Pass |  |  |

2. Hardware Compatibility

A. Component Compatibility- CPU

CPU Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Intel | Pentium-75 | #### Pass |  |
| Intel | Pentium-100 | Pass |  |
| Intel | Pentium-120 | Pass |  |
| Intel | Pentium-133 | Pass |  |
| Intel | Pentium-150 | Pass |  |
| Intel | Pentium-166 | Pass |  |
| Intel | Pentium-200 | Pass |  |
| Intel | Pentium-200 MMX | Pass |  |
| Intel | Pentium-233 MMX | Pass |  |
| Cyrix | 6X86-110M | Pass |  |
| Cyrix | 6X86-120M | Pass |  |
| Cyrix | 6X86-133M | Pass |  |
| Cyrix | 6X86-166M | Pass |  |
| Cyrix | 6X86-200 M1 | Pass |  |
| Cyrix | 6X86-200 M2 | Pass |  |
| AMD | 5K86-75 | Pass |  |
| AMD | 5K86-100 | Pass |  |
| AMD | K6-200 | Pass |  |
| AMD | K6-233 | Pass |  |

B. Component Compatibility- DRAM SIMM Module

DRAM SIMM Module List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PANASONIC | MN4117400BSJ-06 32M FPM RAM | #### Pass |  |
| MITSUBISH | M5M417400CT-7 16M FPM RAM | Pass |  |
| HITACHI | HM5118165AJ6R 8M EDO RAM | Pass |  |
| SAMSUNG | KM44C4000AK-6 16M FPM RAM | Pass |  |
| SIEMENS | HYB5117405BJ-60 32M EDO RAM | Pass |  |
| HYNDAI | HY5117400AJ-70 16M FPM RAM | Pass |  |
| NPNX | NN514405J-60 8M EDO RAM | Pass |  |
| VITELIC | V53C408HK45 8M EDO RAM | Pass |  |
| SAMSUNG | KM41C16000BK-6 64M FPM RAM | Pass |  |
| PANASONIC | PP716M66 16M W/PARITY | Pass |  |
| NEC | NX716-6 16M EDO RAM | Pass |  |
| SIEMENS | HYB5118165BSJ-60 8M EDO RAM | Pass |  |
| SIEMENS | HYB5117405BSJ-60 8M EDO RAM | Pass |  |
| PANASONIC | MN414400BSJ-07 8M FPM RAM | Pass |  |
| LGS | GM71C17403CJ6 32M EDO RAM | Pass |  |
| NEC | 421865-60 8M EDO RAM | Pass |  |
| NEC | 4217400-60 32M EDO RAM | Fail |  |
| MICON | MT4C4M4E8DJ-6 16M EDO RAM | Pass |  |
| TI | TMS417409ADJ-6 16M EDO RAM | Pass |  |
| ### MITSUBISHI | M5M418165CJ 8M EDO RAM | Pass |  |
| LGS | GM71C17403CJ6 16M EDO RAM | Pass |  |
| LGS | GM71C18163CJ6 8M EDO RAM | Pass |  |
| HYNDAI | HY5118164B 8M EDO RAM | Pass |  |
| HITACHI | HM5216805TT10H 16M SDRAM | Pass |  |
| NEC | ### D42S18165LG-A60-7JF 16M SDRAM | Pass |  |
| NEC | SDNX64M 64M SDRAM | Pass |  |
| NEC | SDNX128M 128M SDRAM | Pass |  |

C. Adapter Compatibility- VGA

VGA Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| CARDEX | ET-6000 | #### Pass |  |
| TopView | ET 4000 W32P | Pass |  |
| TopView | ET4000 /AX | Pass |  |
| TopView | S3 86C968 | Pass |  |
| TopView | S3 Trio64 V2 | Pass |  |
| TopView | S3 VERGE | Pass |  |
| On_board | SIS5598 | Pass |  |
| ISA BUS |  |  |  |
| ORCHID | KELVIN 64 ISA (CIRRUS 5429) | Pass |  |
| TopView | ET 4000 ISA | Pass |  |

D. Adapter Compatibility- LAN

LAN Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| D-Link | DE530 | #### Pass |  |
| 3COM | 3C905 | Pass |  |
| Intel | Ether Express PRO/100B S82557 | Pass |  |
| ISA BUS |  |  |  |
| 3COM | 3C509B-C | #### Pass |  |
| INTEL | ISA PILA 8560 | Pass |  |
| SVAC | ETHER 16 | Pass |  |

E. Adapter Compatibility- SCSI CARD

SCSI Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| PCI BUS |  |  |  |
| AHA | 2940UW PCI | #### Pass |  |
| ISA BUS |  |  |  |
| AHA | 1520K ISA | #### Pass |  |

F. Adapter Compatibility- Other Card

Adapter Card List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| MPEG Card |  |  |  |
| LEMELMagic | MPEG Card MPG256V | #### Pass |  |
| Sound Card |  |  |  |
| Creative | AWE 32 | #### Pass |  |
| Creative | SB 16 Value PNP | Pass |  |

G. Adapter Compatibility- Industry Control Card

Industry Control Adapter List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| ISA Card |  |  |  |
| AAEON | RDM-1020N | #### Pass |  |
| AAEON | RDM-1010A | Pass |  |
| AAEON | RDM-211A | Pass |  |
| AAEON | PCD-896 | Pass | VCC MUST OVER +5.00V |
| AAEON | PCD-894A | Pass |  |
| AAEON | PCD-895A | Pass |  |

H. Peripheral Compatibility-HDD Device

HDD List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Seagate | ST3123ON SCSI | #### Pass |  |
| Seagate | ST32132A | Pass |  |
| Seagate | ST33232A | Pass |  |
| Seagate | ST51270A | Pass |  |
| WD | WDAC22100 | Pass |  |
| WD | WDAC33100 | Pass |  |
| Quantum | FIRE BALL TM1700A | Pass |  |
| Quantum | ST3240AT | Pass |  |
| MAXTOR | 83240D2 | Pass |  |
| MAXTOR | 85120A8 | Pass |  |

I. Peripheral Compatibility- Storage Device

Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| CD/ROM |  |  |  |
| Creative | CD 820E | #### Pass |  |
| Philips | PCA123CD | Pass |  |
| VUGEO | 624A-004 | Pass |  |
| Panasonic | CR-585-B | Pass |  |
| Pioneer | DR-A24X | Pass |  |
| FAX/Modem |  |  |  |
| LEMEL | 急速33.6 HIGH SPEED MODEM | #### Pass |  |
| Printer |  |  |  |
| HP | LaserJet III | #### Pass |  |

J. Peripheral Compatibility- Input Device

Input Device List:

| Brand | Device Model | Result | Note |
| --- | --- | --- | --- |
| Keyboard |  |  |  |
| BTC | 5140 | #### Pass |  |
| ACER | 6511 | Pass |  |
| Mouse |  |  |  |
| Microsoft | Serial Mouse | #### Pass |  |
| IBM | PS/2 Mouse | Pass |  |
| Logitech | First Mouse SUPER | Pass |  |
| Logitech | First Mouse | Pass |  |
| Logitech | Mouse Man (PS/2,Serial) | Pass |  |

3. Stability Testing

A. Burn-In Stability Testing

Test Condition: Room Temp. 48 hours

| Test Software | Result | Note |
| --- | --- | --- |
| AMIDIAG4.5 | #### Pass |  |

B. Booting Stability Testing

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 1000 | 0 |  |
| KB Reset 20 Times | 20 | 20 | 0 |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0 |  |

C. Continuous Burn-In Booting Stability Testing

Test Condition: Burn-In Under Temp. 60℃

| Test Method | Actual | Successful | Fail Rate | Note |
| --- | --- | --- | --- | --- |
| Power On/Off 1000 Times | 1000 | 999 | 0.1﹪ |  |
| H/W Reset 1000 Times | 1000 | 1000 | 0 |  |

4. Performance Testing

System Information:

| DRAM Type & Capacity | Panassonic 16MBX2 |
| --- | --- |
| VGA Card | OnBoard (SIS 5598) |
| HDD Type | Segate ST51270A(Mode4) |
| ### VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 95 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | Intel p 200 | Intel P 200MMX | Intel P 233MMX | Cyrix M2-200 | Cyrix M2-233 | Cyrix M2-300 | AMD K6-233 | AMD K6-266 | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |
| CPU | 1150.54 | 1304.29 | 1521.68 | 2060.13 | 2325.95 | 2883.76 | 2349.15 | 2684.73 | Mhz |
| FPU | 3370.31 | 3467.09 | 4652.66 | 2313.16 | 2671.5 | 3335.70 | 3665.29 | 4279.24 | Mhz |
| VIDEO | 14456 | 14246 | 14456 | 6597 | 6597 | 6383 | 14456 | 8124 | Chs/sec |
| Norton 8.0 SI | 631.6 | 676.7 | 789.5 | 1127.3 | 1272.5 | 1228.3 | 1380.8 | 1578.1 | ID |
| Power Meter 2.02 | 94.8 | 98.1 | 113.8 | 113.8 | 129.3 | 167.3 | 117.7 | 189.6 | Mips |
| PcBench 9.0 CPUmark16 | 389.87 | 450.47 | 508.99 | 495.24 | 537.11 | 663.33 | 665.41 | 745.15 |  |
| ###### WinStone 98 | 14.8 | 17.7 | 17.8 | 17.1 | 17.7 | 20.4 | 18.6 | - |  |
| WINBENCH 98 |  |  |  |  |  |  |  |  |  |
| High Graphic WinMark 98 | 72.5 | 94.8 | 100 | 110 | 114 | 122 | 112 | 105 | MP/s |
| CPUmark 32 | 331 | 394 | 417 | 415 | 447 | 478 | 485 | 521 |  |
| CORETEST 3.02 |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 12896 | 12640 | 12560 | 11664 | 12976 | 12544 | 13488 | 12416 | KB/sec |
| Performance ID | 81.67 | 80.14 | 79.67 | 74.37 | 83.94 | 81.45 | 85.15 | 80.52 |  |
| Wintech 1.2 | 106.97 | 124.84 | 130.13 | 126.83 | 235.26 | 204.57 | 125.87 | 178.9 |  |
| Note |  |  |  |  |  |  |  |  |  |

System Information:

| DRAM Type & Capacity | NEC D4564841G5-A10-9JF 64M |
| --- | --- |
| VGA Card | OnBoard (SIS 5598) |
| HDD Type | IBM DPTA-372050 |
| ### VGA Testing Mode | 1024X768X256 colors (For WinStone & WinBench) |
| O.S. | DOS V 6.22 & Windows 98 / Windows NT4.0 |

Operating System Environment (DOS Only)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high files=35 Buffers=40 | Prompt $p$g C:\dos\smartdrv.exe /x |

Testing Software

|  | CPU Configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Utility | AMD K6-2 300 |  |  |  |  |  |  |  | Unit |
| LandMark Speed 2.0 |  |  |  |  |  |  |  |  |  |
| CPU | 28641.1 |  |  |  |  |  |  |  | Mhz |
| FPU | 4567.57 |  |  |  |  |  |  |  | Mhz |
| VIDEO | 7801.00 |  |  |  |  |  |  |  | Chs/sec |
| Norton 8.0 SI | 1330.8 |  |  |  |  |  |  |  | ID |
| Power Meter 2.02 | 167.3 |  |  |  |  |  |  |  | Mips |
| PcBench 9.0 CPUmark16 | 863.1 |  |  |  |  |  |  |  |  |
| ###### WinStone 99 | 10.6 |  |  |  |  |  |  |  |  |
| WINBENCH 99 |  |  |  |  |  |  |  |  |  |
| High Graphic WinMark 99 | 148 |  |  |  |  |  |  |  | MP/s |
| CPUmark 32 | 355 |  |  |  |  |  |  |  |  |
| CORETEST 3.02 |  |  |  |  |  |  |  |  |  |
| DTR (Buffered Read) | 10848 |  |  |  |  |  |  |  | KB/sec |
| Performance ID | 72.77 |  |  |  |  |  |  |  |  |
| Wintech 1.2 | 210.43 |  |  |  |  |  |  |  |  |
| Note |  |  |  |  |  |  |  |  |  |

5. Power Consumption

System Information:

| DRAM Type & Capacity | NEC D4564841G5 –A80 –9JF |
| --- | --- |
| VGA Card | OnBoard SIS 5598 |
| DiskOnChip | MD2200-D08 |
| O.S. | DOS Mode |

Operating System Environment (DOS Mode)

| Config.sys | Autoexec.bat |
| --- | --- |
| Device=c:\dos\himem.sys dos=high,umb files=35 Buffers=40 | Prompt $p$g c:\dos\smartdrv.exe /x |

Test Software (DOS Mode):

| Test Software | ### Landmark Speed V.2.0(Speed200.exe) |
| --- | --- |

Test Result:

| Mode Type | CPU Type |  |  |  |  |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pentium 200 | Pentium 200MMX | Pentium 233MMX | Cyrix M2-200 | Cyrix M2-233 | Cyrix M2-300 | AMD K6-233 | AMD K6-266 | AMD K6-2 300 |  |  |
| Normal Mode(+5V) | 3 | 4.6 | 5 | 7.2 | 8 | 9 | 9 | 3.8 | 10.2 | amp/max |  |
| Suspend Mode(+5V) | 1.6 | 2.2 | 2.1 | 1.9 | 1.8 | 1.8 | 2 | 1.6 | 2.1 | amp/max |  |

Note:

    1.Suspend Model all function off.

2.Power supply voltage: +5V

3.Normal Model is measured while run landmark speed Rev.2.0 in DOS

6. Margin Testing

A. Temperature Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 80℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -40℃ |  |

B. Temperature Test (BOOT UP)

| Testing Condition | Result | Note |
| --- | --- | --- |
| High Temperature Margin (Burn-In 2 Hours) | 65℃ |  |
| Low Temperature Margin (Burn-In 2 Hours) | -20℃ | The Fail Rate Is 4% In -25℃. |

C. DC Voltage Test (AMIDIAG 4.5)

| Testing Condition | Result | Note |
| --- | --- | --- |
| Vdc High Voltage Margin (Burn-In 2 Hours) | 5.25v |  |
| Vdc Low Voltage Margin (Burn-In 2 Hours) | 4.65v |  |

D. IDE Drive Capability

Test Equipment:

Primary IDE Port:

Master Device: IBM DHEA-34330

Slave Device: Panasonic 24X

Secondary IDE Port:

Master Device: IBM DHEA-34330

Slave Device: Panasonic 24X

| Testing Condition | Result | Note |
| --- | --- | --- |
| Primary IDE Port Flat-Cable Length Margin | 65CM |  |
| Secondary IDE Port Flat-Cable Length Margin | 65CM |  |