Product Certification Report

Report No: SBC-659P-T-02

| Product Model | SBC659P |
| --- | --- |
| Product description | ### FC / Socket 370 Half-Size CPU Card with LCD, Dual Ethernet , Audio , AGP 2X ,Compact Flash , & 4 USB Ports |
| Test reason | New product PCB : SBC-659P Rev.A0.3 BIOS: SBC-659 BIOS Rev 0.8 + PCM-3536 for LAN receive (09/14/2001) Renew product PCB : BIOS:  Revision change PCB : BIOS: Component: |

| 2001-10-19 |  | Jason Ho |  | Wenyuan Yang |  | Ryan Cheng |
| --- | --- | --- | --- | --- | --- | --- |
| Issue date |  | Manager |  | Assistant Manager |  | Test Engineer |

Test result for certification

| Stability Test - Dynamic / Boot_UP Burn_In Test |  |  | Result |
| --- | --- | --- | --- |
|  | Dynamic Burn-In test. ( Operating at 0℃~60℃ for 48hours) |  | Pass |
|  | Audio Play For Sound Codec Chipset Burn-In Test .( Operating at 0℃ & 60℃ for 2hours) |  | Pass |
|  | LAN Chipset Burn_In Test. ( Operating at 0℃ and 60℃ for 2hours) |  | Pass |
|  | Boot_Up Burn_In Test .( Operating at 0℃~60℃ for 24hours) |  | Pass |
|  | Boot_Up Burn_In Test .( Operating at Room Temperature for 24hours) |  | Pass |
|  | Windows HCT 9.1 system stress test. ( Operating at Room Temp. for 8hours) |  | Pass |
| Timer Accuracy Test |  |  | Result |
|  | RTC timer accuracy of +/-3 sec per day. |  | Pass |
|  | System clock accuracy of +/-5 sec per day. |  | Pass |
|  | Watchdog timer accuracy of +/-1 sec per minute. |  | Pass |
| Margin Test – Power Consumption – PIII 1G , SDRAM 256MB |  | Result |  |
|  | Power VCC 4.75V to 5.25V Margin test . |  | Pass |
|  | Electrical characteristics – Normal Mode( +3.3V) |  | 0.27A |
|  | Electrical characteristics -- Standby Mode(+3.3V) |  | 0.27A |
|  | Electrical characteristics – Normal Mode( +5V) |  | 7.59A |
|  | Electrical characteristics -- Standby Mode(+5V) |  | 3.15A |
|  | Electrical characteristics – Normal Mode( +12V) |  | 0.24A |
|  | Electrical characteristics -- Standby Mode(+12V) |  | 0.24A |
| BIOS Function Test |  |  | Result |
|  | Y2K Test |  | Pass |
|  | Standard CMOS Setup Test |  | Pass |
|  | Boot Sequence Test |  | Pass |
|  | APM Power manager test |  | Pass |
|  | ACPI Power manager test |  | Pass |
|  | Power On Function test |  | Pass |
|  | CMOS backup Test |  | Pass |
|  | Boot ROM Test |  | Pass |
|  | COM Address/IRQ Test |  | Pass |
|  | LPT Address /IRQ Test |  | Pass |
|  | Hardware Monitor Test |  | Pass |
|  | Supervisor / User Password Test |  | Pass |
|  | BIOS Awdflash Program Test |  | Pass |
| Operation System Compatible Test |  |  | Result |
|  | Linux Red Hat 7.0 |  | Pass |
|  | Netware 4.11 |  | Pass |
|  | Netware 5.0 |  | Pass |
|  | Windows 95 English version |  | Pass |
|  | Windows 98SE English version |  | Pass |
|  | Windows ME English version |  | Pass |
|  | Windows NT workstation 4.0 English version with service pack 6 |  | Pass |

|  | Windows 2000 professional English version | Pass |
| --- | --- | --- |
|  | DOS and DOS Utility Compatibility Testing | Pass |
|  | Windows NT 4.0 Hardware Compatibility Test (HCT) | Pass |
| Performance Test – CPU Pentium III 1G , DRAM 128MB |  | Result |
|  | LandMark Speed 2.0 – CPU | 6294.43 |
|  | LandMark Speed 2.0 – FPU | 20910.00 |
|  | LandMark Speed 2.0 – VIDEO | 12136.00 |
|  | WinStone 99 V1.1 – High_end winstone99 | 58.3 |
|  | WinBench 99 V1.1- High_end Graphic winmark99 | 870 |
|  | WinBench 99 V1.1- Business Graphic winmark99 | 223 |
|  | WinBench 99 V1.1- FPU winmark | 5320 |
|  | WinBench 99 V1.1- CPU mark99 | 84.4 |
|  | Final Reality V1.01 BenchMark Test – 2D image | 7.30 |
|  | Final Reality V1.01 BenchMark Test – 3D | 5.88 |
|  | Final Reality V1.01 BenchMark Test – Bus transfer | 8.44 |
|  | Final Reality V1.01 BenchMark Test – Overall | 6.69 |
|  | 3D WinBenech 2000 v1.0 with DirectX7 and 1024x768x16Bits-Processor Test | 1.89 |
|  | Content Creation Winstone 2000 V1.0 – C.C.Winstone 2000 | 32.4 |
| Utility and Driver Test |  | Result |
|  | VGA for Windows 95 (OnBoard Intel 82815E Pro Savage) | Pass |
|  | VGA for Windows 98SE (OnBoard Intel 82815E Pro Savage) | Pass |
|  | VGA for Windows ME (OnBoard Intel 82815E Pro Savage) | Pass |
|  | VGA for Windows 2000P (OnBoard Intel 82815E Pro Savage) | Pass |
|  | VGA for Windows NT 4.0 (OnBoard Intel 82815E Pro Savage) | Pass |
|  | LAN for Windows 95 (OnBoard Intel GD82559 & Intel GD82559) | Pass |
|  | LAN for Windows 98SE (OnBoard Intel GD82559 & Intel GD82559 ) | Pass |
|  | LAN for Windows ME (OnBoard Intel GD82559 & Intel GD82559) | Pass |
|  | LAN for Windows 2000P (OnBoard Intel GD82559 & Intel GD82559) | Pass |
|  | LAN for Windows NT 4.0 (OnBoard Intel GD82559 & Intel GD82559) | Pass |
|  | Sound for Windows 95 (ALC200/ ADVANCE AC‘97) | Pass |
|  | Sound for Windows 98SE (ALC200/ ADVANCE AC‘97) | Pass |
|  | Sound for Windows ME (ALC200/ ADVANCE AC‘97) | Pass |
|  | Sound for Windows NT 4.0 (ALC200/ ADVANCE AC‘97) | Pass |
|  | Sound for Windows 2000P (ALC200/ ADVANCE AC‘97) | Pass |
|  | Red hat Linux 7.0 hardware compatible list for VGA | Support |
|  | Red hat Linux 7.0 hardware compatible list for LAN | Support |
| Memory Test – SDRAM PC-133/100 |  | Result |
|  | Infineon HYB39S128800CT-75 / PC133 / 128MB | Pass |
|  | NEC D45128163G5-A75-9JF / PC133 / 64MB | Pass |
|  | TOSHIBA TC59SM716FT-80 / PC100 / 128MB | Pass |
|  | HITACHI 5225165BTT75 / PC133 / 256MB | Pass |
|  | HITACHI HB52RF648DC-75B / 512MB | Pass |

| CPU Compatible Test |  | Result |
| --- | --- | --- |
|  | Pentium III 1G (133 FSB) | Pass |
|  | Pentium III 850 (100 FSB) | Pass |
|  | Pentium III 750 (100 FSB) | Pass |
|  | Celeron 800 (100 FSB) | Pass |
|  | Celeron 766 (66 FSB) | Pass |
|  | Celeron 633 (66 FSB) | Pass |
|  | Celeron 533 (66 FSB) (.25u) | Note1 |
| HDD Compatible Test |  | Result |
|  | IBM DTLA-307075 / 76.8GB / ATA100 | Pass |
|  | IBM DTLA-307045 / 46.1GB / ATA100 | Pass |
|  | Maxtor 54098U8 / 40.9GB / ATA66 | Pass |
|  | Seagate ST315320A / 15.3GB / ATA66 | Pass |
|  | Quantum QMP30000AS-A / 30GB / ATA66 | Pass |
|  | Quantum K13600AT / 13GB / ATA66 | Pass |
|  | Quantum LM20500AT / 20.5GB / ATA-66 | Pass |
| CDROM / DVDROM Test |  | Result |
|  | NEC C&C-BCD F561B / 52X | Pass |
|  | ASUS CD-S500/A / 50X | Pass |
|  | TEAC CD-540E / 40X | Pass |
|  | SONY CDU4811-81 / 48X | Pass |
|  | CREATIVE DVD1240E / 12X / DVD-ROM | Pass |
|  | PIONEER ATAPI DVD-A03S / 6X / DVD-ROM | Pass |
|  | Acer DVP 1640A /16X /DVD-ROM | Pass |
|  | MITSUMI CRMC-FX4824T / 48X | Pass |
|  | AFREEY CD-2050E / 50X | Pass |
|  | RICOH MP7060S / 24X / SCSI-CDROM | Pass |
| Network HUB / LAN Function Test |  | Result |
|  | HP J4095A 24Ports [ 100 /10 Mbps] | Pass |
|  | D-Link DES-1008 [ 100 / 10 Mbps ] | Pass |
|  | Intel Inbusiness 8 Ports [ 100 / 10 Mbps ] | Pass |
|  | LEMEL LM-S5 5 Ports [ 100 / 10 Mbps ] | Pass |
|  | Accton DeskTop-3005 [ 100 / 10 Mbps ] | Pass |
|  | SVEC FD916H [ 10M Mbps ] | Pass |
| On Board Audio Function Test |  | ### Result |
|  | System Sound | Pass |
|  | Play CD_IN | Pass |
|  | Play Sound Recorder | Pass |
|  | Line Out | Pass |
|  | Line In | Pass |
|  | Line In Sound Recorder | Pass |
|  | MIC Speaker – Dynamic / Condenser | Pass |

|  | MIC Speaker Recorder – Dynamic / Condenser | Pass |
| --- | --- | --- |
| LCD Monitor |  | Result |
|  | Samsung / LTM150XS-T01 / TFT / 15” | Pass |
| SCSI Card Compatible Test |  | ### Result |
|  | AHA-2940UW (PCI bus) | Pass |
|  | AHA-1520B (ISA bus) | Pass |
| VGA Card Compatible Test |  | ### Result |
|  | 3D Blaster Savage 4 (PCI bus) | Pass |
|  | LEMEL S3 TRIO64V2-DX/GX (ISA bus) | Pass |
| LAN Card Compatible Test |  | ### Result |
|  | Intel SB82559 (10/100MB) (PCI bus) | Pass |
|  | Intel PILA8560 (10/100MB) (PCI bus) | Pass |
| Modem Card Compatible Test |  | Result |
|  | Genuine GM56-AMI2019 (PCI bus) | Pass |
| Multi Media Card Compatible Test |  | Result |
|  | Creative CT4170 (ISA bus) | Pass |
|  | Creative SB16 PNP/CT2980 (ISA bus) | Pass |
|  | Lemel ESS ES1868F 3D/16 PnP (ISA bus) | Pass |
| Industry Control Adapter Card Compatible Test-PC104/PC104Plus |  | Result |
|  | AAEON PCM-3600 / Fax/Modem Module | Pass |
|  | AAEON PCM-3420 / Fast SCSI-2 Module | Pass |
|  | AAEON PCM-3660 / RTL9019LAN Module | Pass |
|  | AAEON PCM-3110 / PCMCIA Module | Pass |
|  | AAEON PCM-3115B / PCMICA Module | Pass |
|  | AAEON PCM-3640 / RS-232 Module | Pass |
|  | AAEON PCM-3730 / Intel 82559 LAN Module / PC104 Plus | Pass |
|  | AAEON PCM-3430 / SCSI Module / PC104 Plus | Pass |
| # System Integer Application Test |  | Result |
|  | PCI cards integer application | Pass |
|  | ISA cards integer application | Pass |
|  | PCI and ISA cards integer application | Pass |
|  | Riser Card integer application | Pass |
| ### Function Test |  | Result |
|  | On board LAN with 100 feet cable length | Pass |
|  | On board VGA function | Pass |
| Peripheral Storage Device Compatible test |  | Result |
|  | IOMEGA ZIP100M ( Parallel Type ) | Pass |
|  | IOMEGA ZIP100M ( lDE Type ) | Pass |
|  | LS-120 / LKM-F930-1 (IDE Type) | Pass |
|  | Photo Smart P1000 Printer (LPT Type) | Pass |
|  | 33.6 HIGH SPEED MODEM (RS-232) | Pass |
| Peripheral Input Device Compatible test |  | Result |

|  | BTC 7932 Keyboard ( USB ) | Pass |
| --- | --- | --- |
|  | Logitech M-BB48 Mouse (USB) | Pass |
|  | HP SJ4200C Scanner (USB) | Pass |
|  | HP Photo Smart P1000 Printer (USB) | Pass |
|  | Tekcam P35U Camera (USB) | Pass |
|  | IOMEGA ZIP100MB (USB) | Pass |
|  | D-Link DU-E10 Ethernet Adapter (USB) | Pass |
|  | D-Link DSB-H4/DU-H4 HUB (USB) | Pass |
|  | Microsoft Serial Mouse | Pass |
|  | Logitech Mouse Man | Pass |
|  | Adam RS-485 ( COM2 ) | Pass |
| Miscellaneous Test |  | Result |
|  | Windows 98 Power manager test | Pass |
|  | Windows 2000 Power manager test | Pass |
|  | Floppy Disk Controller | Pass |
|  | H/W Reset | Pass |
|  | On board buzzer | Pass |
|  | HDD LED | Pass |
|  | Ext. KB connector | Pass |
|  | ATX power button | Pass |
|  | CPU FAN connector | Pass |
|  | Primary IDE Master | Pass |
|  | Primary IDE Slave | Pass |
|  | COM1 | Pass |
|  | COM2 | Pass |
|  | COM2-RS-422 | Pass |
|  | COM2-RS-485 | Pass |
|  | LPT1 | Pass |
|  | PS2 Keyboard / Mouse | Pass |
|  | USB Port1 | Pass |
|  | USB Port 2 | Pass |
|  | SIR / 115K bps | Pass |
|  | Compact flash solid state disk / TYPE1 /TYPEII | Pass |