AAEON Technology INC.

ISO-9001/ISO-14001 Certified

Industrial Automation PCs

SBC-658

Thermal Image Analysis Report

##### Release Date: JUNE.29, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager           QE Manager           Test Engineer

### Thermal Image Analysis

Ⅰ. Model Name: SBC-658

Ⅱ. Description: FC/Socket 370 Half-size CPU Card with LCD, Ethernet, & SSD

Ⅲ. Date: JUNE. 29, 2000

Ⅳ. Measure Site: AAEON QE Dept.

Ⅴ. Issued by : Fenglin Dong

Ⅵ.Equipment: TVS-100 series by NIPPON AVIONICS CO., LTD.

Ⅶ. Simulation Environment:

•Temperature: 25.5 degrees C

•CPU: PIII 700

•RAM: 128 MB (NEC D4564841G5-A80-9JF)

•Hardware: DiskOnChip

•Application Software: QAplusfe V5.2

•Take Picture Time: Power on 30 minutes after

Ⅷ. Prediction Result:

- Component Side:

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | U2 | ###### ICS, 9248-39 | 70 | 53.6 | ※ |
| #### 2 | U4 | INTEL, C&TM6900 | 75 | 44.8 | ※ |
| #### 3 | U5 | INTEL, 82559ER | 75 | 41.7 | ※ |
| #### 4 | U7 | LINEAR, LTC485CS8 | 70 | 41.2 | ※ |
| #### 5 | U14 |  |  | 39.9 |  |
| #### 6 | U13 |  |  | 38.8 |  |
| #### 7 | U1 | HARRIS, HIN213CA | 75 | 43.5 | ※ |
| #### 8 | CN6 | 鴻呈, AT-PH13-SMT-14-2-G | 70 | 41.4 | ※ |
| #### 9 | U11 |  |  | 38.0 |  |
| #### 10 | CPU1 |  |  | 36.2 |  |
| #### 11 | CN6 |  |  | 40.0 |  |
| #### 12 | U3 |  |  | 34.7 |  |
| #### 13 | --- |  |  | --- |  |
| #### 14 | U21 |  |  | 37.0 |  |
| #### 15 | --- |  |  | --- |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

- Solder Side:

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | Q6 | ###### PHILIPS, PHD50N03LT | 75 | 59.5 | ※ = -19.5 |
| #### 2 | Q5 | ###### PHILIPS, PHD50N03LT | 75 | 55.1 | ※ = -15.1 |
| #### 3 | Q2 | AMS, AMS1083CM | 75 | 56.6 | ※ = -16.6 |
| #### 4 | --- |  |  | --- |  |
| #### 5 | C115 |  |  | 41.2 |  |
| #### 6 | --- |  |  | --- |  |
| #### 7 | --- |  |  | --- |  |
| #### 8 | --- |  |  | --- |  |
| #### 9 | --- |  |  | --- |  |
| #### 10 | U27 | 74F070 | 70 | 43.5 | ※ = -8.5 |
| #### 11 | Q3 | AMS, AMS1084CM-1.5 | 70 | 45.0 | ※ = -10 |
| #### 12 | C108 | MCC, 1μF,10V,+80/-20%, 0603SMD | 75 | 46.3 | ※ = -6.3 |
| #### 13 | --- |  |  | --- |  |
| #### 14 | --- |  |  | --- |  |
| #### 15 | --- |  |  | --- |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |