AAEON Technology INC.

ISO-9001/ISO-14001 Certified

Industrial Automation PCs

Tiger Series

###### Reliability Prediction Report

(MTBF)

##### Release Date: OCT.31, 2001

__________   __________   __________   __________

# Issue Stamp          QA Manager           QE Manager           Test Engineer

### Reliability Prediction (MTBF)

Ⅰ. Model Name: TBOX-3300/TBOX-3303/TBOX-3306/TD-3212(T/M/MT)/TD-3215(T/M/MT)

Ⅱ. Prediction Date: OCT. 31, 2001

Ⅲ. Prediction Site: AAEON QE Dept.

Ⅳ. Predicted by : Linda Sun

Ⅴ. Equipment: Relex Ver.7.1 Visual Reliability Software

Ⅵ. Simulation Environment:

•Temperature: 25 degrees C

•Standard: Telcordia (Bellcore) Standard

Ⅶ. Term and Definition:

•Unit         An assembly of device.

•Duty Cycle   Used to specify the percentage of time that the element is in an on state, and it is

equal to the percentage of total time the item is in the active environment.

•Quantity     The quantity of the selected item.

•Failure Rate  Used to specify that the failure rate is to be calculated based the selected

Calculation models. The multiplier is 1,000,000.

•MTBF       MTBF is always specified in hours.

Ⅷ. Prediction Result:

•Box PC

| Model |  | MTBF(hr) | Failure Rate | Note |
| --- | --- | --- | --- | --- |
| TBOX-3300 | AC | 17,146 | 58.324148 |  |
|  | DC | 18,501 | 54.051038 | Not include DC Power |
| TBOX-3303 | AC | 15,250 | 65.572993 | Not include AC Power |
|  | DC | 15,257 | 65.542993 | Not include DC Power |
| TBOX-3306 | AC | 12,732 | 78.540829 |  |
|  | DC | 12,918 | 77.408369 |  |

•Front Panel(All)：

| Model |  | MTBF(hr) | Failure Rate | Note |
| --- | --- | --- | --- | --- |
| TD-3212 | T | 19,333 | 51.725591 | Not include Menbrane TS |
|  | M | 24,479 | 39.247401 | Not include Finger Mouse、Menbrane KB |
|  | MT | 19,333 | 51.725591 | Not include Finger Mouse、Menbrane KB、Menbrane TS |
| TD-3215 | T | 21,548 | 46.408001 | Not include Menbrane TS |
|  | M | 29,473 | 33.929811 | Not include Finger Mouse、Menbrane KB |
|  | MT | 21,548 | 46.408001 | Not include Finger Mouse、Menbrane KB、Menbrane TS |

Ⅸ. Key parts reference data :

| # Item | ## MTBF(hr) | Remark |
| --- | --- | --- |
| Slim CD-ROM Part Number:978A160000 | 140,000 | -the operating duty cycle is 5% of power on time. During this time, the drive is either reading or seeking(random multiple block read) |
| 2.5” FDD Part Number:9784666610 | 75,000 | -the operating duty cycle is 18.75% of power on time. |
| 3.5” FDD Part Number:9784023530 | 150,000 | -the operating duty cycle is 18.75% of power on time. |
| Slim HDD Part Number:9783220000 | 300,000 | - Power-on time: 250H/month or less 3000H/years or less. - Operating time: 20% or less of power-on time. - CSS operations: 50/day or less, Total 50,000 or less. - Power on/off: 1/day or more needed. - Environment: 5 to 55℃/8 to 90%. But humidity bulb temperature 29℃ or less. |

Ⅹ. Prediction Table: