AAEON Technology INC.

ISO-9001/ISO-14001 Certified

Industrial Automation PCs

MB-668

Thermal Image Analysis Report

##### Release Date: JUNE.30, 2000

__________   __________   __________   __________

# Issue Stamp          QA Manager           QE Manager           Test Engineer

### Thermal Image Analysis

Ⅰ. Model Name: PCM-668

Ⅱ. Description: Socket 370 LPX Board with LD, LAN, Audio, & 4 COMs

Ⅲ. Date: JUNE. 30, 2000

Ⅳ. Measure Site: AAEON QE Dept.

Ⅴ. Issued by : Fenglin Dong

Ⅵ.Equipment: TVS-100 series by NIPPON AVIONICS CO., LTD.

Ⅶ. Simulation Environment:

•Temperature: 25.2 degrees C

•CPU: Celeron

•RAM: 128 MB

•Hardware: DiskOnChip

•Application Software: QAPLUS/FE V5.2

•Take Picture Time: Power on 30 minutes after

Ⅷ. Prediction Result:

- Component Side: 1/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | U25 | ICS, 9248-39 | 70 | 45.9 | ※ = -10.9 |
| #### 2 | U10 | ###### INTEL,FW82443BX SL2VH | 75 | 34.7 |  |
| #### 3 | U1 |  |  | 33.5 |  |
| #### 4 | U14 |  |  | 33.2 |  |
| #### 5 | U3 | TI, 7407 | 75 | 36.3 |  |
| #### 6 | U36 | ICS, ICS9112-16 | 70 | 34.1 |  |
| #### 7 | U25 | ICS, 9248-39 | 70 | 37.1 | ※ = -2.1 |
| #### 8 | U23 |  |  | 32.7 |  |
| #### 9 | --- |  |  |  |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | U10 |  |  | 43.8 |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Component Side: 2/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | U25 | ICS, 9248-39 | 70 | 45.1 | ※ = -10.1 |
| #### 2 | CPU1 | CPU |  | 39.2 |  |
| #### 3 | --- |  |  |  |  |
| #### 4 | --- |  |  |  |  |
| #### 5 | --- |  |  |  |  |
| #### 6 | U37 |  |  | 35.1 |  |
| #### 7 | U12 | CHIPS, B69000 | 70 | 41.9 | ※ = -6.9 |
| #### 8 | U9 |  |  | 36.1 |  |
| #### 9 | CPU1 |  |  | 31.5 |  |
| #### 10 | U26 | REALTEK, RTL8139B | 70 | 44.4 | ※ = -9.4 |
| #### 11 | Y5 | EPSON, SG-8002JC50 | 70 | 39.0 | ※ = -4 |
| #### 12 | U36 |  |  | 33.8 |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | U15 |  |  | 35.7 |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Component Side: 3/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | U26 | REALTEK, RTL8139B | 70 | 45.1 | ※ = -10.1 |
| #### 2 | Y5 | EPSON, SG-8002JC50 | 70 | 38.7 |  |
| #### 3 | U12 | CHIPS, B69000 | 70 | 41.1 | ※ = -6.1 |
| #### 4 | U19 |  |  | 31.7 |  |
| #### 5 | U9 |  |  | 32.3 |  |
| #### 6 | U37 | Aureal, AU8810-A2ACAA | 70 | 35.3 |  |
| #### 7 | --- |  |  |  |  |
| #### 8 | U12 |  |  | 35.0 |  |
| #### 9 | U26 |  |  | 32.3 |  |
| #### 10 | U12 | CHIPS, B69000 | 70 | 37.4 |  |
| #### 11 | U12 | CHIPS, B69000 | 70 | 32.8 |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | --- |  |  |  |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Component Side: 4/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | U25 | ICS, 9248-39 | 70 | 45.9 | ※ = -10.9 |
| #### 2 | U10 | ###### INTEL, FW82443BX SL2VH | 70 | 43.8 | ※ = -8.8 |
| #### 3 | U10 |  |  | 36.6 |  |
| #### 4 | U10 |  |  | 33.0 |  |
| #### 5 | U38 |  |  | 32.3 |  |
| #### 6 | U14 | ###### TENYUN, 540-99-044-17-400 | 70 | 33.2 |  |
| #### 7 | U3 | TI, 7407 | 75 | 36.4 |  |
| #### 8 | U23 |  |  | 33.0 |  |
| #### 9 | --- |  |  | 29.4 |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | U1 | ###### INTEL, FW82371EB SL37M | 70 | 33.3 |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

- Solder Side: 1/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | --- |  |  |  |  |
| #### 2 | --- |  |  |  |  |
| #### 3 | --- |  |  |  |  |
| #### 4 | --- |  |  |  |  |
| #### 5 | --- |  |  |  |  |
| #### 6 | --- |  |  |  |  |
| #### 7 | --- |  |  |  |  |
| #### 8 | --- |  |  |  |  |
| #### 9 | --- |  |  |  |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | --- |  |  |  |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Solder Side: 2/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | --- |  |  |  |  |
| #### 2 | --- |  |  |  |  |
| #### 3 | --- |  |  |  |  |
| #### 4 | --- |  |  |  |  |
| #### 5 | U16 | TI, NE555 |  | 31.3 |  |
| #### 6 | --- |  |  |  |  |
| #### 7 | --- |  |  |  |  |
| #### 8 | --- |  |  |  |  |
| #### 9 | --- |  |  |  |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | --- |  |  |  |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Solder Side: 3/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | --- |  |  |  |  |
| #### 2 | --- |  |  |  |  |
| #### 3 | --- |  |  |  |  |
| #### 4 | --- |  |  |  |  |
| #### 5 | --- |  |  |  |  |
| #### 6 | --- |  |  |  |  |
| #### 7 | --- |  |  |  |  |
| #### 8 | --- |  |  |  |  |
| #### 9 | --- |  |  |  |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | --- |  |  |  |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |

Solder Side: 4/4

| #### Point | ## Position | Describe | Ts | Tm | Note |
| --- | --- | --- | --- | --- | --- |
| #### 1 | --- |  |  |  |  |
| #### 2 | --- |  |  |  |  |
| #### 3 | --- |  |  |  |  |
| #### 4 | --- |  |  |  |  |
| #### 5 | --- |  |  |  |  |
| #### 6 | --- |  |  |  |  |
| #### 7 | --- |  |  |  |  |
| #### 8 | --- |  |  |  |  |
| #### 9 | --- |  |  |  |  |
| #### 10 | --- |  |  |  |  |
| #### 11 | --- |  |  |  |  |
| #### 12 | --- |  |  |  |  |
| #### 13 | --- |  |  |  |  |
| #### 14 | --- |  |  |  |  |
| #### 15 | --- |  |  |  |  |
| 1. Operation Temperature (℃): Ts = Defined by component specification ; Tm = Measured by QE 2. ※ = Ts - (Tm + 60 - 25 ) |  |  |  |  |  |