| Product Model: ATX-C246A |  | Date: 2023/08/11 |
| --- | --- | --- |
| System BIOS: AX24AM15 |  | VBIOS: 9.2.1014 GOP: 9.0.1098 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Andre Liang (梁家偉) |  | Approved By: Danny Chiang(姜自立) |
| Item. | Root cause and change description |  |
| 01. | Adjust BIOS setup item for UEFI mode. |  |
| 02. | Fix UEFI RAID card didn't present HII setup item in Advanced page. |  |
| 03. | Secure Boot -> Disable. |  |
| 04. | Enable Graphics Output Protocol hot-plug support. |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE M22906ED_000000BE |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM15.BIN -> checksum:0xAA570E9B |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |

| Product Model: ATX-C246A |  | Date: 2022/08/24 |
| --- | --- | --- |
| System BIOS: AX24AM14 |  | VBIOS: 9.2.1014 GOP: 9.0.1098 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Andre Liang (梁家偉) |  | Approved By: Danny Chiang(姜自立) |
| Item. | Root cause and change description |  |
| 01. | Support Windows 11. |  |
| 02. | Show the secure boot item in setup menu and default to enable. |  |
| 03. | Show TPM selection item in setup menu. |  |
| 04. |  |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE M22906ED_000000BE |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM14.BIN -> checksum:0xAA426AB2 |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |

| Product Model: ATX-C246A |  | Date: 2021/01/29 |
| --- | --- | --- |
| System BIOS: AX24AM13 |  | VBIOS: 9.2.1014 GOP: 9.0.1098 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Ayers Deng (鄧政宏) |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |
| 01. | Update ALC897 verb table |  |
| 02. |  |  |
| 03. |  |  |
| 04. |  |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE M22906ED_000000BE |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM13.BIN -> checksum:0xAA8B391E |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |

| Product Model: ATX-C246A |  | Date: 2021/01/19 |
| --- | --- | --- |
| System BIOS: AX24AM12 |  | VBIOS: 9.2.1014 GOP: 9.0.1098 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Ayers Deng (鄧政宏) |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |
| 01. | Update verb table to support new Audio Codec ALC897 |  |
| 02. |  |  |
| 03. |  |  |
| 04. |  |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE M22906ED_000000BE |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM12.BIN -> checksum:0xAA8F7A36 |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |

| Product Model: ATX-C246A |  | Date: 2019/08/19 |
| --- | --- | --- |
| System BIOS: AX24AM11 |  | VBIOS: 9.2.1014 GOP: 9.0.1073 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Ayers Deng (鄧政宏) |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |
| 01. | Add CPU microcode 0x906ED to support CFL-R S Series Processors. (Coffee Lake-S R-0 Stepping) |  |
| 02. | Remove USB Type-C Alternate Function - Remove PD IC TPS65988 settings to prevent yellow mark in Device Manager. - Modify GOP/VBIOS settings. - Remove IGFX Primary/Secondary Boot Display "Type-C" option of Setup item. |  |
| 03. |  |  |
| 04. |  |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE M22906ED_000000B8 |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM11.BIN -> checksum:0x9D5FE4DE |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |

| Product Model: ATX-C246A |  | Date: 2019/08/01 |
| --- | --- | --- |
| System BIOS: AX24AM10 |  | VBIOS: 9.2.1014 GOP: 9.0.1073 |
| ME Type: Corporate ME Version: 12.0.34.1425 |  |  |
| Released By: Ayers Deng (鄧政宏) |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |
| 01. | First Release |  |
| 02. |  |  |
| 03. |  |  |
| 04. |  |  |
| 05. |  |  |
| 06. |  |  |
| 07. |  |  |
| CPU Microcode |  |  |
| M22906EA_000000B4 M02906EB_000000B4 M22906EC_000000AE |  |  |
| Remark |  |  |
| Checksum (BY Checksum.exe): fileName:AX24AM10.BIN -> checksum:0x9E27E2BC |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |