EC_WDT_Driver_User_Guide

Author: Nick Chen

2019.05.17

Contents

1.	Install Debian 9.8 (64-bit)	2

2.	Install EC WDT driver	3

3.	How To Access WDT	4

- Install Debian 9.8 (64-bit)

Step 1. Install Debian 9.8 (64-bit), and check the version

$ lsb_release -a

Step 2. Check the Debian kernel version (4.9.0)

$ uname -a

Step 3. Install require package

$ sudo apt-get install build-essential linux-headers-`uname -r` make

- Install EC WDT driver

Step 1. Move the base driver file to the directory of your choice.

(You need to check your driver file “aaeon_ec_wdt_VX.X.X.X”, and we use “aaeon_ec_wdt_V5.0.0.0” as the sample.)

For example, use ‘/home/username/driver/’

Step 2. Install EC WDT driver

- $ cd ~/driver/aaeon_ec_wdt_V5.0.0.0/

- $ chmod +x install.sh

- $ sudo ./install.sh

- $ sudo reboot

- How To Access WDT

Step 1. AAEON Watchdog Access Code:

$ cd ~/driver/aaeon_ec_wdt_V5.0.0.0/example/

$ sudo ./ecwdt_demo

Step 2. Change the watchdog interval time

$ cd ~/driver/aaeon_ec_wdt_V5.0.0.0/example/

$ sudo ./ecwdt_demo -i [interval]

(Set the watchdog interval time)

Ex: $ sudo ./ecwdt_demo -i 10

Step 3. If your board with the standard start-kick-stop things, you can also try out other watchdog features:

- Set the watchdog timeout. Use IOCTL with WDIOC_SETTIMEOUT

- Get the current watchdog timeout. Use IOCTL with WDIOC_GETTIMEOUT

- Check if the last boot is caused by watchdog or it is power-on-reset. Use IOCTL with WDIOC_GETBOOTSTATUS.

- Usage: ./f81xxx_wdt_demo [commman] [argument]

Then the program will show below select

<w> to kick through writing over device file

<i> to kick through IOCTL

<x> to exit the program