PTN3460 Backlight USER GUIDE

Create Date: 2016.11.30

- Install Ubuntu16.04.01

- Setup Environment

- $ sudo vi /etc/default/grub

Add:

GRUB_CMDLINE_LINUX_DEFAULT="processor.nocst=1"

GRUB_CMDLINE_LINUX="acpi_enforce_resources=lax"

- sudo update-grub

- Save and reboot system.

- Added ptn3460 backlight driver

  - Check the smbus port

    - $ sudo apt-get update

    - $ sudo apt-get install i2c-tools

    - $ sudo modprobe i2c-i801

    - $ sudo i2cdetect -l

    - Get the smbus i2c index, it’s i2c-9 here

  - Insmod ptn3460 backligt driver

    - $unzip aaeon_ptn3460_bl_V1.1.0.0.zip

    - $cd aaeon_ptn3460_bl_V1.1.0.0/driver/

    - $make clean;make

    - $sudo cp ./ptn3460.ko /lib/modules/$(uname -r)/kernel/drivers/gpu/drm/

    - $sudo depmod -a

    - $sudo su

    - # echo ptn3460 0x60 > /sys/bus/i2c/devices/i2c-x/new_device(x: i2c bus number)

Note : If you updated your kernel version, please repeat this step.

  - $ sudo vi /etc/rc.local

Add two lines, please use the index number got in last step:

$ modprobe i2c-i801

$ echo ptn3460 0x60 > /sys/bus/i2c/devices/i2c-x/new_device(x: i2c bus number)

  - Reboot and check the ptn3460 backlight

    - $sudo su

    - $sudo echo 128 > /sys/class/backlight/ptn3460-backlight/brightness