| Product Model: EAPLA |  | Date: 2019/02/11 |  |
| --- | --- | --- | --- |
| System BIOS: EAPLAM15 |  | VBIOS: bxt_1016_00.dat GOP: Vbt_bxt_p_eDp.bin |  |
| TXE Type: Consumer TXE Version: 3.1.50.2222 |  |  |  |
| Released By: Ayers Deng (鄧政宏) |  |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |  |
| 01. | (Ubuntu) Fix COM port 5 & 6 can't connect thought Minicom tool. |  |  |
| 02. |  |  |  |
| 03. |  |  |  |
| 04. |  |  |  |
| 05. |  |  |  |
| CPU Microcode |  |  |  |
| M03506C9_0000002E |  |  |  |
| Remark |  |  |  |
| Checksum (BY Checksum.efi): fileName:EAPLAM15.BIN -> checksum:0xDBF10377 |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |  |

| Product Model: EAPLA |  | Date: 2018/12/21 |  |
| --- | --- | --- | --- |
| System BIOS: EAPLAM14 |  | VBIOS: bxt_1016_00.dat GOP: Vbt_bxt_p_eDp.bin |  |
| TXE Type: Consumer TXE Version: 3.1.50.2222 |  |  |  |
| Released By: Ayers Deng (鄧政宏) |  |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |  |
| 01. | Fix Ubuntu only recognized 4 COM port (MB have 6 COM ports) |  |  |
| 02. |  |  |  |
| 03. |  |  |  |
| 04. |  |  |  |
| 05. |  |  |  |
| CPU Microcode |  |  |  |
| M03506C9_0000002E |  |  |  |
| Remark |  |  |  |
| Checksum (BY Checksum.efi): fileName:EAPLAM14.BIN -> checksum:0xDBF0F135 |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |  |

| Product Model: EAPLA |  | Date: 2018/09/27 |  |
| --- | --- | --- | --- |
| System BIOS: EAPLAM13 |  | VBIOS: bxt_1016_00.dat GOP: Vbt_bxt_p_eDp.bin |  |
| TXE Type: Consumer TXE Version: 3.1.50.2222 |  |  |  |
| Released By: Ayers Deng (鄧政宏) |  |  | Approved By: Roy1 Tseng (曾一峰) |
| Item. | Root cause and change description |  |  |
| 01. | Upgrade FRC to version AMI_040. |  |  |
| 02. | Upgrade Afu tool to version 5.10 |  |  |
| 03. |  |  |  |
| 04. |  |  |  |
| 05. |  |  |  |
| CPU Microcode |  |  |  |
| M03506C9_0000002E |  |  |  |
| Remark |  |  |  |
| Checksum (BY Checksum.efi): fileName:EAPLAM13.bin -> checksum:0xDBF89DBF |  |  |  |
| Note: Please execute the following batch files to update BIOS: Step 1: Unzip the related files into USB floppy. Step 2: Boot to setup menu to ensure the 1 boot priority is UEFI: USB floppy name. Step 3: Reboot the system and enter in Shell. Under the correct fs: location, execute GO.NSH to update BIOS and ME automatically. |  |  |  |

###########################################################################

EMB-APL1		Aptio 5.12_Core

###########################################################################

MicrocodeID:

---------------------------------------------------------------------------

Note:

1. Based on K271AM01 BIOS bring-up.

###########################################################################

BIOS Name:	EAPLAM12.bin

BIOS Revision:	1.2

Build Date:	10/27/2017

Checksum:	B0F3h

Sign-on Message:

R1.2(EAPLAM12) (10/27/2017)

Changes:

1. Fine-tune LVDS initial time.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM11.bin

BIOS Revision:	1.1

Build Date:	10/20/2017

Checksum:	472Eh

Sign-on Message:

R1.1(EAPLAM11) (10/20/2017)

Changes:

1. Fix USB3.0 port speed issue.

2. Add new SPI rom support.

3. Add TPM support.

4. Add EDP brightness support.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM10.bin

BIOS Revision:	1.0

Build Date:	10/06/2017

Checksum:	BAD3h

Sign-on Message:

R1.0(EAPLAM10) (10/06/2017)

Changes:

1. Support AAEON new logo.

2. Update TXE firmware.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM09.bin

BIOS Revision:	0.9

Build Date:	07/26/2017

Checksum:	E774h

Sign-on Message:

R0.9(EAPLAM09) (07/26/2017)

Changes:

1. Fine-tune LVDS display.

2. Fine-tune COM port.

3. Fine-tune Smart FAN.

4. Add CPU information.

5. Hidden Terminal.

6. Fix DIO config.

7. Set BIOS setup menu to full screen.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM08.bin

BIOS Revision:	0.8

Build Date:	06/22/2017

Checksum:	2AD2h

Sign-on Message:

R0.8(EAPLAM08) (06/22/2017)

Changes:

1. Change "TERMINATOR_EN" for RS422/RS232/RS485.

2. Set SIO pin77 to WDTO# for WDT output reset signal.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM07.bin

BIOS Revision:	0.7

Build Date:	06/14/2017

Checksum:	CE7Dh

Sign-on Message:

R0.7(EAPLAM07) (06/14/2017)

Changes:

1. Support SIO NCT6116D & HW Monitor.

2. Enable Network Stack.

3. Set Restore AC power lost to Always Off.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM06.bin

BIOS Revision:	0.6

Build Date:	05/11/2017

Checksum:	8E9Fh

Sign-on Message:

R0.6(EAPLAM06) (05/11/2017)

Changes:

1. Add PS8625 Enable/Disable item in BIOS setup menu.

2. Enhance display stability in OS.

3. Set PL1 to default enable.

4. Disable COM0 debug mode.

5. Add COM1 length support.

6. Support UEFI PXE boot.

7. Support UEFI GOP boot type.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM05.bin

BIOS Revision:	0.5

Build Date:	03/22/2017

Checksum:	DB11h

Sign-on Message:

R0.5(EAPLAM05) (03/22/2017)

Changes:

1. Remove unnecessary string from BIOS setup menu.

2. Fix UART mode issue.

3. Disable CSM.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM04.bin

BIOS Revision:	0.4

Build Date:	03/21/2017

Checksum:	E271h

Sign-on Message:

R0.4(EAPLAM04) (03/21/2017)

Changes:

1. Add DIO support.

2. Support AC Power Loss "Last State" in DOS mode.

3. Enhance FAN2 Auto RPM.

4. Enhance HDMI 4K resolution.

5. Add Smart Fan enable/disable item.

6. Add PCIE port Gen speed item.

7. Enable HDD LED function.

8. Disable USB keyboard & mouse wake from sleep.

9. Add USB Port save remove icon.

10. Show PL1 item in setup page & set default to disable.

11. Disable C-states.

12. Support PS8625 DP to LVDS chip & PWM brightness.

13. Rollback "Active Processor Core" to default disable.

14. Enhance SATA port 1 & 2.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM03.bin

BIOS Revision:	0.3

Build Date:	12/06/2016

Checksum:	7F75h

Sign-on Message:

R0.3(EAPLAM03) (12/06/2016)

Changes:

1. Enable TPM.

2. Enable PCIE wake function for WOL.

3. Enable USB keyboard & mouse wake.

4. Support Power Button event in OS.

5. WOL default enable.

6. Support Restore AC Power Loss function.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM02.bin

BIOS Revision:	0.2

Build Date:	11/11/2016

Checksum:	A780h

Sign-on Message:

R0.2(EAPLAM02) (11/11/2016)

Changes:

1. Add Beep Support.

2. Add UART mode support.

3. Add VerbTable support

4. Add COM1~COM6 support.

5. Fix HW Monitor CPU temperature & System temperature display issue.

6. Add Dual Display support & LVDS resolution support.

7. Fine tune the HW Monitor.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM01.bin

BIOS Revision:	0.1

Build Date:	10/13/2016

Checksum:	CD91h

Sign-on Message:

R0.1(EAPLAM01) (10/13/2016)

Changes:

1. Enable SuperIO F81866.

2. Fix system cold boot two twice if power lose.

3. Enable HW Monitor & Smart Fan.

4. Update VBIOS (APL_10.0.1016).

5. Enable BIST & C-states & Active Processor Core.

6. Add Fintek Super IO patch to prevent Power leakage.

---------------------------------------------------------------------------

BIOS Name:	EAPLAM00.bin

BIOS Revision:	0.0

Build Date:	10/07/2016

Checksum:	BB14h

Sign-on Message:

R0.0(EAPLAM00) (10/07/2016)

Changes:

1. First bring up BIOS.

---------------------------------------------------------------------------