| Product Model: MAX-Q470A |  | Date: 2021/02/24 |
| --- | --- | --- |
| System BIOS: MA47AM10 |  | GOP Version: 9.0.1107 |
| ME Type: Corporate ME Version: 14.0.39.1339 |  |  |
| Released By: Roy1 Tseng(曾一峰) |  |  |
| Item. | Root cause and change description |  |
| 01. | First formal release |  |
| 02. |  |  |
| ## CPU Microcode |  |  |
| ## M22A0650_000000BE, M22A0651_000000C2, M22A0653_000000E0, M22A0654_000000C6, M22A0655_000000E2 |  |  |
| ## Remark |  |  |
| Checksum (BY Checksum.efi): MA47AM10.bin: 0xA778ED02 |  |  |
| BIOS/ME/EC update Steps: Please execute the following batch files to update BIOS, ME and EC: Step 1: Unzip the related files into USB floppy. Step 2: Power on the system and press F7 during POST to enter the boot menu Step 3: Select the UEFI: <USB floppy name> to boot into shell environment. Step 4: Under the correct fs: location, execute GO.nsh to update BIOS/ME automatically. (ME won’t be updated if same version) Step 5: Under the correct fs: location, execute GO_EC.nsh to update EC automatically. Step 6: Remove system power. The process of updating BIOS/ME/EC is complete. (ME won’t be updated if same version) |  |  |